3GPP TSG RAN WG 4 Meeting #95-e		TDoc R4-2006122

Electronic Meeting, 25 May - 5 June, 2020

3GPP TSG RAN WG 1 Meeting #100bis-e		TDoc R1-2002966

Electronic Meeting, 20 - 30 April, 2020

Title:	LS on subcarrier spacing for CLI-RSSI measurement

Response to:

Release:	Rel-16

Work Item:	NR_CLI_RIM-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact person:	Hyunsoo Ko

hyunsoo (dot) ko (at) lge (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 concluded following RAN1 assumptions for CLI-RSSI measurement.

  - UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

    - CLI-RSSI measurement is applicable for RRC_CONNECTED intra-frequency only.

That is, for performing CLI-RSSI measurement, RAN1 assumes the SCS of the active bandwidth part is used. On the other hand, for the purpose of CLI-RSSI measurement resource configuration, RAN1 assumes the parameter rssi-SCS-r16 in RSSI-ResourceConfigCLI-r16 in the IE MeasObjectCLI is used.

In order to make clear the purpose of the parameter rssi-SCS-r16 and to avoid ambiguity which SCS is used for performing CLI-RSSI measurement, RAN1 decided to update the description of the parameter rssi-SCS-r16 as follow:

rssi-scs-r16 Reference subcarrier spacing for CLI-RSSI measurement. Only the values 15, 30 kHz or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

RAN1 kindly ask RAN2 to update the description of the parameter rssi-SCS-r16 in the IE MeasObjectCLI in TS38.331 as above in the future specification work.

# 2	Actions

To RAN2:

ACTION: 	RAN1 kindly ask RAN2 to update the description of the parameter rssi-SCS-r16 in the IE MeasObjectCLI in TS38.331 as above in the future specification work.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-RAN WG1 Meeting #101-e	25 May – 5 June 2020, 		e-Meeting

TSG-RAN WG1 Meeting #102	24 – 28 August 2020, 		Toulouse, France