3GPP TSG-RAN WG4 Meeting #95-e	R4-2006164

Online, May 25 – June 5, 2020

Agenda item:	5.10.3.4

Source: 	Qualcomm Incorporated

Title: 	Remaining issues for PUR in eMTC

Document for:	Discussion

# Introduction

In RAN4#94-e-Bis meeting, further discussions on how to capture PUR agreements in the requirements did not reach a conclusion with the following captured in the WF:

Preconfigured UL resource:

- In current agreements Tsearch is FFS when the UE is not configured with eDRX [R4-1907733]. Tsearch is the time required for performing tracking loop.

- Proposals for Tsearch in normal DRX

  - Option 1: Tsearch = 5 SF in normal coverage and enhanced coverage

  - Other options are not precluded.

In this paper, we further discuss this issue.

# Discussion

First, on the value of Tsearch in normal DRX, our view is that with longer DRX cycles of 10.24 s, the UE requires up to 120ms to perform synchronization and tracking loops. This number is certainly smaller for shorter DRX cycles but in our understanding, RAN4 should consider the worst-case scenario.

Proposal 1. Tsearch in normal DRX is 120ms.

Next, on the need to capture the synchronization requirements and the rules for dropping PUR occasion due to lack of synchronization by UE, our view is still that this should not be captured in the requirement.

Per design, UE may or may not transmit in a PUR occasion depending on whether it has mobile-originated (MO) data which is not enforceable by the NW. In our understanding, one of the following cases can occur:

- UE does not transmit in a PUR occasion due to absence of MO data

- UE has MO data but drops PUR due to insufficient time left for synchronization until an upcoming PUR occasion

- UE has MO data but drops PUR due to other reasons (e.g., insufficient volume of data to transmit; UE prefers to accumulate more data bits and send them all in one PUR occasion to save power as opposed to incremental transmissions)

- UE has sufficient MO data and transmits. UE meets the transmit timing accuracy requirements.

All 4 cases above are allowed in the RAN1/2 design. From NW perspective, the only case that matters is case 4 when UE transmits in a PUR occasion, it has to meet the transmit timing accuracy requirements which implicitly means that UE has to have completed the synchronization to the serving cell.

NW cannot distinguish between any of the cases 1, 2 and 3 above. Neither can the test equipment. The presence of MO data is internal to UE and how UE chooses the transmit data to tradeoff power consumption with complexity is up to UE implementation.

In our view, the NW and operators should only care about case 4. Establishing rules for case 2 is not necessary because differentiating between cases 1, 2 and 3 are not possible. Hence, meeting transmit timing accuracy requirements should really be the only specification required. In clause 7.20.2, this is correctly reflected for NB-IoT. For eMTC, it seems this requirement is not correctly reflected in clause 7.24.2 and instead captured in clause 7.1.2.

Proposal 2. Transmit timing accuracy requirement in clause 7.24.2 to reflect transmission in PUR.

Proposal 3. No further specification is necessary for synchronization requirements prior to PUR transmission.

# Conclusions

Proposal 1. Tsearch in normal DRX is 120ms.

Proposal 2. Transmit timing accuracy requirement in clause 7.24.2 to reflect transmission in PUR.

Proposal 3. No further specification is necessary for synchronization requirements prior to PUR transmission.

# References

[1] R4-2005288