3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2006280

Electronic Meeting, 25 May – 5 June, 2020

Title: 	Discussion on IAB-MT REFSENS

Source: 	CATT

Agenda item:	6.5.2.1.2

Document for:	Approval

- Introduction

This contribution provides our further views for the open issues of IAB-MT REFSENS.

- Discussion

2.1 FR2 local area type 2-O

We support to merge BS medium range and local area REFSENS to local area IAB-MT, i.e. -86 to -114 dBm

2.2 Local area FR1 type 1-H

The local area IAB-MT may need to support medium range and local area, then reusing local area assumption is reasonable. 13 dB noise figure and 2 dB IM can be assumed for local area FR1 type 1-H.

2.3 OTA reference sensitivity level for FR1 IAB-MT type 1-O

We agree the comments in the last meeting that reusing BS antenna gain may need more discussion. Wide area IAB-MT antenna can be directional and local area IAB-MT antenna may need to cover more directions. What’s the correct assumption may need more analysis.

2.4 FRC for reference sensitivity

As IAB-MT behaviour is very similar with UE, we didn’t identify the IAB-MT specific FRC difference with UE. Thus we think UE FRC can be reused for IAB-MT.

- Conclusion

We provide some further views on the open issues for IAB-MT REFSENS.

- Reference

[1] R4-2005492, WF on IAB-MT reference sensitivity