3GPP TSG-RAN WG4 Meeting #95-e                                                                                     R4-2006318

Electrical Meeting, 25h May–5th June, 2020

Source:	Samsung

Title:						Views and simulation results for PMI test cases

Agenda item:			6.18.1.2

Document for:	Discussion

- Introduction

In the RAN4 94bis-e meeting, the test parameters of PMI requirement for NR Rel-15 type II codebook were discussed. The following conclusions are shown as following in WF [1]:

- Test setup - Option 1: Use SU-MIMO test setup as baseline scenario - Option 2: MU-MIMO based test setup - Codebook construction - Option 1: 16Tx ports (N1,N2) = (4,2), (O1, O2) = (4,4) - Option 2: 32Tx ports (N1,N2) = (4,4), (O1, O2) = (4,4) - L (numberOfBeams) - 2 - Npsk (phaseAlphabetSize) - Option 1: 4 - Option 2: 8 - SubbandAmplitude - Option 1: False - Option 2: True - PMI-FormatIndicator - Option 1: Wideband - Option 2: Subband - Propagation condition - TDLA30-5 - MIMO correlation - Option 1: XP High - Option 2: XP Medium - MCS and rank - As baseline, use MCS 20, rank 2 - Beam steering model - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 - Option 2: Use Equation 1 as beam steering model for Type II codebook performance requirements. - Option 3: Use option 1 if L = 2, and use option 2 if L > 2 - Test metric - TP ratio between following PMI and rand PMI

Companies are encouraged to give opinions for parameter configurations and initial simulation results for alignment purpose. In this contribution, we provide our views on them.

2. Simulation Results

In last meeting, we suggest to employ the channel model for Type II with ’TDL-A’ and  ’XP-Medium correlation’ because the medium correlation channel could provide rich scatter which reflects the superiority of Type II CSI more obviously. Consequently, we simulate the Type II performance based on ‘XP-Medium’ correlation channel model in the next contents.

In this subsection, we make comparison of Type II codebook performances among groups of multiple parameters, such as different NPSK values, different ‘SubbandAmplitude and’ PMI-FormatIndicator’.

- NPSK

Fig1: Throughput with different NPSK

Table 1: SNR point with different NPSK cases

|  | NPSK=4 | NPSK=8 |
| --- | --- | --- |
| SNR point at 70% throughput (dB) | 12.4 | 12.0 |
| SNR point at 90% throughput (dB) | 15.9 | 15.5 |

As shown in the above, with the ‘NPSK’ increase, the SNR point decrease slightly.

- SubbandAmplitude

Fig2: Throughput with different ‘SubbandAmplitude’

Table 2: SNR point with different ‘SubbandAmplitude’ cases

|  | SubbandAmplitude = True | SubbandAmplitude = False |
| --- | --- | --- |
| SNR point at 70% throughput (dB) | 12.3 | 12.4 |
| SNR point at 90% throughput (dB) | 15.8 | 15.9 |

The gain is marginal when turn on the ‘SubbandAmplitude’ parameter.

- PMI-FormatIndicator

Fig3: Throughput with different ‘PMI-FormatIndicator’

Table 3: SNR point with different ‘PMI-FormatIndicator’ cases

|  | Wideband | Subband |
| --- | --- | --- |
| SNR point at 70% throughput (dB) | 12.4 | 12.4 |
| SNR point at 90% throughput (dB) | 16.8 | 15.9 |

Table 4: TP ratio of SNR point with 90% full throughput

|  | Wideband | Subband |
| --- | --- | --- |
| TP ratio of SNR point at 90% throughput | 3.997 | 1051.5 |

The subband PMI achieves 0.9dB gain than wideband PMI at 90% throughput.

The gap between following PMI and random PMI is larger in subband PMI feedback than in the wideband PMI feedback. Considering the number of Type II parameters increase than Type I’s, the random PMI in Type II could lead to mismatch to channel with higher probability, the subband random PMI hardly achieves the workable SNR point.  Therefore, we prefer to select wideband PMI for the TP radio test.

According to the observations we present that:

- The Type II performance have limited differences with ‘NPSK=4’ and ‘NPSK=8’

- The gain is marginal if ‘SubbandAmplitude=True’ under TDL-A XP Medium correlation channel

- The subband PMI achieves about 1dB gain than wideband PMI at 90% throughput

- The subband random PMI hardly achieves the workable SNR point because the subband beam randomness lead to channel mismatch dramatically.  The corresponding  TP ratio among following PMI and random PMI under sub-band PMI set-up is extremely high, which not feasible to introduce proper performance requirements.

Based on the above observations, we propose that:

Proposal 1:  According to the simulation results, we define the PMI requirement for Rel-15 Type II as following:

      - phaseAlphabeSize:  Npsk= 4

      - subbandAmpltitude: false

      - PMI-FormatIndicator: Wideband

3. Conclusion

In this contribution, we provide our views and PMI simulation results for Type II codebook, based on the results, the following proposals are made:

Proposal 1:  Define the PMI requirement of Rel-15 type II codebook construction as

      - phaseAlphabeSize:  Npsk= 4

      - subbandAmpltitude: false

      - PMI-FormatIndicator: Wideband

4. Reference

[1] R4-2005549,”Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, Samsung, 3GPP RAN4 #94-e-Bis

[2] R4-2002974, “Discussion and simulation results for PMI requirement with NR Rel-15 type II codebook”, Samsung, 3GPP RAN4 #94bis-e

[3] R4-2002979,”Test case design for Rel-16 enhanced type II codebook”, Samsung, 3GPP RAN4 #94bis-e

[3] 3GPP TS 36.101 V15.8.0, User equipment (UE) radio transmission and reception (Release 15), 2019-09

5. Annex

Simulation assumption

| Parameter |  | Unit | Test 5.1 | Test 5.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | Medium XP 16 x 2 (N1,N2) = (4,2) |  |
| Beamforming Model |  |  | Option 1: As specified in Annex B.2.3B.4A of TS36.101 -Relative power ratio among two beams can be fixed as 1 (p=1) Other options not precluded |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | 5/1 |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,12) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband /Subband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | 5/1 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | 2 |  |
|  | NPSK (phaseAlphabetSize) |  | 4/8 |  |
|  | subbandAmplitude |  | Ture/False |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | FFS |  |
|  | RI Restriction |  | FFS |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | MCS 20 FDD |  |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |