3GPP TSG-RAN WG4 Meeting # 95-e    	                                         R4-2006558

Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.8.2.3

Source:	Intel Corporation

Title:	Further considerations on gNB measurement requirements

Document for:	Discussion

# Introduction

In the last RAN4 meetings, some agreements on gNB positioning requirements was achieved in [1]. Therefore, in this contribution, we provide further considerations on the remaining issues below.

- Selection of option for gNB measurement accuracy requirements: FFS

- Optionality of gNB accuracy requirements: FFS

- Side condition: FFS

- Beam configuration: FFS

- Accuracy for different BS types (1-C, 1-H, 1-O, 2-O): FFS

# Discussion

## gNB measurement accuracy requirements

In the last RAN4 meeting, the following options on selection of gNB measurement accuracy requirements for NR positioning were agreed [1].

Candidate options: - Option 1: - Define accuracy for SRS-RSRP and gNB Rx-Tx time difference - Option 2: - Define accuracy for SRS-RSRP, gNB Rx-Tx time difference and UL RTOA

The most controversial issues between these two options is whether UL-RTOA requirements shall be specified in Rel16. Regarding to the timeline of NR Pos WI in Rel16, we prefer to define gNB requirements as Option 1.

Proposal 1: Define the following requirements for NR positioning gNB measurements in Rel16.

- gNB Rx-Tx time difference

- UL SRS-RSRP

Alternatively, if the requirements of UL-RTOA can reuse these of other gNB requirements (e.g. gNB Rx-Tx time difference), we can also compromise to Option 2.

Proposal 1a: Define the following requirements for NR positioning gNB measurements in Rel16.

- gNB Rx-Tx time difference

- UL SRS-RSRP

- UL RTOA

Regarding to the optionality of NB measurement accuracy requirements, our understanding is if gNB claimed to support NR positioning, such performance requirements shall be applicable for gNB.

Proposal 2: gNB positioning measurement accuracy requirement is mandatory for gNBs which declare to support  corresponding NR Positioning functionality.

## Side condition

For the necessary gNB measurements for NR positioning below needs the measurement from multiple gNBs/TRPs which may not the serving cells as configured by LMF. That is the measurement side condition shall be considered for all neighbour cells.

Proposal 3: The side condition for gNB accuracy requirements shall be applicable for serving cell or neighbour cells.

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 SRS for positioning | UL RTOA | UL-TDOA |
| Rel.16 SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| Rel.16 SRS for positioning, Rel.16 DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| Rel.16 SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

# Conclusion

In this contribution, we have provided a further discussion on gNB requirements for NR positioning based on up-to-date RAN4 and RAN1 conclusions. In summary, we proposed that requirement for gNB measurements are desirable and simplified procedures can be considered.

Proposal 1: Define the following requirements for NR positioning gNB measurements in Rel16.

- gNB Rx-Tx time difference

- UL SRS-RSRP

Proposal 1a: Define the following requirements for NR positioning gNB measurements in Rel16.

- gNB Rx-Tx time difference

- UL SRS-RSRP

- UL RTOA

Proposal 2: gNB positioning measurement accuracy requirement is mandatory for gNBs which declare to support  corresponding NR Positioning functionality.

Proposal 3: The side condition for gNB accuracy requirements shall be applicable for serving cell or neighbour cells.

# References

- R4-2005380, “WF on NR Positioning gNB measurement requirements and report mapping”