3GPP TSG-RAN WG4 Meeting #95-e						 R4-2006561

Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR | 0675 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: PRS RSTD requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-5-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR DL RSTD measurement requirements needs to be defined in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: NR DL RSTD measurement reporting criteria is introduced Change #2: NR DL RSTD measurement requirements are introduced in Section 9.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR DL RSTD measurement requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4, 9.8.1, 9.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change 1 >>

### 9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria

…..(Omit the text unchanged)

Table 9.1.4.2-1: Requirements for reporting criteria per measurement category

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, and SS-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, and SS-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA Scell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA Scell. |
| NR intra-frequency RSTD | 1 | Intra-frequency RSTD measurement reporting for UE supporting DL-TDOA in NR |
| NR inter-frequency RSTD | 1 | Inter-frequency RSTD measurement reporting for UE supporting DL-TDOA in NR |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

## << End of Change 1 >>

## << Start of Change 2 >>

## 9.8	  NR measurements for positioning

### 9.8.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements. These measurements for UE NR positioning include DL RSTD, DL PRS-RSRP, UE Rx-Tx time difference and others for NR E-CID  (e.g. SS-RSRP, SS-RSRQ, CSI-RSRP, and CSI-RSRQ) which are defined in TS38.215 [4].

### 9.8.2	DL RSTD measurements

### 9.8.2.1	Introduction

The requirements in section 9.8.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [31] requesting the UE to report one or more of the following measurements for NR DL TDoA positioning [22]: DL RSTD defined in TS38.215 [4].

All RSTD measurement requirements specified in Sections 9.8.2.2 shall apply without DRX as well as for any DRX specified in TS 38.331 [2].

UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

A measurement is defined as a RSTD intra-frequency measurement provided DL PRS resources of the reference/neighbor/serving cells/TRPs to be measured belong to the same positioning frequency layer. Otherwise, a measurement is defined as a RSTD inter-frequency measurement.

Editor’s note: Intra/inter frequency RSTD measurement definition above can be updated up to R4#95e meeting agreements.

### 9.8.2.2 Requirements applicability

The requirements in clause 9.8.2 apply, provided:

-	PRS resource for the cells being measured is detectable.

A PRS resource shall be considered detectable when for each relevant PRS resource:

-	PRS-RSTD related side conditions given in clauses 10.X.Y and 10.X.YY for FR1 and FR2, respectively, for a corresponding Band.

Editor’s note: the exact section to include PRS RSTD measurement side condition will be updated later.

### 9.8.2.3	Number of positioning frequency layers

Editor’s note: RAN4 agreed to not define the minimum for the number of positioning frequency layers (X1) for positioning requirements. But whether to define the minimum for X2, …, X7 to be discussed after the conclusion of RAN1 UE capability signalling.

### 9.8.2.4 	Measurement Reporting Requirements

### 9.8.2.4.1	Intra-frequency DL RSTD Measurements Requirements

The UE can perform intra-frequency RSTD based measurements without measurement gaps if

-	the PRS resource to be measured is completely contained in the active BWP of the UE

For intra-frequency RSTD measurements without measurement gaps, UE may cause scheduling restriction when the following conditions is met.

Editor’s note: Scheduleing restriction when RSTD intra-measurement without gap is TBA or not up to RAN4#95e agreements.

The RSTD measurement accuracy for all the measurements shall be fulfilled according to the accuracy as specified in the sub-clause 10.x.y.

When the physical layer cell identities of neighbour cells together with the DL TDOA assistance data [31] are provided, the UE shall be able to detect and measure intra-frequency RSTD without gap and intra-frequency RSTD with gap, within TRSTD_intra_nonMG, NR  and TRSTD_intra_MG, NR ms respectively, as given below

TRSTD_intra_nonMG, NR=KPRS_RXBeam*( TPRS ⋅(Ms-1)+NPRS)

TRSTD_intra_MG, NR=KPRS_RXBeam*Kintra  *(TPRS ⋅(Ms-1)+NPRS )

Where in,

NPRS_layer is the number of PRS positioning frequency layers

TPRS  is the PRS resource set periodicity,

Ms is the number of PRS periods need for all PRS resource (LPRS_total)  within a PRS resource occasion, which

Ms=ceilLPRS_totalTPRS∙TN

NPRS=NTTPRS if BWPRS<B

UE DL PRS processing capability is indicated by {N,T, B}, defined in TS38.214 [26]

KPRS_RXBeam is the scaling factor for FR2 RX beam sweeping, which is [8] for UE supporting FR2 power class 1 and [4.8] for UE supporting FR2 power class 2/3/4.

CSSFintraRSTD_noMG= CSSFoutside_gap,i is the scaling factor factor for the SSB-based measurements on the carrier frequency i as defined in clause 9.1.5.1.2 for the equal splitting gap sharing scheme only.

Editor’s note: The  RSTD intra-measurement delay requirements above will be updated  up to RAN4#95e agreements.

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells j within TRSTD_intra_nonMG, NR provided:

-6 dB for all Frequency Bands for the reference cell,

-13 dB for all Frequency Bands for neighbour cell j,

is defined as the ratio of the average received energy per PRS RE during the useful part of the symbol to the average received power spectral density of the total noise and interference for this RE, where the ratio is measured over all REs which carry PRS.

### 9.8.2.4.1.1	Measurement Reporting Delay

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported DL RSTD measurement values contained in periodically triggered measurement reports shall be based on the measurement report mapping requirements specified in sections 10.1.x for DL RSTD.

The UE shall not send any measurement reports as long as no corresponding reporting criteria specified in section 9.1.4 are fulfilled.

### 9.8.2.4.2	Inter-frequency DL RSTD Measurements

The RSTD measurement accuracy for all the measurements shall be fulfilled according to the accuracy as specified in the sub-clause 10.x.y.

When the physical layer cell identities of neighbour cells together with the DL TDOA assistance data [31] are provided, the UE shall be able to detect and measure inter-frequency RSTD, within TRSTD_inter, NR  ms, as given below

TRSTD_inter, NR=KPRS_RXBeam*i=1NPRS_layerKinter,i*TRSTD_perlayer, NR(i)

Where in,

TRSTD_perlayer, NR(i)=TPRS(i) ⋅(Ms(i)-1)+NPRS(i)  ms

NPRS_layer is the number of PRS positioning frequency layers

TPRS (i) is the PRS resource set periodicity which is in the i th RSTD positioning frequency layer,

Ms(i) is the number of PRS periods need for all PRS resource (LPRS_total)  within a PRS resource occasion in the i th RSTD positioning frequeny layer , which is

Ms(i)=ceilLPRS_totalTPRS(i)∙T(i)N(i)

NPRS(i)=NiTiTPRS(i) if BWPRS<B(i)

UE DL PRS processing capability is indicated by {N(i),T(i), B(i)} for the i th RSTD positioning frequeny layer, defined in TS38.214 [26]

KPRS_RXBeam is the scaling factor for FR2 RX beam sweeping, which is [8] for UE supporting FR2 power class 1 and [4.8] for UE supporting FR2 power class 2/3/4.

CSSFinterRSTD =CSSFwithin_gap,i is the scaling factor determined by the gap sharing scheme for the RSTD measurements on the carrier frequency i as defined in clause 9.1.5.2.2

Editor’s note: The  RSTD inter-measurement delay requirements above will be updated  up to RAN4#95e agreements.

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells j within TRSTD_intra_nonMG, NR provided:

-6 dB for all Frequency Bands for the reference cell,

-13 dB for all Frequency Bands for neighbour cell j,

is defined as the ratio of the average received energy per PRS RE during the useful part of the symbol to the average received power spectral density of the total noise and interference for this RE, where the ratio is measured over all REs which carry PRS.

### 9.8.2.4.2.1	Measurement Reporting Delay

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported DL RSTD measurement values contained in periodically triggered measurement reports shall be based on the measurement report mapping requirements specified in sections 10.1.x for DL RSTD.

The UE shall not send any measurement reports as long as no corresponding reporting criteria specified in section 9.1.4 are fulfilled.

## << End of Change 2 >>