3GPP TSG-RAN WG4 Meeting #95-e                                                               R4-2006606

Online, 25 May - 5 June 2020

Source:	Huawei, Hisilicon

Title:	TP to TR 38.716.02-00: Addition of CA configuration for CA_n5-n66 with dual UL

Agenda item:	8.2.2

Document for:	Approval

# Background

This contribution provides text proposal on the CA configuration CA_n5-n66 with dual UL as defined in Revised WID [1].

# Text Proposal

##### ---Start of changes---

## 6.55		CA_n5-n66

### 6.55.1		Common for 1 band UL and 2 bands UL CA

#### 6.55.1.1		Operating bands for CA

Table 6.55.1.1-1: CA band combination of band n5+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869MHz | – | 894 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.55.1.2		Channel bandwidths per operating band for CA

Table 6.55.1.2-1: Supported bandwidths per CA band combination of band n5+n66

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum Aggregated bandwidth [MHz] | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n66A | CA_n5A-n66A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |

#### 6.55.1.3		UE co-existence studies

Table 6.55.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n66.

Table 6.55.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |
| n5 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 |

Based on the table above, there is no harmonic relation.

Table 6.55.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |

Based on above tables, there is no harmonic mixing relation.

#### 6.55.1.4		∆TIB and ∆RIB values

For CA_n5-n66, the TIB,c and RIB,c values are given in tables below. For this combination, the results are mainly reused values from CA_5-66.

Table 6.55.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n66 | n5 | 0.3 |
|  | n66 | 0.3 |

Table 6.55.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n66 | n5 | 0 |
|  | n66 | 0 |

#### 6.55.1.5		REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

### 6.55.2		Specific for 2 bands UL CA

#### 6.55.2.1		UE co-existence studies

Table 6.55.2.1-1 lists Band n5 +Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.55.2.1-1: Band n5 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 956 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 132 | 12 | 2571 | 2736 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4327 | 5954 | 6189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1912 | 1722 | 5068 | 5258 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6296 | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3692 | 3432 | 873 | 1088 |

It can be seen that the IMD2 and IMD5 of the dual UL may fall into the DL frequency range of Band n5.

Table 6.55.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.55.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

NOTE:	All the tables mentioned in the note of above table are specified in TS38.101-1.

#### 6.55.2.2		REFSENS requirements

Because of 2nd and 5th IMD issue, the MSD requirements are shown below referring to DC_5_n66 or DC_66_n5.

Table 6.55.2.2-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5A-n66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD21 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| Note 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

##### ---End of changes---

# Reference

[1]	R4-1913628, “Revised WID for NR CA_DC 2 band DL with up to 2 band UL”, ZTE