3GPP TSG-RAN WG4 Meeting #95-e	R4-2006780

Online Meeting, 25 May – 5 Jun. 2020

Title:	TP to TR38.831: FR2 UE architectures for DL Intra-band CA BW Enhancement

Source:	Qualcomm, Apple, Nokia, Nokia Shanghai Bell

Agenda item:	6.14.1.3

Release:	Rel-16

Work Item:	NR_RF_FR2_req_enh

Responsible WG:	RAN4

Document for:	Approval

# 1.	Introduction

RAN4 established that the DL CA BW enhancement feature for Rel-16 would be captured as a UE capability to support DL-only coverage spectrum [1]. ‘DL Coverage Spectrum’ here means the range of DL frequencies that can be populated with a DL CC in CA mode

The agreement was based partly on the study of practical UE architecture choices to enable this feature [2]. In this contribution, we propose a TP to TR to capture the UE architecture study, the resulting UE DL-only extensions cases and the frequency separation classes for DL-only spectrum.

# 2. 	Text Proposal

< Start of TP >

## 4.4	FR2 UE requirements for non-contiguous intra-DL CA for Frequency separation classes larger than 1400 MHz

4.4.1		General

Enhanced DL CA aggregated BW is gated by:

- DL data pipe size in the digital domain

- Signal-handling bandwidth of the ‘front end’, which include RF electronics, ADCs, filters, etc. This parameter is identical to frequency separation, which is defined as the frequency span between the lower edge of the lowest component carrier and the upper edge of the highest component carrier that a UE can support simultaneously.

DL data pipe size enhancement is left to UE implementation. UE frequency separation enhancement is addressed in the UE architecture study in the next subsection. The following assumptions apply:

- The UE’s UL electronics (frequency separation ability) and data handling capacity (‘data pipe’) are left untouched while the DL CA capability is enhanced

- The UE’s DL CA frequency spectrum coverage, i.e the band where a UE can support placing DL CCs is contiguous. This assumption does not mean DL CA must also be contiguous.

- All DL CCs have the same AoA

The UE can convey sufficient information about its RF CA capabilities to the network by signaling the following three aspects:

-	Support for specific band combinations

-	The frequency separation class

-	A restriction on the allocation of DL carriers relative to the UL carriers

Based on this information, the network can configure the UE with CCs from the set of UE supported combinations and according to the applicable restrictions.  We can envision the following potential allocation cases.

4.4.2 	UE Architecture Choices

4.4.2.1		Single receive chain with expanded capability

The receiver design comprises a single chain with enhanced capability relative to rel. 15.

Figure 4.4.2.1-1: Single enhanced receiver to support wider frequency separation

The UL architecture in a UE with independent T/R LOs allows UL coverage to match DL coverage with retuning of TxLO. In the topology with common LOs, the enhanced DL coverage will extend past the UL coverage equally on both sides, unless the UL coverage is enhanced as well.

Figure 4.4.2.1-2: UL options for UE with single enhanced receiver chain

4.4.2.2		Multiple parallel receive chains

This implementation arranges multiple receiver chains to work in parallel to deliver CA BW enhancement.

Figure 4.4.2.2-1: Multiple receiver chains to support wider frequency separation

For UEs with independent Rx and Tx LOs, the UE’s UL coverage can be made to match it with retuning of TxLO. For UEs that utilize a common LO, the UL coverage must be closely related to DL coverage of one of the receivers.

Figure 4.4.2.2-2: UL options for UE with multiple receiver chains

4.4.2.3		UE architecture summary

The UE topologies above exhibit differing UL and DL coverage, summarized in table 4.4.2.3-1.

|  | UL Coverage in Relation to enhanced DL coverage type |  |
| --- | --- | --- |
|  | Independent R/T LOs | Common R/T LOs |
| Single Enhanced receiver | Matches DL coverage | DL coverage extends past UL coverage equally on both sides |
| Multiple Rel. 15 receivers | Matches DL coverage | DL coverage extends past UL coverage on one side. Additional DL-only spectrum is configurable to be on higher-frequency side or lower-frequency side of UL spectrum. |

Table 4.4.2.3-1: UL and DL coverage comparison for UEs

4.4.3		Feature Definition

For Rel-16, the DL intra-band CA BW enhancement feature is the definition of UE capability to support DL-only coverage spectrum. This enhancement is in addition to the common UL+DL coverage spectrum of UE carried over from Rel-15.

### 4.4.4	UE DL-only spectrum extension cases

The assumption for the DL-only spectrum extension cases assumes that DL Fs ≥ 1400 MHz as an enhancement over Rel-15 UE capability. Based on this assumption the network should be constrained by two factors:

- Frequency separation class for UL and DL

- UL coverage spectrum contained within the DL coverage spectrum

| Case 1: No restriction | Case 2: Symmetric extension | Case 3: One-sided extension |
| --- | --- | --- |

In Case 1 the UE supports different UL & DL coverage spectrum with no restriction and represents the most flexible frequency separation class enhancement in Rel-16. As illustrated in Case 1 the IF bandwidth of the UL and DL can be as wide as 1400 MHz for the UL and 2400 MHz for the DL. The UE implementation in this case may need a dedicated UL LO which increases complexity.

In Case 2 the UE supports symmetric extension of bidirectional coverage spectrum for DL only. These UEs need to further restrict the allocation of DL carriers such that the UL coverage spectrum is centered within the DL coverage spectrum. The UE implementation in this case may reduce the RF architecture complexity by sharing the LO between UL and DL, but it may increase the network’s difficulty in exploiting available DL spectrum in the general case.

In Case 3 the UE supports one-sided extension of bidirectional coverage spectrum for DL only.  The UE implementation in this case provides a tradeoff between full flexibility/high complexity in case 1 and high restrictive allocation/large DL captured spectrum in case 2.

#### 4.4.4.1	Frequency separation classes for DL-only spectrum

For Rel-16, extend the frequency separation class definition to include the following values for frequency separation ‘Fs’: {800, 1000, 1200, 1400, 1600, 1800, 2000, 2200, 2400} MHz, where Fs values larger 1400 MHz apply to downlink frequency separation only. The spectrum covered by the DL-only frequency separation shall be located on one-side relative to the bidirectional spectrum.  The frequency separation classes for DL only are specified in Table 4.4.2-1, which indicates the maximum DL-only frequency separation span of DL CCs that UE can support per band.

Table 4.4.2-1: Frequency separation classes for DL-only spectrum

| Frequency separation class | Frequency separation (Fsd) |
| --- | --- |
| I | Fsd ≤ 200 MHz |
| II | Fsd ≤ 400 MHz |
| III | Fsd ≤ 600 MHz |
| IV | Fsd ≤ 800 MHz |
| V | Fsd ≤ 1000 MHz |

< End of TP >

# 3. 	References

[1]	R4-1913042, ‘WF on DL Intra-band CA BW Enhancement’, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #92-Bis, Chongqing, CN, Oct 2019

[2]	R4-1908164, ‘UE Architecture for DL CA BW Enhancement for Rel. 16 FR2’, Qualcomm, RAN4#92, Ljubljana, Sl, Aug 2019

[3]	RP-190761, “NR RF requirements for FR2,” Nokia, Nokia Shanghai Bell, 3GPP RAN #83, March 2019

[4]	R4-1916021, “WF on FR2 DL Intra-band CA BW Enhancement,” Qualcomm, Apple, Nokia, Samsung, 3GPP RAN4 #93, November 2019

[5]	R4-2000013, “Remaining issues with the FR2 frequency separation class” Apple Inc., 3GPP RAN4 #94-e, February 2020

[6]	R4-2000014, “CR to 38.101-2 on FR2 frequency separation class enhancement,” Apple Inc., 3GPP RAN4 #94, February 2020