3GPP TSG-RAN WG4 Meeting # 95-e                                        R4-2006862

# Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.1.5.13

Source:	MediaTek Inc.

Title:	Discussion on timing requirement for NR-U

Document for:		Discussion

- Introduction

In this paper we provide our view on the remaining issues of timing requirements for NR-U.

- New reference cell if a reference cell is unavailable

In the last meeting, the CR for timing requirement has been discussed [1], and companies had different views on the UE behavior regarding the new reference cell if a reference cell is unavailable. The corresponding text in the CR is attached below:

… If the reference cell is unavailable at the UE during the last 160 ms on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in section 7.1.2; otherwise the UE shall not transmit any uplink signal.

If a reference cell on a carrier frequency belonging to the PTAG, which is subject to CCA, is unavailable at the UE for more than 160 ms then the UE [is allowed or shall] use any of available activated SCell(s) at the UE in PTAG as a new reference cell.

If a reference cell on a carrier frequency belonging to the STAG, which is subject to CCA is unavailable at the UE for more than 160 ms then the UE [is allowed or shall] use any of available activated SCell(s) at the UE in STAG as a new reference cell.

In our view, on the 2nd and the 3rd paragraphs, UE should be allowed to use any of available activated SCell as a new reference cell, provided that the UE meets all the transmit timing requirements defined in section 7.1.2. This is the same principle as used in the 1st paragraph.

Mandating UE to use an activated SCell is not always the best choice. For example, if the timing difference between the reference Pcell and the SCell is larger than a CP, and this timing jump would introduce additional interference in the UL reception. Thus, it is not necessary to mandate UE to switch timing to a SCell, as long as UE meets the timing requirement.

Proposal 1: If a reference cell is unavailable, UE should be allowed to use any of available activated SCell as a new reference cell, provided that the UE meets all the transmit timing requirements defined in section 7.1.2.

- Summary

In this paper, timing requirements for NR-U have been discussed. We have the following observations and proposals:

Proposal 1: If a reference cell is unavailable, UE should be allowed to use any of available activated SCell as a new reference cell, provided that the UE meets all the transmit timing requirements defined in section 7.1.2.

References

[1] R4-2005374 CR for timing requirement for NR-U, MediaTek Inc.