# 3GPP TSG-RAN WG4 Meeting #95-e                     	R4-2007111

# Electronic Meeting, 25 May – 5 June, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	UE and BS demodulation requirements for LTE_eMTC5

Agenda item:	5.10.4

Document for:	Discussion and Agreement

Introduction

At RAN4 #94bis-e, the discussion on UE and BS demodulation requirements for the WI on Additional MTC enhancements for LTE [1] was continued. In the WF [2], the following agreements on UE and BS demodulation requirements are reflected.

Agreements on UE/BS demodulation requirements - FFS to define new demodulation requirements of PUSCH with multi-TB scheduling. - FFS to define new demodulation requirements of PDSCH with multi-TB scheduling.

This contribution discusses the two remaining open issues in regard to multi-TB scheduling.

Discussion

The discussion in this section refers to the two open issues on UE/BS demodulation requirements for the multi-TB scheduling enhancement in the WF [2], as depicted above.

The matter was discussed at RAN4 #94-e with different views of companies. In [3] and [4], it was proposed to define new PDSCH and PUSCH demodulation requirements for the interleaved transmission of TB repetitions due to observed improved throughput performance. At the same time, it was observed in [4] that there is a “time diversity gain in this interleaving way”. While we agree on this observation, it is obvious that the improved throughput performance is achieved through this interleaving gain, which shows a clear dependency on the number of repetitions. Hence this improvement is majorly caused by increased time diversity of the channel and not by refinements to the receiver, and thus no additional verification of the receiver is required.

The following observation is made:

For multi-TB scheduling, the performance benefit can be reasoned by increased time diversity of the radio channel due to interleaved mode rather than refinements to PHY layer reception.

Moreover, as commented during RAN4 #94-e [5], a gain of around 1 dB for the maximum number of 8 TBs and 16 or 32 repetitions is identified in [4], while for other configurations with smaller number of TBs, the gain is expected to proportionally decrease and thus will remain significantly below 1 dB.

This was confirmed by simulation results for PDSCH in [6], which observes that there is no performance difference between single TB transmission and interleaved multi-TB transmission for 2 TB or 4 TB, if the comparison is based on demodulation requirement parameters defined in TS 36.101.

Thus, we concur with views expressed in [6] and [7] for PDSCH and PUSCH and in [8] for PUSCH, that interleaved multi-TB transmission does not affect neither UE nor BS demodulation requirements.

The following proposal is made:

No separate UE / BS demodulation requirements are required for interleaved multi-TB transmission for PDSCH / PUSCH.

Conclusion

This contribution has discussed two remaining open issues

- on the need for separate PDSCH demodulation requirements for multi-TB scheduling and

- on the need for separate PUSCH demodulation requirements for multi-TB scheduling.

Following proposal for agreement is made:

- No separate UE / BS demodulation requirements are required for interleaved multi-TB transmission for PDSCH / PUSCH.

References

- RP-192875, "Revised WID: Additional MTC enhancements for LTE", source: Ericsson

- R4-2005513, "Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE", source: Ericsson

- R4-2001480, "Discussion on UE performance requirements for additional MTC enhancements for LTE", source: Huawei, HiSilicon

- R4-2001479, "Discussion on BS performance requirements for additional MTC enhancements for LTE", source: Huawei, HiSilicon

- R4-2002514, "Email discussion summary for RAN4#94e_#85_LTE_eMTC5_Demod", source: Moderator (Ericsson)

- R4-2004020,"Open issues on UE/BS demodulation requirements for Rel-16 eMTC", source: Ericsson

- R4-2003563, "Simulation results for MPDCCH performance improvement tests in R16 ", source: Qualcomm Incorporated

- R4-2000311, "View on BS demodulation requirement for LTE additional enhancement for eMTC", source: Samsung