3GPP TSG-RAN WG4 Meeting #95-e	  R4-2007192

# Electronic Meeting, 25th May – 5th June, 2020

Title: 	Discussion and simulation for URLLC UE PDSCH demodulation requirements for high reliability with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2.1

Document for:	Discussion

Background

When considering the high reliability of URLLC PDSCH performance requirements, RAN4 agreed to test the ultra-low BLER target (10^-5) and a higher BLER target separately. In this paper, the higher BLER target test is discussed. From #94 e-meeting discussion results, the higher BLER target of 10^-2 was agreed. Based on the agreements on #94 and #94bis e-meeting, test parameters except the MCS were all decided. In this paper, the simulation results are provided with MCS13/14/15/16. A proper MCS is proposed based on the simulation results.

Agreement of #94bis:

- PDSCH Aggregation level: 2 for FDD and TDD

- MCS: Select a suitable MCS value as per the evaluation results for MCS13/14/15/16

- FR2 requirements for High reliability: Keep it open meanwhile prioritize discussion on introducing FR1 requirements in Q2; and interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios

In this paper, open issues are discussed based on the agreed parameters. Our proposals are given.

Simulation

Simulation assumption:

The agreed parameters are concluded in table below.

Table 2-1: Simulation assumption for UE PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 2x2, ULA low 2x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Processes |  | 4 |
| PT-RS |  | Disabled |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 3, MCS 13/14/15/16 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 1% (BLER is calculated after all transmission) |

Simulation results:

PDSCH performance with different MCS for TDD and FDD were simulated.

- FDD

- Antenna configuration 2x2                               (b) Antenna configuration 2x4

Figure 2.2-1: PDSCH performance with different MCS

The SNR values with MCS13/14/15/16 when BLER=0.01 for 2x2 and 2x4 are shown in table below:

Table 2-2: SNR values for PDSCH FDD performance requirements

| MCS Antenna configuration SNR | 2x2 | 2x4 |
| --- | --- | --- |
| MCS13 | 1.5 | -3 |
| MCS14 | 3.3 | -2 |
| MCS15 | 5.1 | 0.3 |
| MCS16 | 5.5 | 1.1 |

The simulation results show that MCS13 is fine for both of 2x2 and 2x4.

- TDD

- Antenna configuration 2x2                                    (b) Antenna configuration 2x4

Figure 2.2-2: PDSCH performance with different MCS

The SNR values with different MCS when BLER=0.01 for 2x2 and 2x4 are shown in table below:

Table 2-3: Test parameters for PDSCH TDD performance requirements

| MCS Antenna configuration SNR | 2x2 | 2x4 |
| --- | --- | --- |
| MCS13 | 2.5 | -2.5 |
| MCS15 | 3.7 | 0.8 |
| MCS16 | 9.5 | 4.2 |

The simulation results show that MCS13 is good for both of FDD and TDD.

Proposal 1: We propose to use MCS13 from table 3.

Proposal

In this paper, the requirements for PDSCH demodulation performance are defined. Proposal is listed below:

Proposal 1: We propose to use MCS13 from table 3.

Reference

[1] R4-2005527, Way forward for NR UE URLLC performance requirements, #94bis, Intel Corporation.