3GPP TSG-RAN WG4 Meeting # 95-e 															 R4-2007346

Electronic Meeting, 25 May – 5 June, 2020

Title: 		 		On interruption for FR1 Tx switching between two uplink carriers

Source: 	 		OPPO

Agenda item:	  	6.13.2.1

Document for:	Discussion

- Introduction

In last meeting, a WF [1] on RRM requirements for Tx switching between two uplink carriers was approved.

In this document, we provide our view on DL interruption length/starting point.

- Discussion

As background, RAN4 has agreed on the candidate UL switching periods in RF session as bellow.

- Length of UL switching period for defining UE RF requirements and capability reporting:

- For SUL and UL CA： {35us, 140 us, 210us} or {1, 4, and 6} OFDM symbols for 30kHz SCS

- For EN-DC：        {35us, 140 us} or {1, 4} OFDM symbols for 30kHz SCS

As agreed in last meeting, DL interruption is defined in the unit of symbol. And two options are proposed in WF [1] to be further analysis and decide. In our view, DL interruption time may be larger than UL switching time due to both RF tuning and baseband adjustment.

Observation 1: DL interruption time may be larger than UL switching time.

For the DL interruption location/Starting point:

- Option 1: the DL interruption are the OFDM symbols fully or partial overlapped with the UL switching period.

- Option 2: the starting time is determined by TA+TA uncertainty+MRTD

In our view, the DL interruption may still exist in a short while even after RF UL switching finished. So the starting time should not be later than the end of UL switching period, which can be determined by TA + TA uncertainty + MRTD. We support option 2.

Proposal 1: The starting time should not be later than the end of UL switching period, which can be determined by TA + TA uncertainty + MRTD.

For EN-DC, when uplink switching is conducted, UE is allowed to cause DL interruption of X OFDM symbols, which overlap with the UL switching period, on NR or E-UTRA carrier(s) depending on UE capability [TBD]. The DL interruption lengths of X for NR and E-UTRA carrier(s) are defined in Table 1 and 2.

Table 1: DL interruption on NR carrier(s) for switching between two uplink carriers in EN-DC (FR1)

|  | NR Slot length (ms) | Uplink Tx switching period(symbol) Note1 |  |
| --- | --- | --- | --- |
|  |  | 35us | 140us |
| 0 | 1 | 3 | 5 |
| 1 | 0.5 | 4 | 7 |
| 2 | 0.25 | 7 | 13 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD] |  |  |  |

Table 2: DL interruption on E-UTRA carrier(s) for switching between two uplink carriers in EN-DC (FR1)

| Uplink Tx switching period(symbol) Note1 |  |
| --- | --- |
| 35us | 140us |
| 3 | 5 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD] |  |

For NR Standalone, when uplink switching is conducted, UE is allowed to cause DL interruption of Y OFDM symbols, which overlap with the UL switching period, on NR carrier(s) depending on UE capability [TBD]. The DL interruption lengths of Y are defined in Table 3.

Table 3: DL interruption on NR carrier(s) for switching between two uplink carriers in NR SA (FR1)

|  | NR Slot length (ms) | Uplink Tx switching period(symbol) Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 3 | 5 | 5 |
| 1 | 0.5 | 4 | 7 | 9 |
| 2 | 0.25 | 7 | 13 | 17 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD] |  |  |  |  |

Proposal 2: Define DL interruption length for EN-DC and NR SA according to Table 1, 2 and 3.

- Conclusions

In this contribution, we have the following observations and proposals:

Observation 1: DL interruption time may be larger than UL switching time.

Proposal 1: The starting time should not be later than the end of UL switching period, which can be determined by TA + TA uncertainty + MRTD.

Proposal 2: Define DL interruption length for EN-DC and NR SA according to Table 1, 2 and 3.

Table 1: DL interruption on NR carrier(s) for switching between two uplink carriers in EN-DC (FR1)

|  | NR Slot length (ms) | Uplink Tx switching period(symbol) Note1 |  |
| --- | --- | --- | --- |
|  |  | 35us | 140us |
| 0 | 1 | 3 | 5 |
| 1 | 0.5 | 4 | 7 |
| 2 | 0.25 | 7 | 13 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD] |  |  |  |

Table 2: DL interruption on E-UTRA carrier(s) for switching between two uplink carriers in EN-DC (FR1)

| Uplink Tx switching period(symbol) Note1 |  |
| --- | --- |
| 35us | 140us |
| 3 | 5 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD] |  |

Table 3: DL interruption on NR carrier(s) for switching between two uplink carriers in NR SA (FR1)

|  | NR Slot length (ms) | Uplink Tx switching period(symbol) Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 3 | 5 | 5 |
| 1 | 0.5 | 4 | 7 | 9 |
| 2 | 0.25 | 7 | 13 | 17 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD] |  |  |  |  |

- Reference

- R4- R4-2005416, WF on RRM requirements for Tx switching between two uplink carriers, Huawei, HiSilicon