3GPP TSG-RAN WG4 Meeting #95-e	R4-2007372

Electronic Meeting, 25 May – 5 June 2020

Agenda Item:	5.10.4

Source:	Ericsson

Title:	Simulation results of CSI-RS based PMI reporting test

Document for:	Discussion

# 1	Introduction

RAN4#94-e-Bis agreed with the simulation assumption of CSI-RS based PMI reporting test for non-BL UE [1]. This contribution provides our initial simulation results.

# 2	CSI-RS based PMI reporting test

## 2.1	Simulation parameters

Table 1 and Table 2 show the simulation parameters and FRC used for the simulation according to [1].

Table 1	Simulation parameters for CSI-RS based CSI reporting tests.

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| PDSCH transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 6 |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 |
|  | σ | dB | -3 |
|  | δ | dB | 0 |
| CRS reference signals |  |  | Antenna ports 0, 1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |
| CSI reference signal configuration |  |  | 0 |
| Propagation condition and antenna configuration |  |  | High XP 8 x 2 |
| Beamforming Model |  |  | As specified in TS36.101 B.4.3 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 0000 FFFF |
| SNR |  | dB | TBD |
|  |  | dB[mW/15kHz] | TBD |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Reporting periodicity |  | ms | 10 |
| PMI delay |  | ms | 10 |
| cqi-pmi-ConfigurationIndex |  |  | 12 |
| ce-csi-rs-feedback-config |  |  | Configured |
| Frequency hopping |  |  | Disabled |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |
| PDSCH repetition level |  |  | 1 |
| MPDCCH repetition level |  |  | 1 |
| Beamforming Precoder for MPDCCH |  |  | No precoding |
| Precoder update granularity for MPDCCH |  |  | N/A |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |
| PDSCH PRB size |  | PRB | 3 |
| PDSCH MCS |  |  | QPSK 1/2 |
| RI |  |  | 1 |

Table 2	RMC for CSI-RS based PMI reporting test.

| Parameter | Unit | Values |
| --- | --- | --- |
| Allocated resource blocks |  | 4 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/2 |
| Information bit payload for non CSI-RS subframe | bits | 472 |
| Information bit payload for CSI-RS subframe | bits | 472 |
| Binary channel bits for non CSI-RS subframe | bits | 896 |
| Binary channel bits for non CSI-RS subframe | bits | 960 |

## 2.2	Simulation results

Figure 1 (a) compares the relative throughputs with follow PMI and random PMI, and Figure 2 (b) is the throughput gain of follow PMI over random PMI. Table 3 summarizes the throughput gain at SNRs where the 70, 80, 90% of the maximum throughput with follow PMI are achieved.

Observation 1: Throughput gain of 1.4 is achieved with 70% of maximum throughput with follow PMI.

| (a) Relative throughput of follow PMI and random PMI | (b) Throughput gain of follow PMI over random PMI |
| --- | --- |

Figure 1	Comparison of follow PMI and random PMI.

Table 3	Throughput gain.

| SNR (dB) | X (%) of maximum throughput with follow PMI | Throughput gain of follow PMI over random PMI |
| --- | --- | --- |
| -2 | 73 | 1.41 |
| -1 | 83 | 1.41 |
| 0 | 90 | 1.36 |

Considering the implementation margin, we propose to set the throughput gain of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI.

Proposal 1: Set γ=1.2 for CSI-RS based PMI reporting test for non-BL UE at the SNR where 70% of the maximum throughput is achieved with the follow PMI.

# 3	Summary

Observation 1: Throughput gain of 1.4 is achieved with 70% of maximum throughput with follow PMI.

Proposal 1: Set γ=1.2 for CSI-RS based PMI reporting test for non-BL UE at the SNR where 70% of the maximum throughput is achieved with the follow PMI.

# 4	References

- R4-2005513, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”, Ericsson.