3GPP TSG-RAN WG4 Meeting # 95-e 														R4-2007582 Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.5.2.2.3

Source: 	Ericsson

Title:	TP to TS 38.174: OTA RX spurious

Document for:	Approval

- Introduction

Section 2 proivde some reasoning on the text proposal and Section 3 provides text proposal on chapter 10 for Rx spurious for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Disucssion

The text proposal is based on the WF [1] and WF [2]. WF [4] agrees the free permutation of different IAB-MT class and IAB-DU class. BS spurious requirement does not differentiate class, so this agreement does not introduce additional complexity to specifiy IAB-MT RX spurious requirement. However, whether the free permutation of the different Type is not decided and thus there is uncertainty on this aspect. There is a possibility that different type could be mixed in one IAB node, For example, IAB-MT of type 1-O and IAB-DU of type 1-H. As the spurious requirement is different for different BS type, so the question is how the IAB-MT spurious should be specified. In our companion paper [4], we propose the TX spurious should follow the IAB-DU of the declared class and type irrespective the IAB-MT class and type. This is to guarantee the protection of the BS coexisting service. We believe it is the same for RX spurious requirement and IAB-MT spurious should follow the spurious requirement of the IAB-DU within the same node. Then perhaps one thing needs to mention is how to relate the IAB-MT type 1-O requirement to the IAB-DU type 1-H. Here the only thing to change is the total emission limit which should be refer to the fixed scaling factor as the same as type 1-H. This is the case when IAB-MT type 1-O is mixed with IAB-DU type 1-H or vice versa, which should be sepecified separately and cannot be refered.

The suggested text proposal below captured the above proposals.

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 10.7	OTA receiver spurious emissions

### 10.7.1 General

The OTA RX spurious emission is the power of the emissions radiated from the antenna array from a receiver unit.

The metric used to capture OTA receiver spurious emissions for IAB-MT and IAB-DU for IAB type 1-O and IAB type 2-O is total radiated power (TRP), with the requirement defined at the RIB.

### 10.7.2 IAB-DU OTA receiver spurious emissions

#### 10.7.2.1 Minimum requirement for IAB-DU type 1-O

Minimum requirement of IAB-DU for IAB type 1-O is specified in TS 38.104[2], subclause 10.7.2.

#### 10.7.2.2 Minimum requirement for IAB-DU type 2-O

Minimum requirement of IAB-DU for IAB type 2-O is specified in TS 38.104[2], subclause 10.7.3.

### 10.7.3	IAB-MT OTA receiver spurious emissions

#### 10.7.3.1 Minimum requirement for IAB-MT type 1-O

For a IAB-MT operating in TDD, the OTA RX spurious emissions requirement shall apply during the transmitter OFF period only.

The RX spurious emissions requirements for IAB-MT type 1-O depends on the declared IAB-DU type.

For the case IAB-DU type 1-O is declared by the manufacturer, the RX spurious emissions requirements for BS type 1-O, in TS 38.104[2] subclause 10.7.2 shall apply to IAB-MT type 1-H.

For the case IAB-DU type 1-H is declared by the manufacturer, The OTA RX spurious emissions requirement for IAB-MT type 1-O is that for each basic limit specified in table 10.7.3.1-1, the power sum of emissions at the RIB shall not exceed limits specified as the basic limit + X, where X = 10log10(NRXU,countedpercell) , unless stated differently in regional regulation.

Table 10.7.3.1-1: General IAB-MT receiver spurious emission basic limits for IAB-MT type 1-O

| Spurious frequency range | Basic limit (Note 4) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -30 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: Additional limits may apply regionally. |  |  |  |

#### 10.7.3.2 Minimum requirement for IAB-MT type 2-O

The OTA RX spurious emissions requirement shall apply during the transmitter OFF period only.

For the IAB-MT type 2-O, the power of any RX spurious emission shall not exceed the limits in table 10.7.3.2-1.

10.7.3.2-1: Radiated Rx spurious emission limits for IAB-MT type 2-O

| Spurious frequency range (Note 4) | Limit (Note 5) | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1. NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [19], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 10.7.3-2. NOTE 5: Additional limits may apply regionally. |  |  |  |

Table 10.7.3.2-2: Step frequencies for defining the radiated Rx spurious emission limits for IAB-MT type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2002484, WF on TS referencing rules, Nokia

- R4-2002483, WF on IAB RX RF requirement, Samsung