3GPP TSG RAN WG4 Meeting #95-e	R4-2007595

Electronic Meeting, 25 May – 5 June, 2020

Source:		Rohde & Schwarz

Title:	TP to 37.941: MU tables for additional Tx test cases for PWS

Agenda Item:	6.19.5

Document for:	Approval

- Introduction

Additional test cases for Plane Wave Synthesizer (PWS) were introduced in TR 37.941 [1] during last meeting. Even though the MU calculation tables were presented in [2], the corresponding MU tables were not added to avoid editorial issues.

This contribution is introducing the TP for the missing tables for PWS. It also introduces the text proposal for EVM MU value derivation for FR1.

- References

- R4-2005609, “Draft TR 37.941 v0.2.0”, Huawei, RAN4 #94-e-Bis, April 2020

- R4-2004527, “OTA BS testing Tx FR1 MU calculation tables”, Huawei, RAN4 #94-e-Bis, April 2020

- R4-2004393, “Justification for additional Tx test cases for PWS”, Rohde & Schwarz, RAN4 #94-e-Bis, April 2020

- R4-2004394, “TP to TR 37.941 to introduce additional Tx test cases for PWS”, Rohde & Schwarz, RAN4 #94-e-Bis, April 2020

< Unchanged Text Deleted >

< Beginning of Changes >

### 9.4.5		Plane Wave Synthesizer

#### 9.4.5.1	Measurement system description

Measurement system description is captured in clause 7.6.

#### 9.4.5.2 	Test procedure

##### 9.4.5.2.1	Stage 1: Calibration

Calibration procedure for the Plane Wave Synthesizer is captured in clause 8.6.

##### 9.4.5.2.2	Stage 2: BS measurement

The testing procedure consists of the following steps:

1)	Set up BS in place of SGH from calibration stage. Align BS with beam peak direction of range antenna.

2)	Configure TX branch and carrier according to maximum power requirement and test configuration.

3)	Set the BS to transmit the applicable test signal.

4)	Measure the PDL_RS which is the measured signal power of DL RS EIRP (in the beam peak direction).

5)	Calculate EIRPDL_RS using the following equation:

EIRPDL_RS_p(x) = PDL_RS_meas + LA→C.

And

EIRPDL_RS = EIRPDL_RS_p1 + EIRPDL_RS_p2 where the declared beam is the measured signal for any two orthogonal polarizations (denoted p1 and p2).

6)	Repeat steps 2-5 for all conformance test beam direction pairs and test conditions.

#### 9.4.5.3	MU value derivation, FR1

As the DL RS power is an absolute measurement most of the uncertainties form the EIRP accuracy remain the same. Also it can be noted that the measured signal is a wanted signal and hence will be beam formed in the same way as the wanted signal, hence any errors which may be dependent on the beam shape will be the same as for the EIRP accuracy measurement.

Table 9.4.5.3-1: PWS MU value derivation for OTA E-UTRA DL RS power measurement

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A7-1a | Misalignment DUT & pointing error | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| C3-1 | DL-RS MU derived from conducted spec | 0.41 | 0.56 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 | 0.56 |
| A7-2a | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for DUT antenna | 0.05 | 0.14 | [0.14] | Rectangular | 1.73 | 1 | 0.03 | 0.08 | [0.08] |
| A7-3 | RF leakage (calibration antenna connector terminated) | 0.09 | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 | 0.09 |
| A7-4a | QZ ripple with DUT | 0.42 | 0.43 | [0.43] | Rectangular | 1.73 | 1 | 0.24 | 0.25 | [0.25] |
| A7-5 | Miscellaneous Uncertainty | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A7-14 | System non-linearity | [0.06] | [0.06] | [0.06] | Rectangular | 1.73 | 1 | [0.04] | [0.04] | [0.04] |
| A7-13 | Frequency Flatness | 0.13 | 0.13 | 0.13 | Rectangular | 1.73 | 1 | 0.08 | 0.08 | 0.08 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 |  | Uncertainty of the network analyzer |  |  | 0.13 |  | 0.20 |  | 0.20 |  | Gaussian |  | 1.00 |  | 1 |  | 0.13 | 0.20 |  | 0.20 |  |
| A7-6 |  | Mismatch (i.e. reference antenna, network analyser and reference cable) |  |  | 0.13 |  | 0.33 |  | 0.33 |  | U-shaped |  | 1.41 |  | 1 |  | 0.09 | 0.23 |  | 0.23 |  |
| A7-7 |  | Insertion loss variation |  |  | 0.18 |  | 0.18 |  | 0.18 |  | Rectangular |  | 1.73 |  | 1 |  | 0.10 | 0.10 |  | 0.10 |  |
| A7-3 |  | RF leakage (calibration antenna connector terminated) |  |  | 0.09 |  | 0.09 |  | 0.09 |  | Gaussian |  | 1.00 |  | 1 |  | 0.09 | 0.09 |  | 0.09 |  |
| A7-8 |  | Influence of the calibration antenna feed cable |  |  | 0.10 |  | 0.10 |  | 0.10 |  | Rectangular |  | 1.73 |  | 1 |  | 0.06 | 0.06 |  | 0.06 |  |
| C1-4 |  | Uncertainty of the absolute gain of the reference antenna |  |  | 0.50 |  | 0.43 |  | 0.43 |  | Rectangular |  | 1.73 |  | 1 |  | 0.29 | 0.25 |  | 0.25 |  |
| A7-9 |  | Misalignment of positioning system |  |  | 0.00 |  | 0.00 |  | 0.00 |  | Exp. normal |  | 2.00 |  | 1 |  | 0.00 | 0.00 |  | 0.00 |  |
| A7-1b |  | Misalignment of calibration antenna & pointing error |  |  | 0.05 |  | 0.05 |  | 0.05 |  | Rectangular |  | 1.73 |  | 1 |  | 0.03 | 0.03 |  | 0.03 |  |
| A7-10 |  | Rotary joints |  |  | 0.00 |  | 0.00 |  | 0.00 |  | U-shaped |  | 1.73 |  | 1 |  | 0.00 | 0.00 |  | 0.00 |  |
| A7-2b |  | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna |  |  | 0.12 |  | 0.12 |  | [0.12] |  | Rectangular |  | 1.73 |  | 1 |  | 0.07 | 0.07 |  | [0.07] |  |
| A7-4a |  | QZ ripple with calibration antenna |  |  | 0.20 |  | 0.20 |  | 0.20 |  | Rectangular |  | 1.73 |  | 1 |  | 0.12 | 0.12 |  | 0.12 |  |
| A7-11 |  | Switching uncertainty |  |  | 0.02 |  | 0.02 |  | 0.02 |  | Rectangular |  | 1.73 |  | 1 |  | 0.01 | 0.01 |  | 0.01 |  |
| A7-12 |  | Field repeatability |  |  | 0.06 |  | 0.12 |  | [0.12] |  | Gaussian |  | 1.00 |  | 1 |  | 0.06 | 0.12 |  | [0.12] |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | [0.63] | [0.78] | [0.78] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | [1.24] | [1.53] | [1.53] |

< End of Changes >

< Unchanged Text Deleted >

< Beginning of Changes >

### 9.5.5		Plane Wave Synthesizer

#### 9.5.5.1	Measurement system description

Measurement system description is captured in clause 7.6.

#### 9.5.5.2	Test procedure

##### 9.5.5.2.1	Stage 1: Calibration

Calibration procedure for the Plane Wave Synthesizer is captured in clause 8.6.

##### 9.5.5.2.2	Stage 2: BS measurement

Reference procedure in clause 9.4.5.2.2 where in step 4 the appropriate measurement is:

The appropriate test parameter in step 4 for the output power dynamics vary depending on the specific measurement as described for the conducted measurement in each case however the EIRP measurement is made on both polarisations and added as follows:

EIRPmeas_p(x) = Pmeas_p(x) + LA→C.

and

EIRPmeas = EIRPmeas_p1 + EIRPmeas_p2

where the declared beam is the measured signal for any two orthogonal polarizations (denoted p1 and p2).

Furthermore, the measurement is performed twice; once with the BS transmitting at Prated,c,EIRP on all PRBs (in case of NR) and then a second time with the BS transmitting on a single PRB. Both measurements are made in the same conformance direction in the same calibrated test setup. The ratio of these two EIRP levels is used to assess compliance for the OTA total power dynamic range.

#### 9.5.5.3	MU value derivation, FR1

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

The uncertainty budget descriptions are the same as those in table 9.2.6.3-1 with the addition descriptions in table 9.5.5.3-1.

The MU uncertainty assessment is shown in table 9.5.5.3-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 9.5.5.3-1: PWS MU value derivation for OTA total power dynamic range measurement

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| C3-2 | Total power dynamic range conducted uncertainty | 0.20 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.20 | 0.20 | 0.20 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.20 | 0.20 | 0.20 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 0.39 | 0.39 | 0.39 |

< End of Changes >

< Unchanged Text Deleted >

< Beginning of Changes >

### 9.7.5		Plane Wave Synthesizer

#### 9.7.5.1	Measurement system description

Measurement system description is captured in clause 7.6.

#### 9.7.5.2 	Test procedure

##### 9.7.5.2.1	Stage 1: Calibration

Calibration procedure for the Plane Wave Synthesizer is captured in clause 8.6.

##### 9.7.5.2.2	Stage 2: BS measurement

Reference procedure in clause 9.4.5.2.2 where in step 4 the appropriate measurement parameter is the EVM for the applicable test case and the specific test models. In this case, however testing should be carried out in the OTA conformance reference direction, not the beam peak direction of the OTA peak directions set reference direction.

#### 9.7.5.3 	MU value derivation, FR1

As both the wanted signal and the noise signal are at the same frequency they will be measured at the same time the requirement is effectively differential and most of the OTA chamber errors will cancel out.

The wanted signal will be beam formed and hence the errors used for the EIRP accuracy will be valid, however the co-channel noise may not be beam formed and hence could suffer different errors due to the chamber quite zone, and phase profile. These items are included in both the calibration error and the measurement error, as the requirement is differential if there is a difference between the wanted and the unwanted it will only be due to the measurement phase. The calibration errors will cancel as calibration is only done one so they will be the same for both wanted and unwanted signals.

Potentially, the EVM may vary in space due to different patterns of wanted signal and distortion. Thus for narrow beams, it may be possible that beam pointing and alignment errors could impact EVM results.

As EVM is also dependent on the phase of the calibrated path it is possible that phase ripple in the quite zone or elsewhere, which arises due to multipath reflections, may lead to frequency ripple and cause additional EVM errors which do not appear in a power accuracy analysis as done for EIRP accuracy.

The potential impacts of both beam pointing misalignment and scattering within the chamber on the received waveform and measurement accuracy were investigated. The potential deviation in the measured EVM arising from beam pointing errors was examined considering a worst case scenario, in which variation in space of EVM is maximal due to the ideal signal being correlated and the distortion uncorrelated; hence the impact of misalignment error would be the difference between array gain and element gain. Even in this circumstance, alignment errors of several degrees would not lead to a significant error in the measured EVM. Considering all likely chamber sizes, for E-UTRA any scattering would fall within the cyclic prefix of the OFDM symbol and hence not cause ISI. Furthermore, the likely delay spread of any scattering would relate to coherence bandwidths much larger than any UTRA/E-UTRA channel bandwidth. Even if the scattered energy would cause interference, the interference level would anyhow not lead to a significant EVM increase. Thus it was concluded that the impact scattering within the measurement chamber would be negligible.

The uncertainty causing by power variations when measuring OTA EVM is indicated in table 9.7.5.3-1:

Table 9.7.5.3-1: PWS MU value derivation for power uncertainty aspects of OTA EVM, FR1

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |  |  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A7-2a |  | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for DUT antenna | 0.05 |  | 0.14 |  | [0.14] |  | Rectangular |  | 1.73 |  | 1 |  | 0.03 |  | 0.08 |  | [0.08] |  |
| A7-4a |  | QZ ripple with DUT | 0.42 |  | 0.43 |  | [0.43] |  | Rectangular |  | 1.73 |  | 1 |  | 0.24 |  | 0.25 |  | [0.25] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.24 | 0.26 | [0.26] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 0.48 | 0.51 | [0.51] |

The CATR budget is carried out without consideration of the measurement equipment as this MU is given in %, converting to dB gives, for example:

2% is equivalent to 20*log10(2/100) = -33.98 dB

If the unwanted signal is 0.5 dB higher than the wanted due to the test system then this will be degraded to -33.48 dB, and

-33.48 dB is equivalent to:	10(-33.48/20) *100 = 2.12%

Additional error due to potential phase error has not been considered however the potential increase due to then OTA test equipment is well within the contribution allowable with a 1% linear MU.

NOTE:	Analysis of the phase uncertainties indicates that the contributions are not significant to affect the final MU value, however if future work indicates that phase or any other errors not related to amplitude calibration may affect the EVM measurement uncertainty the MU analysis may be re-examined.

< End of Changes >