3GPP TSG-RAN WG4 Meeting # 95-e 	                             R4-2007713

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:	4.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on reporting criteria for EN-DC

Document for:	Discussion

- Introduction

During the previous RAN4 meetings, the issues about reporting criteria were discussed. The principles for defining the number of reporting criteria are captured in the WF [1], and the corresponding changes are reflected in the CR [2]. However, there is still issues need to be further clarified based on the changes in the last meeting. We present our views in this paper.

- Discussion

In the last RAN4 94-e-bis meetings, the principles for defining the number of reporting criteria for defining NR reporting criteria for EN-DC is captured as follows:

- The reporting criteria per measurement category are specified in Table 8.2.2-1 in TS 36.133 and in Table 9.1.4.2-1 in TS 38.133 - The total number of reporting criteria (without counting other categories that the UE shall always support in parallel) is - Ecat,EN-DC,NR+Ecat,EN-DC,E-UTRA - The total number of reporting criteria will be specified in TS36.133 and TS 38.133 explicitly - Ecat,EN-DC,NRis specified in TS38.133 explicitly and requirements for this reporting criteria in TS36.133 shall refer to TS38.133. - Ecat,EN-DC,E-UTRAis specified in TS36.133 explicitly and requirements for this reporting criteria in TS38.133 shall refer to TS36.133. - When CA is configured, the reporting criteria will be increased by 9xn, where n is the number of SCell carrier frequencies, just the same principle as in the existing requirements in TS38.133 and TS36.133

It could be observed from the endorsed CR [2] that for UE configured with EN-DC, all the NR reporting criteria configured by PSCell and PCell including NR serving cells are refer to TS 38.133.

“ The UE, which is capable of supporting EN-DC operation with NR PSCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1 and according to table 9.1.4.2-1 in TS 38.133 [50]. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria Ecat,EN-DC,NR+Ecat,EN-DC,E-UTRA in total.

Ecat,EN-DC,NR is the total number of NR reporting criteria configured by NR PSCell and PCell, including NR PSCell and NR SCells carrier frequencies, as specified in TS 38.133 [50] for UE configured with EN-DC operation.”

In TS 38.133, Ecat,EN-DC,NR is described as follows:

Ecat,EN-DC,NR=10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and  is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies,

From our understanding, the NR reporting criteria for UE configured with EN-DC including cells on serving frequencies configured by PCell and PSCell, and inter-frequency measurement (10 as defined in Table 9.1.4.2-1 in TS 38.133) configured by PSCell and NR inter-RAT measurement (10 as defined Table 8.2.2-1 in TS 36.133) configured by PCell. Based on the current version of TS36.133, where the total number of NR reporting criteria configured by NR PSCell and PCell, including NR PSCell and NR SCells carrier frequencies is refer to TS38.133, it seems that the NR inter-RAT reporting criteria (10) is not reflected in equation in TS 38.133. So, the equation in TS 38.133 for EN-DC NR reporting criteria shall be modified as follows to reflect the total number including NR inter-RAT configured by PCell.

Ecat,EN-DC,NR=10+10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and  is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies,

Proposal 1:

The equation in TS 38.133 for EN-DC NR reporting criteria shall be modified as follows to reflect the total number including NR inter-RAT configured by PCell.

Ecat,EN-DC,NR=10+10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and  is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies,

We also present an accompanied CR to capture the change mentioned in this paper [3].

- Conclusion

Proposal 1:

The equation in TS 38.133 for EN-DC NR reporting criteria shall be modified as follows to reflect the total number including NR inter-RAT configured by PCell.

Ecat,EN-DC,NR=10+10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and  is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies,

References

[1] R4-2005266, WF on remaining issue on NR reporting criteria, ZTE

[2] R4-2005429, Draft CR to 36.133 for NR reporting criteria, ZTE

[3] R4-2007710