3GPP TSG-RAN4 Meeting #95-e	R4-2007840

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0845 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption requirements for SCell dormancy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The requirement on switching interruption between SCell dormancy and non-dormancy is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add interruption requirements for switching between SCell dormancy and non-dormancy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirements for switching between SCell dormancy and non-dormancy |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections 8.2.1.2.X, 8.2.2.2.Y, 8.2.3.2.Z, 8.2.4.2.A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.1.2.X	Interruptions due to SCell dormancy

8.2.1.2.X.1	Interruptions due to SCell dormancy switch

When one SCell in SGC is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7],

-	the UE is allowed an interruption on any active serving cell in SCG as defined in clause 8.2.1.2.7

The requirement applies for dormancy switch triggered when UE is in DRX active time.

Editor’s Note: FFS if the requirement applies for dormancy switch triggered when UE is outside DRX active time.

8.2.1.2.X.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells in SCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PSCell or any activated SCell with up to [0.5]% probability of missed ACK/NACK.

8.2.1.2.X.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells in SCG are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on PSCell or any activated SCell with up to [0.5]% probability of missed ACK/NACK.

<End of Change 1>

<Start of Change 2>

8.2.2.2.Y	Interruptions due to SCell dormancy

8.2.2.2.Y.1	Interruptions due to SCell dormancy switch

When one SCell is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7],

-	the UE is allowed an interruption on any active serving cell as defined in clause 8.2.2.2.5

The requirement applies for dormancy switch triggered when UE is in DRX active time.

Editor’s Note: FFS if the requirement applies for dormancy switch triggered when UE is outside DRX active time.

8.2.2.2.Y.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on any activated SCell with up to [0.5]% probability of missed ACK/NACK.

8.2.2.2.Y.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on any activated SCell with up to [0.5]% probability of missed ACK/NACK.

<End of Change 2>

<Start of Change 3>

8.2.3.2.Z	Interruptions due to SCell dormancy

8.2.3.2.Z.1	Interruptions due to SCell dormancy switch

When one SCell is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7],

-	the UE is allowed an interruption on any active serving cell in MCG as defined in clause 8.2.3.2.7

The requirement applies for dormancy switch triggered when UE is in DRX active time.

Editor’s Note: FFS if the requirement applies for dormancy switch triggered when UE is outside DRX active time.

8.2.3.2.Z.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

8.2.3.2.Z.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on PCell or any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

<End of Change 3>

<Start of Change 4>

8.2.4.2.A	Interruptions due to SCell dormancy

8.2.4.2.A.1	Interruptions due to SCell dormancy switch

When one SCell is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7],

-	the UE is allowed an interruption on any active serving cell in MCG as defined in clause 8.2.4.2.5

The requirement applies for dormancy switch triggered when UE is in DRX active time.

Editor’s Note: FFS if the requirement applies for dormancy switch triggered when UE is outside DRX active time.

8.2.4.2.A.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PSCell and any activated SCell in SCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

When one or more SCells in SCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PCell and any activated SCell in MCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

8.2.4.2.A.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PSCell and any activated SCell in SCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

When one or more SCells in SCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PCell and any activated SCell in MCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

<End of Change 4>