3GPP TSG-RAN4 Meeting #95-e	R4-2009012

# Electronic meeting, 25th May – 5th June, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0639 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on scheduling restriction for CSI-RS based intra-frequency measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  |  | Date: |  | 2020-06-01 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CSI-RS L3 measurement was introduced to RAN4 in Rel-16. The CR aims to add restrictions in the scheduling availability for UE during CSI-RS L3 intra-frequency measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the scheduling restriction for CSI-RS based intra-frequency measurement for RRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based UE measurement requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.9.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.9.2.x	Scheduling availability of UE during CSI-RS based intra-frequency measurements

UE shall be capable of measuring without measurement gaps when CSI-RS resources are completely contained in the active BWP of the UE [3]. Note the CSI-RS symbol to be measured is indicated in firstOFDMSymbolInTimeDomain included in CSI-RS-ResourceConfigMobility [2] for RRM. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction.

##### 9.9.2.x.1	Scheduling availability of UE performing CSI-RS based measurements with simultaneous serving cell reception in FDD bands

When the UE performs CSI-RS based intra-frequency measurements and the serving cell PDCCH/PDSCH reception simultaneously, the following restrictions apply when UE is not able to support the mixed numerology of serving cell data and CSI-RS L3 mobility.

Editor’s note: following paragraph is pending on the agreement on Issue 2-5-1 tracked in [95e][226]NR_CSIRS_L3meas_RRM_2 and it is also related to the agreement on the synchronization assumption between the serving cell and its neighbour cells.

- If a CSI-RS resource is configured with an associated SSB, the UE is not expected to receive PDCCH/PDSCH/TRS on CSI-RS symbols to be measured and on 1 data symbol before each consecutive CSI-RS symbols to be measured and 1 data symbol after each consecutive CSI-RS symbols to be measured within the configured slot as indicated in slotConfig [2] of the corresponding CSI-RS resource.

- If the associated SSB is not configured, above restriction is not needed.

##### 9.9.2.x.2	Scheduling availability of UE performing CSI-RS based measurements in TDD bands

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to the UL and DL collision.

Editor’s note: following paragraph is pending on the agreement on Issue 2-5-2 tracked in [95e][226]NR_CSIRS_L3meas_RRM_2.

- The UE is not expected to transmit PUCCH/PUSCH/SRS on CSI-RS symbol to be measured and on 1 OFDM symbol before each CSI-RS symbol to be measured and 1 OFDM symbol after each CSI-RS symbol to be measured within the configured slot as indicated in slotConfig [2] of the corresponding CSI-RS resource.

##### 9.9.2.x.3	Scheduling availability of UE performing CSI-RS based measurements with non-serving UE Rx beam in FR2

Editor’s note: the entire sub-section is pending on the agreement on Issue 2-5-3 tracked in [95e][226]NR_CSIRS_L3meas_RRM_2.

When the UE performs CSI-RS based intra-frequency measurements for L3 mobility management, if the associated SSB is configured but not QCLed with the corresponding CSI-RS, UE sweeps the Rx beam for refining the measurement. The following restrictions apply.

- The UE is not expected to receive PDCCH/PDSCH/TRS on CSI-RS symbols to be measured and on 1 OFDM symbol before each CSI-RS symbol to be measured and 1 OFDM symbol after each CSI-RS symbol to be measured within the configured slot as indicated in slotConfig [2] of the corresponding CSI-RS resource.

- If the associated SSB is not configured, above restriction is not needed.

##### 9.9.2.x.4	Scheduling availability of UE performing CSI-RS based measurements with simultaneous L1 measurements

When the UE performs CSI-RS based intra-frequency measurements for L3 mobility management, the following restrictions apply if the CSI-RS for L3 mobility shares the same OFDM symbol as the CSI-RS or SSB for L1-RSRP measurement, or CSI-RS for CQI.

Editor’s note: following paragraphs are pending on the agreement on Issue 2-5-4 tracked in [95e][226]NR_CSIRS_L3meas_RRM_2.

- If the UE supports the simultaneous reception of CSI-RS of neighbour cell and an SSB, the UE shall be able to perform SSB for L1-RSRP and CSI-RS for L3 mobility without restrictions.

- If the UE does not support the simultaneous reception of CSI-RS of neighbour cell and an SSB, UE is not expected to receive CSI-RS for L3 mobility in the OFDM symbol that overlaps with an SSB.

- When both CSI-RS for L1-RSRP or CQI shares the same OFDM symbol as the CSI-RS for L3 mobility, the UE is not expected to receive CSI-RS for L3 mobility in the OFDM symbol that overlaps with the CSI-RS for L1-RSRP or CQI.

<End of Change 1>