3GPP TSG-RAN WG4 Meeting # 96-e 		                       R4-2009828

Electronic Meeting, 17-28 Aug., 2020

Title: 	On general requirement and additional requirement for NR V2X

Source: 	CATT

Agenda item:	7.3.1

Document for:	Discussion

- Introduction

In RAN4#95e meeting, the applicability of general requirements for NR V2X was discussed [1]. The general requirements and additional requirements are required to be clarified for NR V2X. This contribution will provide our view on this topic.

- Discussion

In RAN4#95e meeting, the following options regarding general requirements and additional requirements were discussed based on [1]:

Issue #2-1-1: General part in TS

- Option 1

  - General requirements apply to V2X (suffix E) unless otherwise stated, so V2X requirements (suffix E) are written only when the requirement differs from the general. This eliminates suffix E clauses.

- Option 2

  - For pedestrian-V2X UEs, both the general core requirements and suffix E core requirements apply

  - For vehicular UEs supporting the only the NR-V2X sidelink, the suffix E core requirements apply

  - Note: At this time the terms ‘vehicular UE’ and ‘pedestrian UE’ have not been defined in the requirements section of the TR, however we anticipate in future this may change.

As per TS 38.101-1 Section 4.3, the following description that explains the relationship between general requirements and addition requirements is provided as follows:

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffix A, B, C, D and E) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffix A, B, C, D and E) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.

The first issue to be clarified is whether and how to define the terms “vehicular V2X UE” and “pedestrian V2X UE”.  This should be dependent on the real scenarios for V2X. In real scenarios, vehicular V2X UEs have the demand to communicate with pedestrian V2X UEs. However, vehicular V2X UEs and pedestrian V2X UEs have the different hardware implementations such that the different requirements should apply. For example, the cable loss caused by longer cable distributed in vehicular V2X UEs needs to be considered in the RENSENS requirements, but it is not required in pedestrian V2X UEs. So the different types of V2X UE, i.e. vehicular V2X UE and pedestrian V2X UE, should be first clarified and defined based on the real scenarios for V2X.

Proposal 1: Different types of V2X UE, i.e. vehicular V2X UE and pedestrian V2X UE, should be first clarified and defined based on the real scenarios for V2X.

Given the different types of V2X UE characterized by different hardware implementations, different requirements will apply respectively. The part of additional requirements is exclusively specified for NR V2X while the rest directly reuse the general requirements of NR Uu. The common understanding is that the additional requirements specified in the specific suffix apply for the vehicular V2X UE at least. In current specifications, the additional requirements are all particularly stated to be only applied to the terminal that supports this feature, e.g. NR V2X UE.

When it comes to pedestrian V2X UEs, the tighter requirements should apply compared to vehicular V2X UEs. In this case, both the general requirements and the additional requirements can be applied to pedestrian V2X UEs. If there is a difference in requirement between the general requirements and the additional requirements, the tighter requirements will be applicable providing that there is no additional declaration. This is exactly the way what the specification suffix information specified in TS 38.101-1 presents.

Overall, to differentiate the requirements of vehicular V2X UEs and pedestrian V2X UEs, the “V2X UE” in requirement part in current specifications should be changed to “vehicular V2X UE”. For pedestrian V2X UE, no additional statement in requirements is needed for simplicity as per the specification suffix information specified in TS 38.101-1. Indeed, the additional statement in requirement for pedestrian V2X UE, like “the tighter requirements between the general requirements and additional requirements apply for pedestrian V2X UEs”, can be also acceptable.

Proposal 2: The additional requirements specified in the suffix should apply for vehicular V2X UEs, while the tighter requirements between general requirements and additional requirements should apply for pedestrian V2X UEs.

Proposal 3: The “V2X UE” in requirement part in current specifications should be changed to “vehicular V2X UE”. For pedestrian V2X UE, no additional statement in requirements is needed.

- Conclusion

This contribution discusses the applicable requirements for vehicular V2X UE and pedestrian V2X UE. The following proposals are derived:

Proposal 1: Different types of V2X UE, i.e. vehicular V2X UE and pedestrian V2X UE, should be first clarified and defined based on the real scenarios for V2X.

Proposal 2: The additional requirements specified in the suffix should apply for vehicular V2X UEs, while the tighter requirements between general requirements and additional requirements should apply for pedestrian V2X UEs.

Proposal 3: The “V2X UE” in requirement part in current specifications should be changed to “vehicular V2X UE”. For pedestrian V2X UE, no additional statement in requirements is needed.

- Reference

[1]	R4-2008446, WF on TR/TS structures and common terminology for NR V2X UE, Qualcomm, RAN4#95e