3GPP RAN WG4 Meeting #96-e		R4-2009957

Electronic meeting, 17th – 28th August 2020

Agenda item:	10.24.1

Source:	Apple Inc.

Title:	Preliminary views on the new 47 GHz band

WI/SI:	NR_47GHz_Band

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

During RAN #88e a work item to define a new band in the 47.2 – 48.2 GHz frequency range was approved [1].  This contribution provides our preliminary views on the regulatory emission requirements which are currently available as well as views on the scope of the requirements needed to introduce this new band as well.

# 2	Discussion

## 2.1	Emission requirements

The FCC regulation for Upper Microwave Flexible Service [2] includes the frequencies in the range of 47.2 – 48.2 GHz and defines the following requirements:

-	For mobile stations, the average power of the sum of all antenna elements is limited to a maximum EIRP of +43 dBm (§ 30.202)

-	The conductive power or the total radiated power of any emission outside a licensee's frequency block shall be −13 dBm/MHz or lower. However, in the bands immediately outside and adjacent to the licensee's frequency block, having a bandwidth equal to 10 percent of the channel bandwidth, the conductive power or the total radiated power of any emission shall be −5 dBm/MHz or lower. (§ 30.203)

-	Compliance with this provision is based on the use of measurement instrumentation employing a resolution bandwidth of 1 megahertz or greater.

-	When measuring the emission limits, the nominal carrier frequency shall be adjusted as close to the licensee's frequency block edges as the design permits.

-	The measurements of emission power can be expressed in peak or average values.

Observation 1:	The Max EIRP requirement of 43 dBm for Power Class 3 defined in TS38.101-2 is applicable to the new 47 GHz band.

TS38.101-2 defines the following general emission mask:

Table 1: General NR spectrum emission mask for frequency range 2 [38.101-2, 6.5.2.1]

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |

Observation 2:	The general NR spectrum emission mask for FR2 is aligned with the FCC regulation.

The FCC regulation provides the following table of frequency allocations [3]:

Figure 1: FCC table of frequency allocations

The following footnotes to the table of frequency allocations are relevant to mobile service operation in the 47.2 – 48.2 GHz frequency range:

-	NG65 In the bands 24.75-25.25 GHz, 47.2-48.2 GHz, and 50.4-51.4 GHz, stations in the fixed and mobile services may not claim protection from individually licensed earth stations authorized pursuant to 47 CFR 25.136. However, nothing in this footnote shall limit the right of Upper Microwave Flexible Use Service licensees to operate in conformance with the technical rules contained in 47 CFR part 30. The Commission reserves the right to monitor developments and to undertake further action concerning interference between Upper Microwave Flexible Use Service and Fixed-Satellite Service, including aggregate interference to satellite receivers, if appropriate.

-	US156 In the bands 49.7-50.2 GHz and 50.4-50.9 GHz, for earth stations in the fixed-satellite service (Earth-to-space), the unwanted emissions power in the band 50.2-50.4 GHz shall not exceed −20 dBW/200 MHz (measured at the input of the antenna), except that the maximum unwanted emissions power may be increased to −10 dBW/200 MHz for earth stations having an antenna gain greater than or equal to 57 dBi. These limits apply under clear-sky conditions. During fading conditions, the limits may be exceeded by earth stations when using uplink power control.

-	US246 No station shall be authorized to transmit in the following bands: 73-74.6 MHz, 608-614 MHz, except for medical telemetry equipment [1] and white space devices, [2] 1400-1427 MHz, 1660.5-1668.4 MHz, 2690-2700 MHz, 4990-5000 MHz, 10.68-10.7 GHz, 15.35-15.4 GHz, 23.6-24 GHz, 31.3-31.8 GHz, 50.2-50.4 GHz, 52.6-54.25 GHz, 86-92 GHz, 100-102 GHz, 109.5-111.8 GHz, 114.25-116 GHz, 148.5-151.5 GHz, 164-167 GHz, 182-185 GHz, 190-191.8 GHz, 200-209 GHz, 226-231.5 GHz, 250-252 GHz.

The FCC issued the following order related to fixed satellite service (FSS) use of the 48.2 – 50.2 GHz frequency range [4]:

Further, the FCC issued the following order related to FSS use of the 50.4 – 51.4 GHz frequency range [5]:

Most recently, the FCC issued the following order related to FSS use of the 50.4 – 51.4 GHz fruency range [6]:

Observation 3:	RAN4 should study whether A-MPR is needed for UEs operating in the new 47 GHz band to comply with the FCC requirement of unwanted emissions power in the band 50.2-50.4 GHz shall not exceed −20 dBW/200 MHz.

Observation 4:	FCC does not define unwanted emission limits for FSS  protection in the 48.2 – 50.2 GHz frequency range beyond the general emission maks.

Observation 5:	There exists the potential for the FCC to take further action to define emission requirements for the protection of FSS in 24.75-25.25 GHz, 47.2-48.2 GHz, and 50.4-51.4 GHz frequency ranges.

Proposal 1:	Companies are encouraged to reach a common understanding of the regulatory requirements for the new 47 GHz band.

As described in the WID [1], the ITU has designated the frequency range as a global band.  However, regulatory requirements are not yet available by regulators other than the FCC.

Observation 6:	At this time, only regional regulations provided by the FCC are available for the new 47 GHz band.

Given the available of regional regulation by the FCC, RAN4 should discuss how to define general and regional emission requirements in a way that can allow for future extensions of the requirements to additonal regions.

Proposal 2:	RAN4 should discuss how to define general and regional emission requirements in a way that can allow for future extensions of the requirements to additonal regions.

## 2.2	Work scope

To determine the total work scope for UE RF associated with the introduction of the new 47 GHz band, we consider the requirements currently listed in TS38.101-2, identify requirements which are band-dependent, and further evaluate potential work effort needed to specify the related requirements for the new band.  Table 2 below summarizes the evaluation.

Table 2: Potential work scope for the new 47 GHz band

| Clause | Band dependent? | Potential work scope for 47 GHz |
| --- | --- | --- |
| 6.2.1 UE maximum output power | yes | mmWave array EM sim, NW performance sim, multi-band framework |
| 6.2.2 UE maximum output power reduction | no |  |
| 6.2.3 UE maximum output power with additional requirements | yes | TBD whether NS values specific to 47 GHz band are needed |
| 6.2.4 Configured transmitted power | no |  |
| 6.2A.1 UE maximum output power for CA | yes | Reuse requirements from 6.2.1 under CA configuration |
| 6.2A.2 UE maximum output power reduction for CA | no |  |
| 6.2A.3 UE maximum output power with additional requirements for CA | yes | TBD whether NS values specific to 47 GHz band are needed |
| 6.2A.4 Configured transmitted power for CA | no |  |
| 6.2D.1 UE maximum output power for UL-MIMO | yes | Reuse requirements from 6.2.1 under UL-MIMO configuration |
| 6.2D.2 UE maximum output power for modulation / channel bandwidth for UL-MIMO | no |  |
| 6.2D.3 UE maximum output power with additional requirements for UL- MIMO | yes | Reuse requirements from 6.2.3 under UL-MIMO configuration |
| 6.2D.4 Configured transmitted power for UL-MIMO | no |  |
| 6.3.1 Minimum output power | no |  |
| 6.3.2 Transmit off power | no |  |
| 6.3.3 Transmit ON/OFF time mask | no |  |
| 6.3.4 Power control | no |  |
| 6.3A.1 Minimum output power for CA | no |  |
| 6.3A.2 Transmit off power for CA | no |  |
| 6.3A.3 Transmit ON/OFF time mask for CA | no |  |
| 6.3A.4 Power control for CA | no |  |
| 6.3D.1 Minimum output power for UL-MIMO | no |  |
| 6.3D.2 Transmit off power for UL-MIMO | no |  |
| 6.3D.3 Transmit ON/OFF time mask for UL-MIMO | no |  |
| 6.3D.4 Power control for UL-MIMO | no |  |
| 6.4 Transmit signal quality | no |  |
| 6.4 Transmit signal quality for CA | no |  |
| 6.4 Transmit signal quality for UL-MIMO | no |  |
| 6.5.1 Occupied bandwidth | no |  |
| 6.5.2.1 Spectrum emission mask | no |  |
| 6.5.2.3 Adjacent channel leakage ratio | yes | New round of coexistence simulations if ACLR from n260 cannot be reused |
| 6.5.3 Spurious emissions (general) | no |  |
| 6.5.3.1 Spurious emission band UE co-existence | yes | Requirements protecting the new band and existing bands are needed |
| 6.5.3.2 Additional spurious emissions | yes | TBD whether NS values specific to the 47 GHz band are needed |
| 6.5A.1 Occupied bandwidth for CA | no |  |
| 6.5A.2.1 Spectrum emission mask for CA | no |  |
| 6.5A.2.3 Adjacent channel leakage ratio for CA | yes | Reuse requirements from 6.5.2.3 under CA configuration |
| 6.5A.3 Spurious emissions for CA (general) | no |  |
| 6.5A.3.1 Spurious emission band UE co-existence for CA | yes | Requirements protecting the new band and existing bands are needed |
| 6.5A.3.2 Additional spurious emissions for CA | yes | TBD whether NS values specific to the 47 GHz band are needed |
| 6.5D.1 Occupied bandwidth for UL-MIMO | no |  |
| 6.5D.2.1 Spectrum emission mask for UL-MIMO | no |  |
| 6.5D.2.3 Adjacent channel leakage ratio for UL-MIMO | yes | Reuse requirements from 6.5.2.3 under UL-MIMO configuration |
| 6.5D.3 Spurious emissions for UL-MIMO (general) | no |  |
| 6.5D.3.1 Spurious emission band UE co-existence for UL-MIMO | yes | Reuse requirements from 6.5.3.1 under UL-MIMO configuration |
| 6.5D.3.2 Additional spurious emissions for UL-MIMO | yes | TBD whether NS values specific to the 47 GHz band are needed |
| 6.6 Beam correspondence | yes | Beam correspondence tolerance sim |
| 6.6A Beam correspondence for CA | no |  |
| 7.3.2 Reference sensitivity power level | yes | Determine common assumption on UE receiver noise figure, reuse mmWave array sim and multi-band framework |
| 7.3.4 EIS spherical coverage | yes | Reuse mmWave array sim and multi-band framework |
| 7.3A.2 Reference sensitivity power level for CA | yes | Reuse requirements from 7.3.2 under CA configuration |
| 7.3D Reference sensitivity for UL-MIMO | yes | Reuse requirements from 7.3.2 under UL-MIMO configuration |
| 7.4 Maximum input level | no |  |
| 7.4A Maximum input level for CA | no |  |
| 7.4D Maximum input level for UL-MIMO | no |  |
| 7.5 Adjacent channel selectivity | yes | New round of coexistence simulations if ACS from n260 cannot be reused |
| 7.5 Adjacent channel selectivity for CA | yes | Reuse requirements from 7.5 under CA configuration |
| 7.5 Adjacent channel selectivity for UL-MIMO | yes | Reuse requirements from 7.5 under UL-MIMO configuration |
| 7.6.2 In-band blocking | yes | Analyze IBB scenario for the 47 GHz band and define new requirements |
| 7.6A.2 In-band blocking for CA | yes | Analyze IBB scenario for the 47 GHz band and define new requirements |
| 7.6D Blocking characteristics for UL-MIMO | yes | Reuse requirements from 7.6.2 under UL-MIMO configuration |
| 7.9 Spurious emissions | no |  |

To address the band-dependent RF requirements identified in Table 2, we suggest making progress in the following work areas:

-	For single-carrier TX requirements:  identify the regulatory requirements for 47 GHz operation in the US and investigate whether global regulation exists for this band, determine whether max TRP limit from band n260 can be reused, derive MPR requirements and, if needed A-MPR to comply with regional requirements, derive peak EIRP requirements, derive spherical coverage EIRP requirements.

-	For single-carrier RX requirements: derive REFSENS requirements, derive spherical coverage EIS requirements, define in-band blocking and ACS requirements.

-	For multi-band and CA aspects:  define the multi-band relaxation framework, define TX & RX requirements for inter-band CA, including beam management type assumptions.

-	We recommend focusing the effort on Power Class 3 requirements, thereby recognizing commercial needs for a handheld UE to support the new 47 GHz band

As was done with the introduction of Band n259, the big CR introducing requirements for the band should only be agreed after all of the requirements are understood with agreement on corresponding parameters and values.

Proposal 3:	The big CR introducing requirements for the new 47 GHz band shall only be agreed after all of the requirements are understood with agreement on corresponding parameters and values, including single-carrier requirements, CA aspects, and the multi-band relaxation framework.

# 3	Conclusions

In this discussion paper we have presented our preliminary views on the regulatory emission requirements which are currently available and considerations for the work plan related to the new 47 GHz band.  The following observations and proposals have been made:

Observation 1:	The Max EIRP requirement of 43 dBm for Power Class 3 defined in TS38.101-2 is applicable to the new 47 GHz band.

Observation 2:	The general NR spectrum emission mask for FR2 is aligned with the FCC regulation.

Observation 3:	RAN4 should study whether A-MPR is needed for UEs operating in the new 47 GHz band to comply with the FCC requirement of unwanted emissions power in the band 50.2-50.4 GHz shall not exceed −20 dBW/200 MHz.

Observation 4:	FCC does not define unwanted emission limits for FSS  protection in the 48.2 – 50.2 GHz frequency range beyond the general emission maks.

Observation 5:	There exists the potential for the FCC to take further action to define emission requirements for the protection of FSS in 24.75-25.25 GHz, 47.2-48.2 GHz, and 50.4-51.4 GHz frequency ranges.

Observation 6:	At this time, only regional regulations provided by the FCC are available for the new 47 GHz band.

Proposal 1:	Companies are encouraged to reach a common understanding of the regulatory requirements for the new 47 GHz band.

Proposal 2:	RAN4 should discuss how to define general and regional emission requirements in a way that can allow for future extensions of the requirements to additonal regions.

Proposal 3:	The big CR introducing requirements for the new 47 GHz band shall only be agreed after all of the requirements are understood with agreement on corresponding parameters and values, including single-carrier requirements, CA aspects, and the multi-band relaxation framework.

# 4	References

RP-201232, " New WID: introduction of NR 47 GHz band", T-Mobile USA, Dish Network, 3GPP RAN #88e, June 2020

47 CFR § 30, “Upper Microwave Flexible Service,” FCC Code of Federal Regulations

47 CFR § 2.106, “Table of Frequency Allocations,” FCC Code of Federal Regulations

FCC 17-152, “Use of Spectrum Bands Above 24 GHz For Mobile Radio Services,” November 16, 2017

FCC 18-73, “Use of Spectrum Bands Above 24 GHz For Mobile Radio Services,” June 8, 2018

FCC 19-30, “Use of Spectrum Bands Above 24 GHz For Mobile Radio Services,” April 15, 2019