3GPP RAN4#96-e									R4-2009989

Online Meeting, August 17th – August 28th, 2020

Title: 	TP for IAB-MT classes

Source: 	Qualcomm Incorporated

Agenda item:	7.4.1.2

Document for:	Discussion

# Introduction

RAN4 has been discussing the requirements of IAB networks during several meetings [1]. The definition of IAB-MT classes is still absent in the spec [2]. We provide a TP for the definition in this contribution.

# Discussion

RAN4 agreed to the following regarding the definition of IAB-MT classes during the last meeting [3]

The IAB-MT class definition options are: Option 1: use target deployment scenario and below parameters for IAB-MT class definition and description: - option 1a: Minimum distance - option 1b: Typical distance Option 2: use target deployment scenario for IAB class definition and description from the following options: - option 2a: (minimal information) Wide Area IAB-MT nodes are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios. Local Area IAB-MT nodes are characterised by requirements derived from Pico Cell and /or Micro Cell scenarios. - option 2b: (additional link based information) Wide Area IAB-MT nodes are characterised by requirements derived from Macro Cell and Micro Cell scenarios where the path loss is higher but stable. Local Area IAB-MT nodes are characterised by requirements derived from Pico Cell and Micro Cell scenarios where the path loss is lower but more variable Agreement: Using option 2a for TS , other information can be introduced into TR if needed.

# TP for Section 4.4 of 38.174

-------------------------------------------------------------------  TP starts ----------------------------------------------------------

## IAB classes

### IAB-DU classes

The requirements in this specification apply to Wide Area IAB-DU, Medium Range IAB-DU and Local Area IAB-DU unless otherwise stated. The associated deployment scenarios for each class are exactly the same for IAB-DU with and without connectors.

For IAB-DU type 1-O and 2-O, BS classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

For IAB-DU type 1-H, BS classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

### IAB-MT classes

The requirements in this specification apply to Wide Area IAB-MT and Local Area IAB-MT classes unless otherwise stated. The associated deployment scenarios for each class are exactly the same for IAB-MT with and without connectors.

For IAB type 1-O, 2-O and 1-H, IAB-MT classes are defined as indicated below:

- Wide Area IAB-MT nodes are defined by requirements derived from Macro Cell and/or Micro Cell scenarios.

- Local Area IAB-MT nodes are defined by requirements derived from Pico Cell and /or Micro Cell scenarios.

-------------------------------------------------------------------  TP ends ----------------------------------------------------------

# References

- Chairman’s notes, 3GPP TSG RAN WG4 #95e.

- 3GPP TS 38.174 V0.1.0, Integrated Access and Backhaul Radio Transmission and Reception (Release 16).

- R4-2008767, “WF on IAB-MT class descriptions”, Huawei, 3GPP TSG-RAN WG4 Meeting #95-e.