3GPP TSG-RAN4 Meeting #95e	R4-2010051

# Online, August 17-28, 2020

Agenda item:	7.12.2.1

Source:	Apple

Title:   On MRTD requirement for FR2 inter-band CA

Document for:	Approval

# Introduction

In RAN4 #95e-bis meeting, MRTD for FR2 inter-band DL CA was discussed and the following agreements have been made

- At least 260ns is feasible from UE perspective

- At least 3us MRTD is feasible from network perspective for co-located deployments

- Further study feasibility to support up to 3us MRTD from UE perspective under assumption of co-located deployment in terms of impact on performance (e.g. possible scheduling restrictions)

  - Option 1: complete this work by Rel-16. If not consensus can be reached by RAN4#96e, do not define CBM RRM requirements in Rel-16

  - Option 2: continue discussing this in Rel-17. No CBM RRM requirements in Rel-16 are defined.

- If no consensus can be made to define MRTD value for CBM and the study on the feasibility to support up to 3us MRTD by RAN4#96e, no CBM RRM requirements in Rel-16 are defined

- 8us MRTD is defined for IBM based FR2 inter-band CA

In this contribution, MRTD for FR2 inter-band CA is further discussed.

# MRTD for common and independent beam management

The current MRTD requirement in TS38.133 is as below for FR2 inter-band CA.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2; 8. Between FR1 and FR2; 25.

When MRTD for FR2 inter-band CA was discussed and agreed, the discussion is mainly based on L+M band combination where independent beam management is typically assumed. In this case, it is proposed to make clarification to limit Table 7.6.4-2 to independent beam management only.

Proposal 1: The following revision is proposed for TS38.133

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2 | 8note1 |
| Between FR1 and FR2 | 25 |
| Note1: this MRTD requirement applies to independent beam management only. |  |

- Common beam management (CBM)

Common beam management is assumed for FR2 intra-band CA since Rel-15. In this section, the assumptions of CBM and the related RRM requirements impacts are summarized.

All RRM requirements for intra-band FR2 are defined based on the assumption of CBM, which is defined in 3.6.3, TS38.133 as

3.6.3 Applicability for intra-band FR2

For the requirements in RRC connected state specified in this version of the specification, UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in the same band in FR2. Otherwise, the UE is not supposed to satisfy any requirements for SCell.

This assumption in section 3.6.3 consists of multiple important information for CBM, which includes

- Same Tx beam is assumed for all CC during one OFDM symbol across all CCs

- Symbol level alignment is assumed across all CC

If symbol boundary is not aligned with CP, one OFDM symbol from one CC will be partially overlapped with two OFDM symbols from the other CC. In this case, same Tx beam can never be assumed on one OFDM symbol across all CCs. In other words, Rx beam switch will happen in the middle of OFDM symbols for some CCs and the corresponding symbols will be interrupted. Since Rx beam switching is UE’s decision, NW will have no information which symbols are interrupted. In the worst case, UE will be failed to receive the whole slot where one or more than one symbol are interrupted.

Observation 1: For FR2 intra-band CA, symbol level alignment within CP length is essential to guarantee the same downlink spatial domain transmission filter on one OFDM symbol.

With this assumption, the follow RRM requirements are specified

- In section 8.3.2 TS38.133, all FR2 being activated SCell can be considered as known cell if there is at least one active serving cell on that FR2 band

Typically, a cell can be defined as known cell only if the corresponding measurement is reported not long time ago such that the related timing and spatial information can still be assumed up to date. Otherwise, time and spatial tracking is needed. The aforementioned RRM requirement suggest that fine timing and spatial information from one CC can be directly used by the other CC when common beam management is assumed.

Observation 2: FR2 SCell activation requirements for intra-band CA suggests that when common beam management is assumed, fine timing and spatial information from one CC can be directly re-used by the other CC. This again makes it important that symbol level alignment should be with CP length.

The other related RRM requirements where L1 or L3 measurement on one CC can be reused for the other CCs include L3-RSRP/RSRQ, CBD, BFD etc.

Observation 3: For CBM based FR2 intra-band CA, L1 and L3 measurements on one CC can be reused for all the other CC. That implies the same Tx and Rx beams used across all CCs per OFDM symbol.

As defined in FR2 intra-band CA, CBM in FR2 inter-band CA should be also based on the assumption that the same Tx and Rx beams used across all CCs per OFDM symbol. That means UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in all CCs. To guarantee the same DL spatial domain transmission filter, the common beam management is only viable when the gNB for all CC are collocated and all CC should be aligned in slot level with MRTD less than CP. If different SCS is configured for different CC, the CP length of the largest SCS is considered. Meanwhile, it should be assumed as the baseline that single Rx antenna array at UE is shared by all CC.

Proposal 2: Common beam management in FR2 inter-band CA is defined as that UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in all CCs.

Proposal 3: In case of common beam management, it is assumed that gNB for all CC are collocated and single Rx antenna array at UE is shared by all CC.

Obviously, BS TAE requirement for inter-band CA, i.e. 3us, is too large for common beam management. More specifically, when MRTD exceeds ½ CP length, it is impossible to assume the the same downlink spatial domain transmission filter on one OFDM symbol in all CCs. With this it is proposed to reuse FR2 intra-band CA MRTD for FR2 inter-band CA in case of common beam management.

Proposal 4: It is proposed to reuse FR2 intra-band CA MRTD, i.e. 260ns for the MRTD of FR2 inter-band CA in case of common beam management.

# Conclusion

In this contribution, MRTD for FR2 inter-band CA is further studied and it is observed that

Observation 1: For FR2 intra-band CA, symbol level alignment within CP length is essential to guarantee the same downlink spatial domain transmission filter on one OFDM symbol.

Observation 2: FR2 SCell activation requirements for intra-band CA suggests that when common beam management is assumed, fine timing and spatial information from one CC can be directly re-used by the other CC. This again makes it important that symbol level alignment should be with CP length.

Observation 3: For CBM based FR2 intra-band CA, L1 and L3 measurements on one CC can be reused for all the other CC. That implies the same Tx and Rx beams used across all CCs per OFDM symbol.

Based on this, it is proposed that

Proposal 1: The following revision is proposed for TS38.133

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2 | 8note1 |
| Between FR1 and FR2 | 25 |
| Note1: this MRTD requirement applies to independent beam management only. |  |

Proposal 2: Common beam management in FR2 inter-band CA is defined as that UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in all CCs.

Proposal 3: In case of common beam management, it is assumed that gNB for all CC are collocated and the same Rx antenna array at UE is shared by all CC.

Proposal 4: It is proposed to reuse FR2 intra-band CA MRTD, i.e. 260ns for the MRTD of FR2 inter-band CA in case of common beam management.

# References