3GPP TSG-RAN4 Meeting #95e	R4-2010052

# Online, August 17-28, 2020

Agenda item:	7.14.1.1

Source:	Apple

Title:   On other remaining issues of CSI-RS based L3 measurement

Document for:	Approval

# Introduction

In this contribution, the following issue is discussed

- FFS on whether to define additional configuration {D=1 with PRBs ≥ 96} for the requirements

# Additional configuration {D=1 with PRBs ≥ 96} for the requirements

It has been discussed for many meeting cycles to introduce additional CSI-RS configuration {D=1 with PRB≥96}, in addition to {D=1 with PRB≥48}. Based on all simulations from different companies, there is no significant performance differences in terms of accuracy between these two configurations. Meanwhile, the related system impact is unclear either, from the perspective of measurement delay and wideband operation. There is similar discussion in LTE to introduce WB-RSRP, where the benefit from cell loading perspective is well justified. It is proposed before introducing additional CSI-RS configuration, similar practice should be done also.

It is proposed

Proposal: Do not introduce additional CSI-RS configuration {D=1 with PRBs ≥ 96} for the requirements in Rel-16.

# Conclusion

In this contribution, the following issue is discussed

- FFS on whether to define additional configuration {D=1 with PRBs ≥ 96} for the requirements

It is proposed

Proposal: Do not introduce additional CSI-RS configuration {D=1 with PRBs ≥ 96} for the requirements in Rel-16.