3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010085

# Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0974 | Rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of interruption requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Need to notice sync./async. assumption between NR Uu and SL in interruption requirements. And, interruption requirement on NR WAN due to switching between LTE SL and NR SL is needed to be defined regarding NR V2X con-current operation band, n71+47+n47 specified in TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add note in Table 12.7.1-1 Note1 : It is assumed to be synchronized between NR Uu and V2X SL when gNB or SyncRef UE directly/indirectly synchronized to gNB is applied as synchronization reference source. Add 12.7.3 Interruptions to WAN due to UE switching between E-UTRA sidelink and NR sidelink |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambituity on sync./async. between NR Uu and SL is remained. And interruption to WAN due to UE switching between NR SL and LTE SL in case of NR V2X con-current operation band, n71+47+n47 is missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

12.7	Interruption

12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This sub-clause contains the requirements related to the interruptions on the PCell due to V2X sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformation.

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the serving cell(s) during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message sl-V2X-ConfigDedicated (setup and release). This interruption is for both uplink and downlink of the PCell.

Table 12.7.1-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |
| NOTE 1: It is assumed to be synchronized between NR Uu and V2X SL when gNB or SyncRef UE directly/indirectly synchronized to gNB is applied as synchronization reference source. |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

12.7.3	Interruptions to WAN due to UE switching between E-UTRA sidelink and NR sidelink

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to switching between E-UTRA V2X sidelink and NR V2X sidelink on a dedicated carrier.

The UE is allowed an interruption of up to the duration shown in table 12.7.3-1 on the PCell/serving cell during the switching. This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.3-1 : Interruption length due to UE switching between E-UTRA sidelink and NR sidelink

|  | NR Slot length (ms) | Interruption length (number of slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 1 | 2 |
| 3 | 0.125 | 2 | 2 |
| Note1: It is assumed to be synchronized between NR Uu and V2X SL when gNB or SyncRef UE directly/indirectly synchronized to gNB is applied as synchronization reference source. |  |  |  |

< END OF CHANGE #2 >