3GPP TSG-RAN WG4 Meeting #96e                                           R4-2010149

Electronic Meeting, 17th– 28thAugust, 2020

Agenda Item:   7.4.2.2.3

Source:        Samsung

Title:           IAB-MT Receiver FRC for QPSK

Document for:  Discussion

Introduction

In RAN4#96e it is agreed that the FRC for type 1-H, 1-O and 2-O IAB-MT should be “Selected UE FRC can be for IAB-MT based the same criteria as BS”. [1] This contribution discusses this aspect further especially on how to address this in IAB spec based on discussion presented in [2].

Discussion

In NR BS specification except the Receiver dynamic range the FRCs are for QPSK only. While for IAB-MT, receiver dynamic range is agreed as not needed for FR2 and still not decided for FR1. Hence in this contribution will focus on FRC for QPSK only. Furthermore, there is agree to remove the ICS requirement for IAB-MT receiver for both FR1 and FR2. Hence here we only consider the FRCs apply for below cases:

- FR1:  reference sensitivity level, ACS, in-band blocking, out-of-band blocking, [receiver intermodulation] , OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, and [OTA receiver intermodulation]

- FR2: OTA reference sensitivity level, OTA ACS, OTA in-band blocking and OTA out-of-band blocking

And for TS38.104 the FRC for above cases are summarized in below table for FR1 and FR2 respectively.

Table 1: FR1 NR BS FRC

| FRC for target requirement | BS Channel BW[MHz] | Subcarrier spacing | Number of PRBs |
| --- | --- | --- | --- |
| G-FR1-A1-1 | 5, 10, 15 | 15 kHz | 25 |
| G-FR1-A1-2 | 10, 15 | 30 kHz | 11 |
| G-FR1-A1-3 | 10, 15 | 60 kHz | 11 |
| G-FR1-A1-4 | 20, 25, 30, 40, 50 | 15 kHz | 106 |
| G-FR1-A1-5 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 kHz | 51 |
| G-FR1-A1-6 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 kHz | 24 |

Table 2: FR2 NR BS FRC

| FRC for target requriement | BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | PRB allocation |
| --- | --- | --- | --- |
| G-FR2-A1-1 | 50, 100, 200 | 60 | 66 |
| G- FR2-A1-2 | 50 | 120 | 32 |
| G- FR2-A1-3 | 100, 200, 400 | 120 | 66 |

Hence the FRC for IAB-MT to be defined should follow tables below.

| FRC for IAB-MT | Applicable FR1 IAB-MT Channel BW[MHz] | Subcarrier spacing | Number of PRBs |
| --- | --- | --- | --- |
| MT-FR1-A1-1 | 5, 10, 15 | 15 kHz | 25 |
| MT-FR1-A1-2 | 10, 15 | 30 kHz | 11 |
| MT-FR1-A1-3 | 10, 15 | 60 kHz | 11 |
| MT-FR1-A1-4 | 20, 25, 30, 40, 50 | 15 kHz | 106 |
| MT-FR1-A1-5 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 kHz | 51 |
| MT-FR1-A1-6 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 kHz | 24 |

| FRC for REFSENS as example | IAB-MT channel bandwidth [MHz] | Subcarrier spacing [kHz] | PRB allocation |
| --- | --- | --- | --- |
| MT-FR2-A1-1 | 50, 100, 200 | 60 | 66 |
| MT- FR2-A1-2 | 50 | 120 | 32 |
| MT- FR2-A1-3 | 100, 200, 400 | 120 | 66 |

Based on above SCS and PRB allocation, the corresponding FRC suits for configurations are abstracted from in TS38.101-1 and TS38.101-2 for FR1 and FR2 as Table A.x.x.x-1 and Table A.x.x.x-2 below. It is suggested to incorporate them in IAB specification as FRC for related requirement.

In addition, the general parameters captured in Table A.3.1-1 of TS38.101-1 for FR1 and Table A.3.1-1 of TS38.101-2 for FR2 should also be taken into account for reference channel of IAB-MT.

A.X.X.X	FRC for receiver requirements for QPSK on IAB-MT

Table A.x.x.x-1 Fixed reference channel for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking and OTA out-of-band blocking

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | [MT-FR1-A1-1] | [MT-FR1-A1-4] | [MT-FR1-A1-2] | [MT-FR1-A1-5] | [MT-FR1-A1-3] | [MT-FR1-A1-6] |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 | 60 | 60 |
| Subcarrier spacing configuration |  | 0 | 0 | 1 | 1 | 2 | 2 |
| Allocated resource blocks |  | 25 | 106 | 11 | 51 | 11 | 24 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 5 | 5 | 13 | 13 | 26 | 26 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 1672 | 6912 | 736 | 3368 | 736 | 1608 |
| Transport block CRC | Bits | 16 | 24 | 16 | 16 | 16 | 16 |
| LDPC base graph |  | 2 | 1 | 2 | 2 | 2 | 2 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | CBs | 1 | 1 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 5400 | 22896 | 2376 | 11016 | 2376 | 5184 |
| Max. Throughput averaged over 1 frame | Mbps | 0.669 | 2.765 | 0.810 | 3.705 | 1.766 | 3.859 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1 [of TS38.101-1] NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |

Table A.x.x.x-2 Fixed Reference Channel for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking and OTA out-of-band blocking

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | MT-FR2-A1-1 | MT-FR2-A1-2 | MT-FR2-A1-3 |
| Subcarrier spacing | kHz | 60 | 120 | 120 |
| Subcarrier spacing configuration |  | 2 | 3 | 3 |
| Allocated resource blocks |  | 66 | 32 | 66 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 23 | 47 | 47 |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,39} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} (NOTE 6) | Bits | 4224 | 2088 | 4224 |
| Transport block CRC | Bits | 24 | 16 | 24 |
| LDPC base graph |  | 1 | 2 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,39} (NOTE 5) | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} (NOTE 6) | CBs | 1 | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,39} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} (NOTE 6) | Bits | 14256 | 6912 | 14256 |
| Max. Throughput averaged over 1 frame | Mbps | 9.715 | 9.814 | 19.853 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1 [of TS38.101-2]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {3,4,5,6,7} for i from {0,…,39} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {0,1,2} for i from {0,…,39} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |

Reference

[1] R4-2008785, WF on IAB-MT reference sensitivity

[2] R4-2006801, IAB-MT reference sensitivity requirement

[3] TS38.101-1, "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4] TS38.101-2, "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".