3GPP TSG-RAN WG4 Meeting #96e                                     R4-2010150

Electronic Meeting, 17th– 28th August, 2020

Agenda Item:  7.4.3.2

Source:        Samsung

Title:          TP for TR38.809: IAB RRM general

Document for:  Approval

Introduction

This TP is to include general description on IAB-MT RRM requirement in TR38.809 based on agreement in [2]-[7]

Reference

[1] R4-2006378, TR38.809 V0.2.0

[2] R4-1912829 Way forward on scope on NR IAB RRM requirements

[3] R4-1915799 Summary of offline discussion on NR IAB RRM

[4] RP-193199 Proposed Way Forward on RRM Requirements for the IAB Project

[5] RAN4#94e Chairman Report

[6] RAN4#94bis-e Chairman Report

[7] RAN4#95e Chairman Report

TP for IAB TR 38.809

<Start of the TP>

- 11 	IAB RRM requirements

11.1	General

In Rel-16, the WI on IAB is considered for physically fixed deployment only. Hence the RRM requirements assumed as the cases mainly for UE mobility will not be happened frequently for IAB-MT. Based on this assumption the applicability of RRM requirements for Rel-16 IAB is agreed as table 11-1.

Table 11-1: Summary on RRM requirement applicability for IAB-MT

| Requirements | Comments | Is requirement needed for IAB-MT? | Considerations | Applicability for IAB |
| --- | --- | --- | --- | --- |
| Cell reselection | - Intra-frequency - Inter-frequency | No | Idle/Inactive state mobility is not that important to Rel-16 IAB due to fixed operation. | NA |
| Handover |  | No | Handover is not that important to Rel-16 IAB due to fixed operation. | NA |
| RRC Connection Mobility Control | - RRC re-establishment - Random access - RRC release with redirection | Yes | Important to make sure IAB can remain RRC connection after some failure with original parent node. | Apply for both Local Area and Wide Area IAB-MT |
| SCell activation/deactivation delays |  | No | SCell activation/deactivation is not expected to happen frequently. The requirement only impacts activation/deactivation delays, while at least IAB’s MT PCell can be maintained | NA |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No | SCell activation/deactivation and addition/release are not expected to happen frequently. The requirement only impacts interruptions. | NA |
| PSCell addition/release delays |  | No | PSCell addition and release procedure is not expected to happen frequently for Rel-16 IAB nodes. | NA |
| Active BWP switching | - Interruptions at active BWP switching - Active BWP switching delay | No | BWP switching for IAB is not expected to happen frequently. | NA |
| TCI switching | TCI switching delay | No | For Rel-16 fixed IAB it is expected that the link will remain stable for longer periods of time and there might not be a need to switch TCI state. Hence the no TCI state switching requirements defined for IAB in Rel-16. | NA |
| RLM | Requirements for RLM, e.g., based on SSB, CSI-RS or both | Yes for no-DRX mode | Important feature to maintain PCell’s MT link with parent node. | Apply for Local Area IAB-MT only |
| Link recovery procedures | Requirements for SSB-based and CSI-RS based beam failure detection and candidate beam detection | Yes for no-DRX mode | Important feature to maintain PCell’s MT beam management with parent node. | Apply for Local Area IAB-MT only |
| Measurements requirements (time duration, number of cells, etc.) |  | No | Measurement requirements are not that important to Rel-16 IAB due to fixed operation. | NA |
| Measurement accuracy requirements |  | No | For fixed IAB, even though there may be update on deployment environment it would happen less frequent. Hence the measurement accuracy is agreed to be not defined for IAB-MT | NA |
| MT timing related requirements | - MT maximum receive timing difference - MT maximum transmit timing difference | Yes if carrier aggregation band combinations are introduced | [TBD] | [TBD] |
|  | - MT transmit timing - Timing Advance | Yes | UE timing related requirement shall be reused for IAB-MT | Apply for both Local Area and Wide Area IAB-MT |
| DU timing related requirements | - +- 3 us requirement - T_delta command | Yes for IAB-DU (+- 3 us requirement / cell phase synch) No for Tdelta command | A UE served by an IAB node on the access link (Uu) shall meet existing RRM requirements for the access link (Uu) defined in 38.133 and 36.133 (related to EN-DC or NE-DC). - No additional requirements related to UE operation on the access link are needed. | Apply for IAB-DU |

11.2 RRC_CONNECTED state mobility for IAB-MTs

Detailed structure of the subclause is TBD.

11.3 Timing

Detailed structure of the subclause is TBD.

11.4 Signalling Characteristics for IAB-MTs

Detailed structure of the subclause is TBD.

<End of the TP>