3GPP TSG-RAN4 Meeting #96-e	R4-2010392

Electronic Meeting, Aug. 17-28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.133 CR on the performance of CSI-RS based intra-frequency measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The performance of the CSI-RS based intra-frequency measurement needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The performance of the CSI-RS based intra-frequency measurements are provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 10.1.2.2 and 10.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

# 10	Measurement Performance requirements

## 10.1	NR measurements

### 10.1.2	Intra-frequency RSRP accuracy requirements for FR1

#### 10.1.2.2	Intra-frequency CSI-RSRP accuracy requirements

##### 10.1.2.2.1	Absolute CSI-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRP in this clause apply to the CSI-RS resource with the same center frequency as that of the CSI-RS resource in serving cell in FR1.

The accuracy requirements in Table 10.1.2.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for associated SSB.

-	The bandwidth of CSI-RS resource is 48PRB when density is 3, or 96PRB when density is 1.

Table 10.1.2.2.1-1: CSI-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
| [4] | [8.5] | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| [7.5] | [10.5] | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.2.2.2	Relative CSI-RSRP Accuracy

The relative accuracy of CSI-RSRP is defined as the CSI-RSRP measured from one cell compared to the CSI-RSRP measured from another cell with the same center frequency, or between any two CSI-RSRP levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.2.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for the associated SSB.

Table 10.1.2.2.2-1: CSI-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
| [1.5] | [2.5] | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| [2.5] | [2.5] | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

<End of Change 1>

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.2	Intra-frequency CSI-RSRP accuracy requirements

##### 10.1.3.2.1	Absolute CSI-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRP in this clause apply to the CSI-RS resource with the same center frequency as that of the CSI-RS resource in serving cell in FR2.

The accuracy requirements in Table 10.1.3.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.2.1-1: CSI-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 120kHz | SCSCSI-RS = 240kHz |  |  |
| [6] | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| [8] | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.3.2.2	Relative CSI-RSRP Accuracy

The relative accuracy of CSI-RSRP is defined as the CSI-RSRP measured from one cell compared to the CSI-RSRP measured from another cell with the same center frequency, or between any two CSI-RSRP levels measured on the same cell in FR2.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for associatedSSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.2.2-1: CSI-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCS CSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCS CSI-RS = 120kHz | SCS CSI-RS = 240kHz |  |
| [6] | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

<End of Change 2>