3GPP TSG-RAN WG4 Meeting #96-e				  											R4-2010408 Online, 17th Aug. 2020 – 28th Aug. 2020

Source: 	Nokia, T-mobile

Title: 		TP for 37.717-21-11 to introduce DC_2A-71A_n71A and DC_66A-71A_n71A

Agenda Item: 			10.4.2 [DC_R17_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-21-11 to introduce DC_2A-71A_n71A and DC_66A-71A_n71A. Note that no non-contiguous configuration of DC_71_n71 is needed as this is not used for UL.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.x	DC_2A-71A_n71A

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-71A_n71A | DC_2A_n71A |

Note that DC_71_n71 is not used as uplink configuration.

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-71_n71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-71_n71 | 2 | 0 |
|  | 71 | 0 |
|  | n71 |  |

### 5.x.4	Reference sensitivity exceptions

REFSENS exceptions needed for DC_2A-71A_n71A due to band 71 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

## 5.x	DC_66A-71A_n71A

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_66A-71A_n71A | DC_66A_n71A |

Note that DC_71_n71 is not used as uplink configuration.

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_66-71_n71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_66-71_n71 | 66 | 0 |
|  | 71 | 0 |
|  | n71 |  |

### 5.x.4	Reference sensitivity exceptions

No REFSENS exceptions needed for DC_66A-71A_n71A.

*********************** End of the TP ***************************************************

# 3	References