3GPP TSG-RAN WG4 Meeting #96-e				  											R4-2010422 Online, 17th Aug. 2020 – 28th Aug. 2020

Source: 	Nokia

Title: 		TP for 37.717-21-11 to introduce DC_7A-28A_n1A

Agenda Item: 			10.4.2 [DC_R17_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-21-11 to introduce DC_7A-28A_n1A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.x	DC_7-28_n1

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-28_n1 | 7 | 0 |
|  | 28 | 0.6 |
|  | n1 | 0.3 |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-28_n1 | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.x.4	Reference sensitivity exceptions

REFSENS exceptions needed due to band 28 uplink harmonic into band n1 is already specified for DC_28A_n1A.

*********************** End of the TP ***************************************************

# 3	References