3GPP TSG-RAN WG4 Meeting #96-e	R4-2010477

Electronic Meeting, 17 – 28 August 2020

Agenda Item:	6.2.3.2

Source:	Ericsson

Title:	NPUSCH demodulation requirements with multi-TB transmission

Document for:	Discussion

# 1	Introduction

RAN4#95-e agreed with the way forward on UE/BS demodulation performance for additional enhancements for NB-IOT [1].

- Define NPUSCH format 1 performance requirements with multi-TB scheduled transmission by using the following simulation assumptions. Parameter; Value. Number of tones; 12. SCS; 15kHz. Antenna configuration; 1T2R. Channel model; ETU 1Hz. Frequency error; 0Hz. Timing error; 0μs. Performance target; SNR@ 70% of maximum throughput. NRU; 5. Repetition number; 64. HARQ process; 2. Transmission mode; Interleaved. TB size; 424. - Interested companies are welcome to provide simulation results in the next meeting

This contribution provides our initial simulation results.

# 2	Discussion

## 2.1	Simulation results

Figure 1 compares the ideal simulation results of NUSCH format 1 multi-TB non-interleaved transmission and non-interleaved transmission. Table 1 summarizes the SNR of the 70% of maximum throughput.

Figure 1	Comparison between non-interleaved and interleaved transmission.

Table 1	Simulation results of NPUSCH demodulation with multi-TB transmission.

|  | Non-interleaved transmission | Interleaved transmission |
| --- | --- | --- |
| Alignment results | -14.0 | -14.4 |

Proposal 1: Introduce NPUSCH format 1 demodulation requirements with interleaved multi-TB transmission for BS supporting multiTB-UL and multiTB-UL-Interleaving.

Proposal 2: The performance requirements for NPUSCH format 1 demodulation requirements with interleaved multi-TB transmission are optional.

# 3	Summary

Proposal 1: Introduce NPUSCH format 1 demodulation requirements with interleaved multi-TB transmission for BS supporting multiTB-UL and multiTB-UL-Interleaving.

Proposal 2: The performance requirements for NPUSCH format 1 demodulation requirements with interleaved multi-TB transmission are optional.

# 4	References

- R4-2008759, “Way forward on LTE UE and BS performance requirements for additional enhancements for NB-IOT”, Huawei, HiSilicon.