3GPP TSG-RAN WG4 Meeting #96-e       														   R4-2010545

Online, 17th – 28th August, 2020

Title: 	Further on adding bandwidths for NR bands

Source: 	Huawei, HiSilicon

Agenda item:	10.17.1

Document for:	Discussion

# Introduction

In TS 38.101-1/38.104, NR bandwidths for all the SUL bands are specified in sub clause 5.3.5. Most of the supported bandwidths for those SUL bands are the same with the existing FDD or TDD bands since they were introduced however several additional bandwidths are introduced for the corresponding FDD or TDD bands later. Thus inconsistency is observed between supported SUL UE or BS bandwidths and current corresponding FDD or TDD bandwidths.

In this contribution, we list the inconsistency mentioned above and propose to have the gap filled by adding corresponding bandwidths in the spec for SUL bands so that operators can have full efficiency and flexibility when deploying SUL bands together with their corresponding FDD or TDD bands. Specifically, RAN4 has a basket WI in Rel-17 for adding existing bandwidths to existing bands, as the work mentioned in this paper can be carried out under.

Besides, we also propose to add 90MHz and 100MHz channel bandwidth for n40.

# Bandwidth for SUL bands

## Supported bandwidths

Inconsistency can be observed in the spec TS38.104 sub clause 5.3.5 that the supported bandwidth of some SUL bands are not the same as each of their corresponding FDD or TDD bands (frequency overlapped bands).

For SUL bands, it is specified that

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n86 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
| n89 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n95 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |

However for the corresponding FDD or TDD bands, it is specified that

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |

In the above table, the bandwidths marked by yellow colour are not currently supported by the corresponding SUL bands. In the below table, we summarize the addition work that is to be done by adding bandwidths to SUL bands.

| Band | SCS kHz | Bandwidth to be added |
| --- | --- | --- |
| n80 | 15 | 40MHz |
|  | 30 | 40MHz |
|  | 60 | 40MHz |
| n83 | 15 | 30MHz, 40MHz (BS only) |
|  | 30 | 30MHz, 40MHz (BS only) |
|  | 60 | None |
| n84 | 15 | 25MHz, 30MHz, 40MHz, 50MHz |
|  | 30 | 25MHz, 30MHz, 40MHz, 50MHz |
|  | 60 | 25MHz, 30MHz, 40MHz, 50MHz |
| n86 | 15 | 25MHz, 30MHz |
|  | 30 | 25MHz, 30MHz |
|  | 60 | 25MHz, 30MHz |

## Filling up the gap of inconsistency

Due to the inconsistency, operators cannot fully utilize the deployment of SUL bands. Since from UE implementation perspectives, no additional requirements are seen for a UE to support the bandwidths on SUL bands in case it has already the capability of the same bandwidths on corresponding FDD or TDD bands. Thus to fully enable the efficiency and flexibility of network deployment, filling up the gap is necessary.

Meanwhile, there is a basket WI in RAN4 in Rel-17 to add existing UE bandwidths to the existing bands, which we see the necessity of doing the work under.

- Proposal 1: Supported channel bandwidths for SUL bands needs to be complete by adding the missing ones in Rel-17 bandwidth addition basket work item.

# UE channel bandwidths for n40

In Rel-15 100MHz UE channel bandwidth was proposed in [1]. After discussion the final agreement is to introduce 80MHz channel bandwidth only.

But we still observe the availability of 100MHz spectrum in some region like KSA (Kingdom of Saudi Arabia). And there would be some possibility to allocate the whole 100MHz portion to one operator in some regions like Austria, Slovenia, or South America. Besides, in LTE CA_40F with 100MHz aggregated channel bandwidth is specified, and the co-existence with WiFi was also discussed in [1].

According to the agreement in [2], the limitation for maximum UE channel bandwidth is the relative bandwidth, which is the key to ensure the MPR performance (Memory effects, bandwidth limitation, and output matching Q). The agreement for maximum channel bandwidth criteria in [2] is as follows:

In order to introduce new channel bandwidths in the future we propose to introduce a criteria to decide if the general MPR applies or additional MPR should be evaluated:

- If max UL CH BW rel ≤3% for FDD bands and ≤4% for TDD bands general MPR requirement applies

- If max UL CH BW rel >3% for FDD bands and >4% for TDD bands additional band and channel bandwidth specific MPR should be evaluated

According to equation of maximum relative channel bandwidth as following,

max UL CH BW rel = (2*max CH BW) / (FUL low + FUL high)

we can calculate the maximum channel bandwidth for NR band n40, which is 94MHz and close to 100MHz.

According to the agreement above, we see the possibility to define 100MHz UE channel bandwidth, if the proper RF requirements like A-MPR are defined. Thus we would like to propose having a study targeting at defining 100MHz UE channel bandwidth as an optional channel bandwidth with the channel bandwidth specific A-MPR requirements on NR band n40. It would be OK when 100MHz channel bandwidth is utilized the transmission power is reduced properly, while when the higher transmission power is needed the smaller BWP or per-UE channel bandwidth can be configured.

And it seems feasible to specify 90MHz channel bandwidth even without bandwidth specific additional MPR.

- Proposal 2: RAN4 seeks to specify 90MHz and 100MHz UE channel bandwidth for n40 as optional channel bandwidths.

One of the fundamental reason to have those large bandwidths as optional UE bandwidths lies in the actual capability of the popular RF parts. Thus by this opportunity we would also like to trigger a deep dive into this topic to explore the possibility of widely utilizing large UE bandwidths on band n40 as industry for 2.3GHz thrives.

# Conclusion

In this contribution, we list the inconsistency mentioned above and propose to have the gap filled by adding corresponding bandwidth in the spec for SUL bands so that operators can have full efficiency and flexibility when deploying SUL bands together with their corresponding FDD or TDD bands. Specifically, RAN4 has a basket WI in Rel-17 for adding existing bandwidths to existing bands, as the work mentioned in this paper can be carried out under.

| Band | SCS kHz | Bandwidth to be added |
| --- | --- | --- |
| n80 | 15 | 40MHz |
|  | 30 | 40MHz |
|  | 60 | 40MHz |
| n83 | 15 | 30MHz, 40MHz (BS only) |
|  | 30 | 30MHz, 40MHz (BS only) |
|  | 60 | None |
| n84 | 15 | 25MHz, 30MHz, 40MHz, 50MHz |
|  | 30 | 25MHz, 30MHz, 40MHz, 50MHz |
|  | 60 | 25MHz, 30MHz, 40MHz, 50MHz |
| n86 | 15 | 25MHz, 30MHz |
|  | 30 | 25MHz, 30MHz |
|  | 60 | 25MHz, 30MHz |

- Proposal 1: Supported channel bandwidths for SUL bands needs to be complete by adding the missing ones in Rel-17 bandwidth addition basket work item.

And we also provide the technique analysis on the possibility to introduce the wider channel bandwidth for n40. We propose that

- Proposal 2: RAN4 seeks to specify 90MHz and 100MHz UE channel bandwidth for n40 as optional channel bandwidths.

# References

[1] 	Huawei, HiSilicon, R4-1803909, “Further consideration on supporting 100MHz CBW for band n40”, 3GPP TSG RAN4 Meeting #94e.

[2]	Skyworks, Huawei, Qorvo, Broadcom, R4-1801008, “WF on maximum channel bandwidth criteria”, 3GPP TSG RAN4 Meeting #94e.