3GPP TSG-RAN WG4 Meeting #96-e	R4-2010782

Online, 17 – 28 August 2020

Title:	Draft reply LS on UE capability xDD differentiation for SUL/SDL bands

Response to:	-

Release:	Release 15/ Release 16

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Aijun CAO

E-mail Address:	cao.aijun@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 thanks RAN2 for the LS on UE capability xDD differentiation for SUL/SDL bands.

In RAN4 specs, one SUL band or SDL band originates from an existing band definition, either from the uplink frequency trunk of an existing FDD band, e.g., n80/n81/n82/n83 are uplink of n3/n8/n20/n28 respectively, or an existing TDD band UE, e.g., n95 is from n34. But once the SUL or SDL band is defined, in RAN4 they are treated as a different “duplex mode” in parallel with the conventional TDD/FDD duplex mode. The SUL or SDL band has to be combined with a normal FDD or TDD band in operation. And in RAN4 none of features / functions / requirements /procedures related to the SUL or SDL band alone need to further categorize an SUL or SDL band into an FDD or TDD duplex mode. RAN4 would like RAN2 to provide more inputs on the motivation or need to further categorize SUL/SDL bands as FDD or TDD duplex mode.

With the current input from RAN2, RAN4 answers the questions below:

Question 1: Could per-UE capabilities for SUL/SDL bands be differentiated on the duplex mode(s) for Rel-15 and Rel-16?

Answer: SUL/SDL is regarded as one of duplex modes in RAN4 in parallel to FDD/TDD

Question 2: Which duplex mode(s) (i.e. FDD or TDD) for the per-UE capabilities which are differentiated by FDD and TDD are applied for SUL/SDL in both Rel-15 and Rel-16?

Answer: RAN4 does not see the need to further categorize an SUL or SDL band into an FDD or TDD band.

2. Actions:

To RAN2.

ACTION: 	RAN4 respectfully asks RAN2 to take the above into account in related signalling designs.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4#97		xx – xx November 2020			eMeeting