3GPP TSG-RAN WG4 Meeting #96-e	R4-2010863

# Online,  17th – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1030 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: Corrections to AoA setup information in some test cases (Rel-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorrect AoA setup information present in some TCs: A.5.7.3.2 -> Conflicting Setup 3 & 1 informaiton, though Setup 1 was defined A.7.5.1.9 -> Setup 3 information twice present A.7.7.3.2-> Conflicting Setup 3 & 1 informaiton, though Setup 1 was defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | AoA setup information corrected: A.5.7.3.2 -> Wrong Setup 3 informaiton removed A.7.5.1.9 -> Redundant Setup 3 information removed A.7.7.3.2-> Wrong Setup 3 informaiton removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent AoA Setup information leading to different / wrong test implementaiton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.7.3.2, A.7.5.1.9, A.7.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.5.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.15.1.1 and 10.1.15.1.2 for inter-frequency measurement.

##### A.5.7.3.2.2	Test Parameters

In this test case the two NR cells (i.e., Cell 2 and Cell 3) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.5.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test setup in Table A.5.7.3.2.2-2 and Table A.5.7.3.2.2-3. In all test cases, Cell 2 is the PSCell and Cell 3 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. The TCI status for Cell 1 is defined in Table A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.5.7.3.2.2-2: SS-SINR Inter frequency SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A. 5.7.3.2.2-2: SS-SINR Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  | Freq1 | freq2 | freq1 | Freq2 | freq1 | Freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
|  | dB | -0.5 | -0.5 | 11.0 | 11.0 | -3.0 | -3.0 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.5.7.3.2.2-3: SS-SINR Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Angle of arrival configuration | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 10 |  | Rough |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 |  | -105 |  | -105 |  |
| Note1 | dBm/SCS Note3 | -96 |  | -96 |  | -96 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -96.5 | -96.5 | -85 | -85 | -99 | -99 |
| SS-SINRNote2 | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
|  | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
| IoNote2 | dBm/95.04 MHz Note4 | -69.3 |  | -55.4 |  | -65.24 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in Clause 3.5.2. Note 7: Void Note 8: Void Note 9: Void Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.5.7.3.2.3	Test Requirements

The SS-SINR absolute measurement accuracy in test 1 shall be within the range Nominal SS-SINR+3dB to Nominal SS-SINR -4dB and the SS-SINR measurement accuracy in test 2 shall be within the range Nominal SS-SINR+3.5dB to Nominal SS-SINR -4.5dB  according to the requirements in clause 10.1.15.1.1 with an additional -1dB margin reflecting the possible impact of UE self noise in the test. Nominal SS-SINR is the value shown in table A.5.7.2.2.2-3

The SS-SINR relative measurement accuracy shall fulfil the requirements in clause 10.1.15.1.2.

# < Unchanged sections omitted >

#### A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2

##### A.7.5.1.9.1	Test Purpose and Environment

The purpose is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in clause 8.1.7. This test verifies that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly. The test case is only applicable to UE which supports pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

The test parameters are given in table A.7.5.1.9.1-1, table A.7.5.1.9.1-2 and table A.7.5.1.9.1-3 below. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

Table A.7.5.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.1.9.1-2: General test parameters for NR RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1 | 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC pattern 1 |  |
| DRX cycle length | s | 1 | OFF |  |
| T1 | s | 1 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

Table A.7.5.1.9.1-3: Cell specific test parameters for NR RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
| AoA setup |  | 1 | Setup 3 defined in A.3.15.3 |  |
|  |  |  | AoA1 | AoA2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD | Not sent |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.2 TDD | Not sent |
| TRS configuration |  | 1 | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | 1 | TCI.State.2 | N/A |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 | Not sent |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |
| RLM-RS |  | 1 | SSB with index 0 | SSB with index 1 |
|  | dB | 1 | 3 | N/A |
| Note2 | dBm/SCS | 1 | -84.9 | Not sent |
|  | dB | 1 | 3 | N/A |
| SS-RSRP Note3 | dBm/SCS | 1 | -81.9 | -81.9 |
| Io | dBm/95.04 MHz | 1 | -51.15 | -52.91 |
| Propagation Condition |  | 1 | AWGN | - |

##### A.7.5.1.9.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.1.7.3.

# < Unchanged sections omitted >

#### A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.7.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.15.1.1 and 10.1.15.1.2 for inter-frequency measurement.

##### A.7.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.7.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.7.7.3.2.2-2 and Table A.7.7.3.2.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell. The TCI status for Cell 1 is defined in Table  A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.3.2.2-2: SS-SINR Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
|  | dB | -0.5 | -0.5 | 11.0 | 11.0 | -3.0 | -3.0 |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Antenna configuration |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.7.7.3.2.2-3: SS-SINR Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 10 |  | Rough |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 |  | -105 |  | -105 |  |
| Note1 | dBm/SCS Note3 | -96 |  | -96 |  | -96 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -96.5 | -96.5 | -85 | -85 | -99 | -99 |
| SS-SINRNote2 | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
|  | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
| IoNote2 | dBm/95.04 MHz Note4 | -69.3 |  | -55.4 |  | -65.24 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in Clause 3.5.2. Note 7: Void Note 8: Void Note 9: Void Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.7.3.2.3	Test Requirements

The SS-SINR absolute measurement accuracy in test 1 shall be within the range Nominal SS-SINR +3dB to Nominal SS-SINR -4dB and the SS-SINR measurement accuracy in test 2 shall be within the range Nominal SS-SINR +3.5dB to Nominal SS-SINR -4.5dB  according to the requirements in clause 10.1.15.1.1 with an additional -1dB margin reflecting the possible impact of UE self noise in the test.

The SS-SINR relative measurement accuracy shall fulfil the requirements in clause 10.1.15.1.2.

< End of changes >