3GPP TSG-RAN WG4 Meeting #96-e                                                               R4-2010876

Electronic Meeting, 17th August – 28th August, 2020

Source:	Huawei, HiSilicon

Title:	TP for TR 36.717-04-01 CA_2A-7A-28A-66A / CA_2A-7C-28A-66A

Agenda item:	14.3.2

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_2A-7A-28A-66A / CA_2A-7C-28A-66A as defined in the revised WID [1].

Reference

[1]	RP-200637, “New WID: LTE Advanced inter-band CA Rel-17 for x bands DL (x=4, 5) with 1 band UL”, Nokia, Nokia Shanghai Bell

Text Proposal

---Start of changes---

5.x	CA_2-7-28-66

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-7A-28A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-7C-28A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	∆TIB and ∆RIB values

Table 5.x.2-1: IB,c

| CA_2-7-28-66 | 2 | 0.5 |
| --- | --- | --- |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |

Table 5.x.2-2: R IB,c

| CA_2-7-28-66 | 2 | 0.3 |
| --- | --- | --- |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |

5.x.3 	REFSENS requirements

Table 5.x.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-7A-28A-66A5,6 CA_2A-7C-28A-66A5,6 | 66 |  |  | -89,5 | -88,9 | -88,5 | -88,2 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

Table 5.x.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-7A-28A-66A CA_2A-7C-28A-66A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

---End of changes---