3GPP TSG-RAN WG4 Meeting #96-e	R4-2010979

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion on URLLC BS performance requirements with ultra-low BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.1.3

Document for:	Discussion

Background

The simulation results of URLLC BS high reliability with ultra-low BLER are presented in this paper based on the parameters agreed in WF [1].

Simulation

Simulation assumption

The agreed UE parameters are concluded in Table 2-1:

Table 2-1: Simulation assumption for UE PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA low 1x4, ULA low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Processes |  | 1 |
| PT-RS |  | Disabled |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS5 |
| SCS and BW |  | 15KHz for 10MHz 30KHz for 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Testing metric |  | Target BLER: 1-10-5 |

Simulation results

The simulation results for mapping Type B are shown in below:

Table 2-2: SNR values for PDSCH performance requirements

| SCS Antenna configuration SNR | 1x2 |
| --- | --- |
| 15 kHz/10 MHz | -8.5 |
| 30 kHz/40 MHz | -8.4 |

Reference

[1] R4-2008805, Way forward on ultra-low BLER requirements, #95, Ericsson.