3GPP TSG-RAN WG4 Meeting # 96-e	R4-2011195

Electronic Meeting, 17 – 28 August 2020

Agenda Item:	11.1.3.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.921: Dense urban system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

- 6425-7025MHz and 7025-7125MHz.

- 10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence studies will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7], where the dense urban scenario is down-prioritized and thus the corresponding assumptions are FFS.

This contribution proposes the dense urban system level simulation assumptions for coexistence study on ACLR and ACS of NR BS and UE for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, and the corresponding TP to TR 38.921 [10], as the urban macro and indoor hotspot coexistence simulation results according to the agreed assumptions are provided in [8, 9]. The assumptions proposed here are mostly based on the agreed parameters for dense urban scenario in RAN4#95-e [11, 12]. Note that the agreed AAS BS antenna characteristics in [12] and a correction to a typo in urban macro layout are also included in the TP.

# 2.	Text proposal

<Start of text proposal>

#### 4.2.1.1	Urban macro

Details on urban macro network layout model are listed in Tables 4.2.1.1-1 and 4.2.1.1-2.

Table 4.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location [6] | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 4.2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) | RAN4 has long been using un-coordinated operation in below 6GHz coexistence simulation |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Figure 4.2.1.1-1: Coordinated operation

| Uncoordinated Operation: second network’s sites are located at the first network’s cell edge |  |
| --- | --- |

Figure 4.2.1.1-2: Uncoordinated operation

#### 4.2.1.2	Dense urban

Details on dense urban network layout model are listed in Tables 4.2.1.2-1 and 4.2.1.2-2.

Table 4.2.1.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 65 m (urban) < 130 m (suburban) | Based on inter-site distance: 0.45 km (urban) 0.9 km (suburban) |
| BS antenna height |  | 6 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 450m is assumed. |  |  |  |

Figure 4.2.1.2-1: Network layout for dense urban

Table 4.2.1.2-2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

<Next change>

#### 4.2.2.1	Path loss

The pathloss models are summarized in Table 4.2.2.1-1 and the distance definitions are indicated in Figures 4.2.2.1-1 and 4.2.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 4.2.2.1-1.

| Figure 4.2.2.1-1: Definition of d2D and d3D for outdoor UTs | Figure 4.2.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2	(4.2.2-1)

Table 4.2.2.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 5 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| UMa | LOS | PLUMa-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=28.0+22log10(d3D)+20log10(fc) PL2=28.0+40log10(d3D)+20log10(fc) -9log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=25m |
|  | NLOS | PLUMa-NLOS=max(PLUMa-LOS,PLUMa-NLOS') for 10m≤d2D≤5km | σSF=6 | 1.5m≤hUT≤22.5m hBS=25m Explanations: see note 3 |
|  |  | Optional PL=32.4+20log10fc+30log10d3D | σSF=7.8 |  |
| UMi - Street Canyon | LOS | PLUMi-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=32.4+21log10(d3D)+20log10(fc) PL2=32.4+40log10(d3D)+20log10(fc) -9.5log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=10m |
|  | NLOS | PLUMi-NLOS=max(PLUMi-LOS,PLUMi-NLOS') for 10m≤d2D≤5km | σSF=7.82 | 1.5m≤hUT≤22.5m hBS=10m Explanations: see note 4 |
|  |  | Optional PL=32.4+20log10fc+31.9log10d3D | σSF=8.2 |  |
| InH - Office | LOS | PLInH-LOS=32.4+17.3log10(d3D)+20log10(fc) | σSF=3 | 1m≤d3D≤150m |
|  | NLOS | PLInH-NLOS=max(PLInH-LOS,PLInH-NLOS') PLInH-NLOS'=38.3log10d3D+17.30+24.9log10fc | σSF=8.03 | 1m≤d3D≤150m |
|  |  | Optional PLInH-NLOS'=32.4+20log10fc+31.9log10d3D | σSF=8.29 | 1m≤d3D≤150m |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by Cd2D,hUT=0,hUT<13mhUT-13101.5gd2D,13m≤hUT≤23m, where gd2D=0,d2D≤18m54d2D1003exp-d2D150,18m<d2D. Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

#### 4.2.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 4.2.2.2-1.

Table 4.2.2.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMa | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-outwhere C'(hUT)=0,hUT≤13mhUT-13101.5,13m<hUT≤23m |
| UMi - Street canyon | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out361-18d2D-out,18m<d2D-out |
| Indoor - Mixed office | PrLOS=1,d2D-in≤1.2mexp-d2D-in-1.24.7,1.2m<d2D-in<6.5mexp-d2D-in-6.532.6⋅0.32,6.5m≤d2D-in |
| Indoor - Open office | PrLOS=1,d2D-in≤5mexp-d2D-in-570.8,5m<d2D-in≤49mexp-d2D-in-49211.7⋅0.54,49m<d2D-in |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

<Next change>

### 4.2.3		Antenna and beam forming pattern modelling

#### 4.2.3.1	General

A general antenna model is a uniform rectangular panel array, comprising MgNg panels, as illustrated in Figure 4.2.3.1-1.

-	Mg is number of panels in a column

-	Ng is number of panels in a row

-	Antenna panels are uniformly spaced in the horizontal direction with a spacing of dg,H and in the vertical direction with a spacing of dg,V.

-	On each antenna panel, antenna elements are placed in the vertical and horizontal direction, where N is the number of columns, M is the number of antenna elements with the same polarization in each column.

-	Antenna numbering on the panel illustrated in Figure 4.2.3.1-1 assumes observation of the antenna array from the front (with x-axis pointing towards broad-side and increasing y-coordinate for increasing column number).

-	The antenna elements are uniformly spaced in the horizontal direction with a spacing of dH and in the vertical direction with a spacing of dV.

-	The antenna panel is either single polarized (P=1) or dual polarized (P=2).

The rectangular panel array antenna can be described by the following tuple Mg,Ng,M,N,P.

Figure 4.2.3.1-1: General antenna model

For a uniformly distributed array (ULA) antenna, as shown in Figure 4.2.3.1-2, the radiation elements are placed uniformly along the vertical z-axis in the Cartesian coordinate system. The x-y plane constructs the horizontal plane. A signal acting at the array elements is in the direction of u. The elevation angle of the signal direction is denoted as θ (defined between 0° and 180°, 90° represents perpendicular angle to the array antenna aperture) and the azimuth angle is denoted as φ (defined between -180° and 180°).

Figure 4.2.3.1-2: Antenna Array Geometry

The linear phase progression-based beamforming is assumed, as described in Table 4.2.3.1-1.

Table 4.2.3.1-1: Composite antenna pattern

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB AAθ,ϕ | For beam i: AA,Beamiθ,ϕ=AEθ,ϕ+10log10m=1NHn=1NVwi,n,m⋅vn,m2 the super position vector is given by: vn,m=expi⋅2πn-1⋅dVλ⋅cosθ+(m-1)⋅dHλ⋅sin(θ)⋅sin(ϕ), n=1,2,…NV;m=1,2,...NH; the weighting is given by: wi,n,m=1NHNVexpi⋅2πn-1⋅dVλ⋅sinθi,etilt-(m-1)⋅dHλ⋅cos(θi,etilt)⋅sin(ϕi,escan) |

In this simulation, there is one beam formed using all the antenna elements. Each beam is directed to one scheduled UE.

Note the above gives the correct antenna array radiation pattern, however the correct gain is only achieved if the element pattern AAθ,ϕ is selected for the exact element spacing. For other element spacings, the element pattern AAθ,ϕ must be separately calculated such that it is correct for the element spacing (dg,H and dg,V). If AAθ,ϕ is not linked to the element spacing, then the calculated absolute gain may diverge from the correct value in a manner that varies as the beam is steered.

The correct composite array radiation pattern directivity(D) is given by:

DAθ,ϕ=10⋅log4πAAθ,ϕ2-ππ0πPθ,ϕ2sinθdθdϕ,

The composite array radiation pattern gain can then be calculated as:

GAθ,ϕ=DAθ,ϕ-L

Where L is the Loss associated with the antenna. This is currently included in the estimate for element gain AEθ,ϕ, and is 2dB.

#### 4.2.3.2	BS Antenna modelling

##### 4.2.3.2.1	Urban macro scenario

Table 4.2.3.2.1-1: BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 5.5 dBi |
| (Mg, Ng, M, N, P) note | (1, 1, 16, 8, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

##### 4.2.3.2.2	Dense urban scenario

Table 4.2.3.2.2-1: BS antenna element pattern for Dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 5.5 dBi |
| (Mg, Ng, M, N, P) note | (1, 1, 8, 8, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

##### 4.2.3.2.3	Indoor scenario

A method similar to current 3GPP modelling used for passive antennas is applied for the element radiation pattern model, which is

Where

is the magnitude of the element pattern

is the elevation angle, defined between 0° and 180° (90° represents perpendicular to the array antenna aperture)

is the azimuth angle, defined between -180° and 180°

is the maximum directional gain of the radiation element (in dB), which is assumed to be 8dBi;

is the horizontal pattern of the radiation element,

where  is the horizontal 3dB beam-width, and  is the front- to-back ratio.

is the vertical radiation pattern of the radiation element offset by 90° to point perpendicular to array,

whereis the vertical 3dB beam-width and is the side-lobe level limit.

and  are plotted in Figure 4.2.3.2.3-1 for 3dB=65o, SLAv=30 dB, 3dB=65o and Am=30 dB.

Figure 4.2.3.2.3-1: Vertical and Horizontal element radiation patterns

#### 4.2.3.3	UE antenna element pattern

Omnidirectional antenna.

<Next change>

### 4.2.8		Other simulation parameters

Table 4.2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Carrier frequency | 7GHz, 10GHz | 7GHz, 10GHz | 7GHz, 10GHz |
| Channel bandwidth | 100MHz | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 98.28MHz | 98.28MHz | 98.28MHz |
| Scheduled channel bandwidth per UE (UL) | 32.76MHz | 32.76MHz | 32.76MHz |
| The number of active UE (DL) (Note 1) | 1 | 1 | 1 |
| The number of active UE (UL) (Note 1) | 3 | 3 | 3 |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm | 24 | 43 | [37] |
| UE max TX power in dBm | 23 or 20 (Note 2) | 23 or 20 (Note 2) | 23 or 20 (Note 2) |
| UE min TX power in dBm | -33 | -33 | -33 |
| BS Noise figure in dB | 14 (@7GHz) 15 (@10GHz) | 6 (@7GHz) 7 (@10GHz) | 11 (@7GHz) 12 (@10GHz) |
| UE Noise figure in dB | 9 | 9 | 9 |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1 Same as the number of BS beam(s) Note 2: 20dBm as optional case where CLx-ile should be reduced by 3dB |  |  |  |

<End of text proposal>

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011196, “Urban Macro and Indoor Hotspot Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[9]	R4-2011197, “Urban Macro and Indoor Hotspot Uplink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[10]	R4-20xxxxx, “Draft TR38.821 v0.1.0 Parameters for IMT studies on 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”, Huawei.

[11]	R4-2008925, “WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, CATT.

[12]	R4-2008926, “WF on BS Antenna parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Huawei, Nokia.