3GPP TSG-RAN WG4 Meeting #96-e	R4-2011203

Electronic Meeting, 17th – 28th Aug., 2020

Agenda Item:		7.17.4

Source: 		Rohde & Schwarz

Title: 		Plane Wave Synthesizer – Pending MU term

Document for:		Discussion

# Introduction

Following the effort from previous meetings where Plane Wave Synthesizer (PWS) was included as accepted method in TR 37.941 [1] and the MU assessment was introduced together with remaining methods, this contribution presents an analysis of the MU term that was still pending for the frequency range up to 4.2GHz.

# MU budget

The following is the MU budget table applicable to PWS, as agreed in TR 37.941 [1], for EIRP measurements where the pending MU term that required further analysis is highlighted:

#### 9.2.6.3	MU value derivation, FR1

Table 9.2.6.3-1 captures the uncertainty budget contributors and table 9.2.6.3-2 captures the derivation of the expanded measurement uncertainty values for EIRP accuracy measurements in PWS.

Table 9.2.6.3-1: PWS measurement accuracy contributors for EIRP accuracy measurements, FR1

| UID / Details in annex | Uncertainty source |
| --- | --- |
| Stage 2: BS measurement |  |
| A7-1a | Misalignment BS & pointing error |
| C1-1 | RF power measurement equipment (e.g. spectrum analyzer, power meter) |
| A7-2a | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for BS antenna |
| A7-3 | RF leakage (calibration antenna connector terminated) |
| A7-4a | QZ ripple with BS |
| A7-5 | Miscellaneous Uncertainty |
| A7-14 | System non-linearity |
| A7-13 | Frequency Flatness |
| Stage 1: Calibration measurement |  |
| C1-3 | Uncertainty of the network analyzer |
| A7-6 | Mismatch (i.e. reference antenna, network analyser and reference cable) |
| A7-7 | Insertion loss variation |
| A7-3 | RF leakage (calibration antenna connector terminated) |
| A7-8 | Influence of the calibration antenna feed cable |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |
| A7-9 | Misalignment of positioning system |
| A7-1b | Misalignment of calibration antenna & pointing error |
| A7-10 | Rotary joints |
| A7-2b | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna |
| A7-4a | QZ ripple with calibration antenna |
| A7-11 | Switching uncertainty |
| A7-12 | Field repeatability |

# System non-linearity error contribution

As described in TR 37.941 [1], clause A.7, this uncertainty term is calculated as RSS of the following items, assuming a rectangular distribution:

- System non-linearity in time. This is assessed by repeated measurements over a period of time (e.g. 60 minutes) for the same reference power transmitted by the reference antenna. The largest difference between the results is recorded as the uncertainty.

- System non-linearity in power. This is assessed by repeated measurements over a range of transmitted powers. The largest delta between the increments on the receiving side versus the transmitting side is recorded as the uncertainty.

A testing campaign was performed on R&S PWC200 to obtain the corresponding MU estimation. Figure 3-1 presents the overall system setup:

Figure 3-1: Test setup for System Non-linearity

Following the definition above, system non-linearity in power was tested over a range of transmitted powers, being the highest one the limit of the maximum rated Power Density (PD) per section of the PWS area (e.g. dBm/cm2). A brief example of the results is shown in the following figures:

Figure 3-2: Non-linearity in power, f = 2500 MHz

Figure 3-3: Non-linearity in power, f = 3650 MHz

Using the same system setup, system non-linearity in time was tested over a period time using a fixed transmitted power close to the limit of the maximum rated Power Density (PD) per section of the PWS area (e.g. dBm/cm2). A brief example of the results is shown in the following figure:

Figure 3-4: Non-linearity in time, f = 3650 MHz

According to the results from this measurement campaign, system non-linearity uncertainty is calculated using the worst case for each frequency range and combined as RSS for the two items:

| UID | Uncertainty Source | Value f < 3 GHz | Value 3 > f > 4.2 GHz | Unit | Distr. | Divisor | Sens. | ui (dB) f < 3 GHz | ui (dB) 3 > f > 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A7-14 | System non-linearity | 0.10 | 0.10 | dB | Rectangular | √3 | 1 | 0.058 | 0.058 |

# MU values

Following the same procedure in [4] and [5], full MU tables are presented in the excel spreadsheets attached to this contribution, were the updated tabs and values are highlighted (in red). Just for clarity, only affected tables for PWS in TR 37.941 [1] are presented in the spreadsheets.

Furthermore, MU tables updates are provided in the corresponding CR [6] covering all applicable test cases.

It has to be noted that some uncertainty values remain still between brackets for the frequency range between 4.2GHz and 6GHz. For instance:

- Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for BS antenna / calibration antenna

- QZ ripple with BS

- System non-linearity

- Field repeatability

The cells with corresponding values for this pending terms highlighted (in cyan) in the attached spreadsheet.

Further measurements to cover this frequency range are ongoing and values will be presented in upcoming meetings.

# Conclusion

In this contribution, the results from the test campaign to finalize the value for System non-linearity uncertainty source to PWS have been presented.

# References

- 3GPP TR 37.941 v15.0.0 (2020-06)

- R4-1911849, “Plane Wave Synthesizer – Pending MU terms”, Rohde & Schwarz, RAN4#92-bis, October 2019

- R4-2004393, “Justification for additional Tx test cases for PWS”, Rohde & Schwarz, RAN4#94bis-e, April 2020

- R4-2004527, “OTA BS testing Tx FR1 MU calculation tables”, Huawei, RAN4#94bis-e, April 2020

- R4-2004534, “OTA BS testing RX FR1 MU calculation tables – correction”, Huawei, RAN4#94bis-e, April 2020

- R4-2011215, “CR to TR 37.941: Completion of MU terms for PWS“, Rohde & Schwarz, RAN4#96-e, August 2020

- R.  Haupt,  “Generating  a  Plane  Wave  in  the  Near  Field  with  a  Planar Array Antenna”, Microwave Journal, September 2003

- C. Rowell, A. Tankielun, “Plane Wave Converter for 5G Massive MIMO Basestation Measurements”, the 12th European Conference on Antennas and Propagation (EuCAP2018), London, UK, 9 - 13 April, 2018

- B. Derat, “Creating Far-field Condition at a Tenth of Far-field Distance: An Innovative Technique for 5G OTA measurement”, Electronic Design Innovation Conference (EDICON), Beijing, China, 20 – 22 March, 2018