3GPP TSG-RAN WG4 Meeting#96-e                	R4-2011337

Electronic Meeting, 17 – 28 August 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results of L1-SINR measurement accuracy

Agenda item:	7.9.2.3

Document for:	Discussion

Introduction

According to the agreed simulation assumptions in [1], this document presents simulation results of L1-SINR measurement accuracy for Rel-16 NR eMIMO.

Discussion

Simulation assumptions

From [1], the simulation assumptions are as follows:

- L1-SINR accuracy evaluation simulation assumption for CMR only:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | N/A |
| Side condition (SNR) on CMR | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1, 3, 5 |
| Number of PRBs | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + NZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | option-1: 0dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: 0dB, other options not excluded |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3, 5 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + ZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3, 5 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + NZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | option-1: 0dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: 0dB, other options not excluded |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3, 5 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + ZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3, 5 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |

Simulation results

Tables 1-3 provide the CDF of L1-SINR measurement accuracy at 5th, 50th and 95th percentile for SCS = 15 kHz, 30 kHz and 120 kHz, respectively.

Table 1: Absolute L1-RSRP measurement accuracy for SCS = 15 kHz

| CMR | IMR | Number of Samples (M) | 5th percentile | 50th percentile | 95th percentile | Max(abs(5th, 9th) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | 1 | -1.42 | 0.03 | 1.13 | 1.42 |
|  |  | 3 | -0.83 | 0.03 | 0.65 | 0.83 |
|  |  | 5 | -0.62 | 0.03 | 0.56 | 0.62 |
| SSB | NZP-IMR | 1 | -1.46 | -0.04 | 1.01 | 1.46 |
|  |  | 3 | -0.88 | -0.1 | 0.82 | 0.88 |
|  |  | 5 | -0.86 | -0.05 | 0.78 | 0.86 |
| SSB | ZP-IMR | 1 | -1.42 | -0.07 | 1.01 | 1.42 |
|  |  | 3 | -0.8 | -0.05 | 0.67 | 0.8 |
|  |  | 5 | -0.69 | -0.05 | 0.63 | 0.69 |
| CSI-RS | NZP-IMR | 1 | -1.42 | 0.03 | 1.11 | 1.42 |
|  |  | 3 | -0.94 | 0.03 | 0.79 | 0.94 |
|  |  | 5 | -0.7 | 0.02 | 0.71 | 0.71 |
| CSI-RS | ZP-IMR | 1 | -1.22 | 0.06 | 0.96 | 1.22 |
|  |  | 3 | -0.77 | 0.02 | 0.7 | 0.77 |
|  |  | 5 | -0.61 | 0.03 | 0.61 | 0.61 |

Table 2: Absolute L1-RSRP measurement accuracy for SCS = 30 kHz

| CMR | IMR | Number of Samples (M) | 5th percentile | 50th percentile | 95th percentile | Max(abs(5th, 9th) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | 1 | -1.36 | 0.04 | 1.17 | 1.36 |
|  |  | 3 | -0.8 | 0.03 | 0.74 | 0.8 |
|  |  | 5 | -0.59 | 0.03 | 0.56 | 0.59 |
| SSB | NZP-IMR | 1 | -1.39 | -0.07 | 1.08 | 1.39 |
|  |  | 3 | -0.92 | -0.05 | 0.72 | 0.92 |
|  |  | 5 | -0.72 | -0.06 | 0.59 | 0.72 |
| SSB | ZP-IMR | 1 | -1.35 | -0.07 | 1.01 | 1.35 |
|  |  | 3 | -0.82 | -0.02 | 0.7 | 0.82 |
|  |  | 5 | -0.74 | 0 | 0.55 | 0.74 |
| CSI-RS | NZP-IMR | 1 | -1.33 | 0.03 | 1.13 | 1.33 |
|  |  | 3 | -0.89 | 0.05 | 0.82 | 0.89 |
|  |  | 5 | -0.72 | 0.04 | 0.66 | 0.72 |
| CSI-RS | ZP-IMR | 1 | -1.29 | 0.02 | 1.06 | 1.29 |
|  |  | 3 | -0.73 | 0.04 | 0.85 | 0.85 |
|  |  | 5 | -0.59 | 0.05 | 0.67 | 0.67 |

Table 3: Absolute L1-RSRP measurement accuracy for SCS = 120 kHz

| CMR | IMR | Number of Samples (M) | 5th percentile | 50th percentile | 95th percentile | Max(abs(5th, 9th) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | 1 | -1.37 | -0.06 | 1.05 | 1.37 |
|  |  | 3 | -0.85 | -0.08 | 0.64 | 0.85 |
|  |  | 5 | -0.63 | -0.08 | 0.42 | 0.63 |
| SSB | NZP-IMR | 1 | -1.32 | 0.06 | 1.11 | 1.32 |
|  |  | 3 | -0.79 | 0.07 | 0.87 | 0.87 |
|  |  | 5 | -0.65 | 0.02 | 0.8 | 0.8 |
| SSB | ZP-IMR | 1 | -1.3 | 0.03 | 1.06 | 1.3 |
|  |  | 3 | -0.86 | 0.06 | 0.84 | 0.86 |
|  |  | 5 | -0.65 | 0.07 | 0.75 | 0.75 |
| CSI-RS | NZP-IMR | 1 | -1.41 | -0.05 | 1.06 | 1.41 |
|  |  | 3 | -0.94 | -0.09 | 0.73 | 0.94 |
|  |  | 5 | -0.73 | -0.11 | 0.66 | 0.73 |
| CSI-RS | ZP-IMR | 1 | -1.33 | -0.11 | 1.04 | 1.33 |
|  |  | 3 | -0.9 | -0.06 | 0.71 | 0.9 |
|  |  | 5 | -0.7 | -0.07 | 0.58 | 0.7 |

From the table above, it can be observed that there is no significant difference (< 0.3 dB) between the absolute L1-SINR accuracy of M = 3 and M = 5 samples as compared with the difference between M = 1 and M = 3. The difference for M = 1 and M = 3 is approximately 1 dB. Therefore, it is acceptable to set M = 3 if single shot measurement is not applicable for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

Conclusion

The document has presented the simulation results of L1-SINR measurement accuracy for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

References

[1]    R4-19158501, Way forward on NR eMIMO RRM, Samsung