3GPP TSG-RAN WG4 Meeting # 96-e                                                              R4-2009801

Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0224 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for R16 38.101-2: Correction of in-band emission tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2020-8-19 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - PRB should be in the NOTE 1 of in-band emission tables. - The PRB should also be averaged for CA scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct PRB to in the NOTE 1 in the corresponding tables. - Correct the general requirements for in-band emissions in clause 6.4A.2, change PRB to . - Align the CA note for PRB with non-CA tables. - Correct the description of in-band emission measurement interval in 6.4A.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not correct and the test can’t be done correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.3.2, 6.4.2.3.3, 6.4.2.3.4, 6.4.2.3.5, 6.4A.2.3.2, 6.4A.2.3.3, 6.4A.2.3.4, 6.4A.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-2 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

< Start of the changes >

Table 6.4.2.3.2-1: Requirements for in-band emissions for power class 1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

Table 6.4.2.3.3-1: Requirements for in-band emissions for power class 2

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 16 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 16 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 6 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 6 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

Table 6.4.2.3.4-1: Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

Table 6.4.2.3.5-1: Requirements for in-band emissions for power class 4

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 21 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 21 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 11 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 11 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

6.4A.2.3.2	Inband emissions for power class 1

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.2-1 for power class 1 UEs.

Table 6.4A.2.3.2-1: Requirements for in-band emissions for power class 1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

6.4A.2.3.3	Inband emissions for power class 2

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.3-1 for power class 2.

Table 6.4A.2.3.3-1: Requirements for in-band emissions for power class 2

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 16 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 16 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 6 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 6 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

6.4A.2.3.4	Inband emissions for power class 3

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.4-1 for power class 3 UEs.

Table 6.4A.2.3.4-1: Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

< Nex change >

6.4A.2.3.5	Inband emissions for power class 4

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.5-1 for power class 4 UEs.

Table 6.4A.2.3.5-1: Requirements for in-band emissions for power class 4

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25-10∙log10NRBLCRB, 20∙log10EVM- 5∙¦∆RB-1LCRB, -55.1dBm-PRB max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 21 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 21 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 11 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 11 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

< End of the changes >