3GPP TSG-WG4 Meeting #96-e	R4-2010139

# Electronic Meeting, 17th – 28th August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.886 | CR | 0003 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR38.886: Correction on TR38.886 for V2X UE Tx and Rx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to update A-MPR requirements for each PSSCH /PSCCH /simultaneous PSFCH and S-SSB transmission and other remaining RF requirements for 5G V2X UE RF requirements in TR38.886. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is to treat NR V2X UE Tx/Rx RF requirmeents for 5G V2X Service. The contents are lists as follow - Update A-MPR for simultaneous PSFCH by NS_33 - Update A-MPR for S-SSB transmission by NS_33 - Update A-MPR for PSSCH/PSCCH transmission by NS_52 - Update A-MPR for simultaneous PSFCH by NS_52 - Update A-MPR for simultaneous PSFCH by NS_xx (Future release) - Update A-MPR for S-SSB transmission by NS_xx (future release) - Add measurement point description and Figure at Tx/Rx general part - Update REFSENS requirements for single carrier - Update REFSENS requirements for con-current operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X UE RF requirements are not complied regional regulatory requirements. Also REFSENS requirements is not aligned in RAN1 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, 8.1, 8.1.3, 9.1, 9.1.1, 9.1.2, 9.1.3, 10.1.2 and 10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3 Switching time analysis

The switching time between NR SL and LTE SL for short-term TDM operation was analyzed as part of this study.  The general ON / OFF time mask requirements in subclause 6.3.3.2 in TS 38.101-1 is applicable for NR sidelink also.  In addition to the above general ON / OFF time masks, the additional switching period of about 150us is applicable between the NR SL and LTE SL.  Considering that the NR V2X CBW is 10/20/30/40 MHz and that of LTE V2X is 10/20MHz, the spectrum span for contiguous and non-contiguous transmissions would be similar, the switching time can be similar.

7	Operating bands and channel arrangement

7.1	Operating bands

7.1.1	Operating bands in FR1

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | HD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| Note 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |  |  |  |  |  |

<Uncahnged sections are omitted>

7.2	Channel bandwidth

7.2.1	Channel bandwidth in FR1

The NR V2X Communication channel bandwidths and operating bands are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Channel bandwidths defined for NR V2X licensed band should be a subset of UE channel bandwidths for the same licensed band in NR for Rel-16. If operators do have a request for introducing new channel bandwidths for NR V2X licensed bands, it can be treated in the enhanced sidelink WI for collecting operators’ request and defining additional RF requirements related to these new channel bandwidths in Rel-17.

Table 7.2.1-1 V2X Communication channel bandwidth

|  |  | V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n47 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |

<Uncahnged sections are omitted>

8	Transmitter characteristics

8.1	NR V2X UE Tx requirements in FR1

When a NR V2X UE is TDM operating between NR SL and LTE SL at n47, the NR V2X UE need to satisfy the individual V2X Tx requirements of each RAT. For V2X applications the transmitter characteristics are specified at UE antenna connector without any external components that may be used to relocate the antenna away from the UE as shown in Figure 8.1-1.

Figure 8.1-1 shows a V2X integration block diagram with an example set of components external to the UE such as cables and compensator devices.

Figure 8.1-1 An Example of V2X UE intergrataion block diagram

8.1.1	Maximum output power for NR V2X UE

The following V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 8.1-1: V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| n38 |  |  |  |  | 23 | ±2 |  |  |
| n47 |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: NR Band n47 is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

8.1.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 8.1.2-1 and Table 8.1.2-2.

Table 8.1.2-1: MPR simulation assumption for NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 2.7GHz/5.9GHz |
| Bandwidth | 10/20/30/40MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

For NR V2X, simultaneous transmission of PSCCH and PSSCH in the same subframe is supported. The following constraints in table 8.1.2-2 can be assumed based on current RAN1’s agreement.

Table 8.1.2-2: MPR simulation assumption based on RAN1’s agreement

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | • Support {10, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,15,20,25,30,40,45,50,60,70,75,80,90,100,105,110,120,130,135,140,150,160,165,170,175,180,190,195,200,210 |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH, RAN4 assumed as follow

- The baseline MPR simulation assumptions for multiple PSFCH transmission

  - 1 RB per user

  - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed

    - MPR will be derived by non-contiguous PSFCH RB allocation

  - Total power of all users equals 23dBm

  - All users have the same power per RB

  - Simulation parameters as per Table 8.1.2-1 and Table 8.1.2-2

  - Feedback sequences are per Section 5.2 TS 38.211

  - The MPR will be specified as a formula or table based on number of allocated RBs.

  - PSFCH generation shall be based on existing RAN1 agreements

  - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision

Based on the simulation, RAN4 specify MPR requirements as follow

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 8.1-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for NR V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission.

8.1.2.1	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 8.1.2.1-1 for power class 3.

Table 8.1.2.1-1: Maximum Power Reduction (MPR) for power class 3 V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 2.5 |
|  | 16QAM | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.0 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in TS38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For single V2X UE’s PSFCH transmission for PC3 NR V2X UE, the required MPR is defined as follow

MPR_PSFCH =  3.5 dB

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC3 NR V2X UE, the required MPR are specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

MA_PSFCH  =	7.5		; 0.00 <  NGap/NRB  ≤ 0.55

= 12.0	; 0.55 < NGap/NRB ≤ 1.0

Where

NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart)

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for V2X physical channels on S-SSB transmission are specified as follow

Table 8.1.2.1-2: Maximum Power Reduction (MPR) for S-SSB transmission for power class 3 V2X

| Channel | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations1 | Inner RB allocations1 |
| S-SSB | ≤ 6.0 | ≤ 2.5 |
| NOTE 1: Inner and Outer RB allocations are defined in section 8.1.2.1 |  |  |

8.1.3	UE maximum output power with additional requirements

To comply the EN 302 571 standard  emission mask, RAN4 need to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC had regulatory requirements for 40MHz in ITS spectrum as shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply the regional regulatory requirements, RAN4 specify the A-MPR requirements according to different network signalling by pre-configured radio parameters or the cell and the indication from upper layers has indicated to NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3.1-1 Table 8.1.3.1-2 Table 8.1.3.1-3 Table 8.1.3.1-4 |  |
| NS_52 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3.2-1 Table 8.1.3.2-2 Table 8.1.3.2-3 |  |
| NS_XX | Table 8.1.3-1 (A-SEM) | n47 | 10 | Table 8.1.3.3-1 Table 8.1.3.3-2 Table 8.1.3.3-3 |  |

8.1.3.1	AMPR for NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase which is specified for PSCCH and PSSCH transmission\S-SSB\PSFCH below is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in 38.101-1.

For the contiguous PSSCH and PSCCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 8.1.3.1-1: A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency(MHz) | Resources Blocks (LCRB) | Start Resource Block | A-MPRbase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 24 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤19 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 and ≤ 38 | ≤6 |  |  |
|  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤9.5 |  |  |
|  |  | ≥ 20 and ≤ 25 | ≤8.0 |  |  |
|  | ≥ 10 and ≤ 30 | ≥ 4 and ≤7 | ≤ 16 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | 25 and 30 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  | ≥ 22 and ≤ 27 | ≤ 8.0 |  |  |
|  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  | ≥ 2 and ≤ 5 | ≤ 16 |  |  |
|  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 50 | ≥ 0 | ≤ 16 |  |  |
| Note 1: A-MPRstep =1.2 dB used for RBstart 0 and 1 and A-MPRstep =0.7 dB used for all other RBstart |  |  |  |  |  |

Table 8.1.3.1-2: A-MPR for PSSCH/PSCCH by NS_33 at other carrier frequency

| Carrier frequency(MHz) | RB allocations | A-MPRbase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in section 8.1.2.1 |  |  |  |  |  |  |

For the simultaneous PSFCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 8.1.3.1-3: A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860 | NRB =1 | 19.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

For the S-SSB transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 8.1.3.1-4: A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  |  | >3.6 |  | ≤ 9 |  |  |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

8.1.3.2	AMPR for NS_52

When NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPR

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

For the contiguous PSSCH and PSCCH transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 8.1.3.2-1: A-MPR for PSSCH/PSCCH by NS_52

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Region 1 | Region 2 | Region 3 |
| 5885 | QPSK | ≤ 13.5 | ≤ 8.0 | ≤ 5.5 |
|  | 16QAM |  | ≤ 8.0 | ≤ 5.5 |
|  | 64QAM |  | ≤ 8.5 | ≤ 5.5 |
|  | 256QAM |  | ≤ 8.5 | ≤ 6.0 |

Where the following parameters are defined to specify valid RB allocation ranges for Region1, Region2 and Region3 according to RB allocations:

Table 8.1.3.2-1a: A-MPR Region definitions for PSSCH/PSCCH by NS_52

| Channel Bandwidth, MHz | Carrier frequency(MHz) | A-MPR parameters for region definitions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart or RBend | LCRB |  |
| 40 | 5885 | RBstart ≤ floor(NRB*0.2) or RBend ≥ NRB - floor(NRB*0.2) | LCRB ≤floor(NRB*0.2) | Region 1 |
|  |  | The RB allocation is in Region 2 allocation for all other allocations which are not a Region1 or Region3 allocation. |  | Region 2 |
|  |  | floor(NRB /3.5) ≤ RBstart ≤ NRB –floor(NRB /3.5) – LCRB | LCRB ≤ceil(NRB/3.5) | Region 3 |

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

For the simultaneous PSFCH transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 8.1.3.2-2: A-MPR for simultaneous PSFCH by NS_52

| Channel Bandwidth [MHz] | Carrier frequency [MHz] | A-MPR (dB) |
| --- | --- | --- |
| 40 MHz | 5885 | ≤ 23.5 |

For the S-SSB transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 8.1.3.2-3: A-MPR for S-SSB transmission by NS_52

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPR (dB) |
| --- | --- | --- |
| 5885 | ≤ 7 | ≤ 16.0 |
|  | > 7 and ≤ 12 | ≤ 10.5 |
|  | > 12 and ≤ 19 | ≤ 4.0 |
|  | > 19 and ≤ 25 | ≤ 10.5 |
|  | > 25 | ≤ 16.0 |

8.1.3.3	AMPR for NS_xx

When NS_xx is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPR

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

For the contiguous PSSCH and PSCCH transmission when NS_xx is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 8.1.3.3-1: A-MPR for PSSCH/PSCCH by NS_xx

| Carrier frequency(MHz) | RB allocations | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 5860, 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 |
|  | Outer | ≤ 4.5 |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in section 8.1.2.1 |  |  |  |  |  |

For the simultaneous PSFCH transmission when NS_xx is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 8.1.3.3-2: A-MPR for simultaneous PSFCH by NS_xx

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860, 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

For the S-SSB transmission when NS_xx is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 8.1.3.3-3: A-MPR for S-SSB transmission by NS_xx

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860, 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

<Uncahnged sections are omitted>

9	Receiver characteristics

9.1	NR V2X UE Rx requirements in FR1

When a NR V2X UE is TDM operating between NR SL and LTE SL at n47, the NR V2X UE need to satisfy the individual V2X Rx requirements of each RAT. For V2X applications the receiver characteristics are specified at UE antenna connector without any external components that may be used to relocate the antenna away from the UE as shown in Figure 8.1-1.

9.1.1	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: 2.5 dB is assumed. When the number of RB size equal or less than 24RBs, 0.5dB additional relaxation is allowed.

-	Target SNR: -0.5 dB

-	Diversity gain: 3dB

The REFSENS requirements for NR V2X Bands (PC5) are in Table 9.1.1-1.

Table 9.1.1-1: Reference sensitivity for V2X Band (PC5)

| NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n38 | 15 | -96.5 | -93.2 | -91.4 | -90.1 | TDD |
|  | 30 | -96.1 | -93.4 | -91.7 | -90.2 |  |
|  | 60 | -96.9 | -93.1 | -91.9 | -90.4 |  |
| n47 | 15 | -92.5 | -89.2 | -87.4 | -86.1 | HD |
|  | 30 | -92.1 | -89.4 | -87.7 | -86.2 |  |
|  | 60 | -92.9 | -89.1 | -87.9 | -86.4 |  |

Table 9.1.1-2: Sidelink TX configuration for reference sensitivity of V2X Bands (PC5)

| NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS (kHz) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | 15 | 50 | 105 | 160 | 216 | TDD |
|  | 30 | 24 | 50 | 75 | 105 |  |
|  | 60 | 10 | 24 | 36 | 50 |  |
| n47 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 |  |
|  | 60 | 10 | 24 | 36 | 50 |  |

9.1.2	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

For V2X, the requirement are defined for 10MHz, 20MHz, 30MHz and 40MHz. The parameters of reference measurement channel shall be changed according to the physical layer design specific to NR V2X UE in Table 8.1-1.

Table 9.1.2-1: Maximum input level for V2X

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -251 | -251 | [-231] | [-221] |
|  |  | -272 | -272 | [-252] | [-242] |
| NOTE 1: Reference measurement channel is A.7 for 64 QAM in TS38.101-1 NOTE 2: Reference measurement channel is A.7 for 256 QAM in TS38.101-1 |  |  |  |  |  |

9.1.3	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

ACS reflects the digital domain filter attenuation capability, which is model together with ACLR in ACIR. Based on PRR metric. In co-existence study, the existing NR ACS requirement  reuse for both NR UE to V2X UE and DSRC UE to V2X UE urban interference scenarios, PRR loss is less than 5%. Therefore, it is proposed that the same ACS values shall be kept unchanged for V2X UE at n47.

In licensed band, RAN4 shall reuse the same ACS values since there was legacy NR or LTE system.

As 10MHz is typical channel bandwidth used in the V2X band, the interferer bandwidth shall be used with 10MHz for NR V2X operating bands in Table 8.1-1.

Table 9.1.3-1: Adjacent channel selectivity for V2X

|  |  | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| ACS | dB |  |  |  | 33.0 |  | 27.0 | 25.5 | 24.0 |

Table 9.1.3-2: Test parameters for Adjacent channel selectivity for V2X, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + 14 dB |  |  |  |  |  |  |  |
| PInterferer | dBm |  |  |  | REFSENS +45.5dB |  | REFSENS +39.5dB | REFSENS +38.0 dB | REFSENS +36.5dB |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 | 10 |
| FInterferer (offset) | MHz |  |  |  | 10 / -10 |  | 15 / -15 | 20 / -20 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |  |  |  |  |

Table 9.1.3-3: Test parameters for Adjacent channel selectivity for V2X, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm |  |  |  | -56.5 |  | -50.5 | -49.0 | -47.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |  |  |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 | 10 |
| FInterferer (offset) | MHz |  |  |  | 10 / -10 |  | 15 / -15 | 20 / -20 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |  |  |  |  |

<Uncahnged sections are omitted>

    - Rx requirements for inter-band con-current NR V2X operation

      - REFSENS

For the NR V2X UE RF receiver requirements, RAN4 can refer the 2DL inter-band CA to define general UE RF Rx requirements for inter-band con-current NR V2X UE.

The legacy REFSENS requirement will be applied on CC of NR licensed band and NR V2X REFSENS requirements will be applied on CC of NR Band n47 if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 10.1.2.1-1 and Table 10.1.2.1-2 propose the uplink test configurations for inter-band con-current NR V2X REFSENS requirements. For the uplink configuration, RAN4 consider 10MHz Channel bandwidth.

Table 10.1.2.1-1: Uplink configuration for reference sensitivity of V2X UE (PC5)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X UL band (Uu) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n71 | n71 | 15 | 10 | 52 | FDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 11 |  |

Table 10.1.2.1-2: SL Tx configuration for reference sensitivity of V2X UE (Uu)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X band (PC5) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n71 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |

Table 10.1.2.1-3 is proposed UE REFSENS requirements with inter-band con-current NR V2X UE reception.

Table 10.1.2.1-3: Reference sensitivity for V2X QPSK PREFSENS

| Inter-band V2X reception |  |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band | NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n47 | n71 | n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  | FDD |
|  |  |  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |
|  |  | n47 | 15 |  | -92.5 |  | -89.2 |  | -87.4 | -86.1 | HD |
|  |  |  | 30 |  | -92.1 |  | -89.4 |  | -87.7 | -86.2 |  |
|  |  |  | 60 |  | -92.9 |  | -89.1 |  | -87.9 | -86.4 |  |

Table 10.1.2.1-4 is proposed additional Rx insertion loss according to harmonic trap filter to reduce the harmonic problem based on specific self desense analysis according to specific NR V2X inter-band con-current operation.

Table 10.1.2.1-4: ΔRIB,c (two bands)

| V2X inter-band con-current band Combination | NR V2X Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n71-n47 | n71 | 0.0 |

<Uncahnged sections are omitted>

    - Rx requirements for inter-band con-current NR V2X operation

      - REFSENS

For the V2X UE RF receiver requirements, RAN4 can refer the 2DL inter-band CA to define general UE RF Rx requirements for inter-band con-current V2X UE.

The legacy REFSENS requirement will be applied on each CC of NR licensed bands if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 10.2.2.1-1 and Table 10.2.2.1-2 propose the uplink test configurations for inter-band con-current V2X REFSENS requirements. For the uplink configuration, RAN4 consider 10MHz Channel bandwidth.

Table 10.2.2.1-1: Uplink configuration for reference sensitivity of V2X UE (PC5)

| Inter-band V2X con-current band configuration |  | LTE or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | LTE or NR band (Uu) | LTE or NR UL band | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n38 | B20 | B20 | 10 | 50 | FDD |

Table 10.2.2.1-2: SL Tx configuration for reference sensitivity of V2X UE (Uu)

| Inter-band V2X con-current band configuration |  | LTE or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | LTE or NR band (Uu) | NR V2X band (PC5) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n38 | B20 | n38 | 10 | 50 | HD |

Table 10.2.2.1-3 is proposed the REFSENS requirements with inter-band con-current V2X UE reception.

Table 10.2.2.1-3: Reference sensitivity for V2X QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Band | LTE or NR V2X band (Uu) | LTE or NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | B20 | B20 | 15 | -97 | -94 | -91.2 | -90 |  |  | FDD |
|  |  | n38 | 15 |  | -96.5 |  | -93.2 | -91.4 | -90.1 | HD |
|  |  |  | 30 |  | -96.1 |  | -93.4 | -91.7 | -90.2 |  |
|  |  |  | 60 |  | -96.9 |  | -93.1 | -91.9 | -90.4 |  |

Table 10.1.2.2-4 is proposed additional Rx insertion loss according to harmonic trap filter to reduce the harmonic problem based on specific self desense analysis according to specific NR V2X inter-band con-current operation.

Table 10.2.2.1-4: ΔRIB,V2X (two bands)

| V2X inter-band con-current band Combination | V2X operating Band | ΔRIB,c [dB] | Note |
| --- | --- | --- | --- |
| V2X_20_n38 | 20 | FFS | 3rd harmonic from B20 impact into n38 |
| Note 1: The ΔRIB,V2X is applied on top of ΔRIB,c of DC_20_n38 when use harmonic trap filter to reduce 3rd harmonic impact from Band 20. |  |  |  |

<End of Changes>