3GPP TSG-RAN4 Meeting #96-e	R4-2010614

# Online, 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0239 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for introduction of EESS protection for n257 into general spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NOTE: Corresponding Cat F CR is R4-2010589 (CR number is 0238) - It was approved in R4-2009141 that EESS protection of 1dBm/200MHz for n257 should be introduced into general requirements. - According to WRC-19 decision, EESS protection of 1dBm/200MHz will be applied after 1 January 2021. The protection requirement should be introduced in RAN4#96-e. This CR is submitted apart from EESS protection associated with NS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce EESS protection of 1dBm/200MHz for n257 into general spurious emission requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is NOT confimed that UE can meet EESS protection which will be applied after 1 January 2021. UE may violate regulatory requirements in some regions whose regulation requires EESS protection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 6.5A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

6.5.3.1	Spurious emission band UE co-existence

This clause specifies the requirements for the specified NR band, for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.1-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band/frequency range |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 3 |
| n258 | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

<Sections are omitted>

6.5A.3	Spurious emissions for CA

This clause specifies the spurious emission requirements for carrier aggregation.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

For intra-band contiguous carrier aggregation, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) from the edge of the aggregated channel bandwidth, where FOOB is defined as the twice the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB, the spurious emission requirements in Table 6.5.3-2 are applicable. If carrier leakage or I/Q image lands inside the spectrum occupied by the configured UL and DL CCs, exception to the spurious emissions requirement applies. For carrier leakage the requirements specified in clause 6.4A.2.2 shall apply. For I/Q image the requirements specified in clause 6.4A.2.3 shall apply.

6.5A.3.1	Spurious emission band UE co-existence for CA

This clause specifies the requirements for the specified carrier aggregation configurations for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

For intra-band contiguous carrier aggregation, the requirements in Table 6.5A.3-1 apply.

Table 6.5A.3-1: Requirements for CA

| UL CA for any CA bandwidth class | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band / frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 2 |
| CA_n258 | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

<End of change>