3GPP TSG-RAN4 Meeting #96-e	R4-2010847

# Online, , 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0216 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Increase of step size for FR2 in-band blocking conformance test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The in-band blocking conformance test states that the test should be carried out in steps of 1MHz for both FR1 and FR2. For FR2, the width of the operating band together with ΔfOOB is large and testing in steps of 1MHz will lead to excessive testing, running into tens of thousands of steps when considering the full band and all test directions. Stepping in the order of each channel bandwidth will give the same amount of test coverage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Step size for FR2 IBB test changed to align to the step sizes for the OOB requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Excessive test time for FR2 IBB |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.5.2.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

For general OTA blocking:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.2-1 for BS type 1-O and table 7.5.2.5.3-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in table 7.5.2.5.2-1 for BS type 1-O and table 7.5.2.5.3-1 for BS type 2-O. The interfering signal shall be swept with a step size of 1 MHz for BS type 1-O or as indicated in Table 7.5.2.4.2-1 for BS type 2-O starting from the minimum offset to the channel edges of the wanted signals.

Table 7.5.2.4.2-1: FR2 Interferer signal step size

| Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- |
| 50 | 15 |
| 100 | 30 |
| 200 | 60 |
| 400 | 60 |

For OTA narrowband blocking:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.2-2 for BS type 1-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in tables 7.5.2.5.2-2 and 7.5.2.5.2-3 for BS type 1-O. Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.5.2.5.2-3.

8)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclauses 4.7.2 and 4.8.

9)	Repeat steps 3 to 8 for all the specified measurement directions.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.