3GPP TSG-RAN WG4 Meeting #96-e	R4-2010996

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion and simulation results on LTE-based 5G terrestrial broadcast

Source: 	Huawei, HiSilicon

Agenda item:	6.4.1

Document for:	Discussion

Background

During RAN4#94bis-e meeting, the simulation assumption [1] for LTE-based 5G terrestrial broadcast demodulation requirements was approved. However, the simulation results collected from companies still have large span for PMCH cases until this meeting. In this contribution, we discuss and provide our updated simulation results on LTE-based 5G terrestrial broadcast demodulation requirements.

Discussion

Categories for terrestrial broadcast

As per the following change referenced from LS [2], the MBMS categories issue can be solved.

Unless otherwise stated, the requirements on the maximum number of transport block bits are applicable for a TTI length of 1 ms. For other TTI lengths, the requirements shall be scaled according to clause 7.1.7 or 11.1 in TS 36.213 [22] in order to get the corresponding requirement.

As per CR [3] endorsed last meeting, the binary channel bits per subframe for each slot is 133650 and 121500 for MBSFN RS type 1 and 2 respectively. Considering TTI is equal to 3ms, the maximum number of bits for UE Categories in the following Table 2.1-1 should be multiplied by 3, i.e. UE category ≥ 2 should be applied for both MBSFN RS type 1 and 2 for PMCH 0.37kHz SCS.

Table 2.1-1: Maximum number of bits of a MCH transport block received within a TTI set by the field ue-Category for an MBMS capable UE capable of reception via MBSFN

| UE Category | Maximum number of bits of a MCH transport block received within a TTI |
| --- | --- |
| Category 1 | 10296 |
| Category 2 | 51024 |
| Category 3 | 75376 |
| Category 4 | 75376 |
| Category 5 | 75376 |
| Category 6 | 75376 |
| Category 7 | 75376 |
| Category 8 | 75376 |
| Category 9 | 75376 |
| Category 10 | 75376 |
| Category 11 | 75376 (64QAM) 97896 (256QAM) |
| Category 12 | 75376 (64QAM) 97896 (256QAM) |

Proposal 1: Use MBMS UE category ≥ 2 for both MBSFN RS type 1 and 2 for PMCH 0.37kHz SCS.

AWGN initial phase

At last meeting, the issue is raised that whether initial phase has significant impact on performance. In our simulation, we observe that the performance difference can be huge for different initial phase. To reduce the impact on the initial phase, it is suitable to change the Doppler to [5Hz] for PMCH 0.37 kHz SCS as following Table 2.2-1.

Table 2.2-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 0.37kHz

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [5Hz] |  |
| Relative Delay [μs] | Relative Mean Power [dB] |
| 0 | -11 |
| 130 | -10 |
| 220 | -4.5 |
| 240 | -3.5 |
| 400 | 0 |
| 520 | -13 |
| 650 | -20 |
| 800 | -25 |

Proposal 2: For PMCH cases with 0.37kHz SCS, change the Doppler to [5Hz] for propagation conditions.

Simulations

As per the simulation assumption [1], we provide our simulation results as following.

  - PMCH rooftop scenario

| Test number | Bandwidth (MHz) | MCS | Antenna configuration | MBSFN RS type | Number of MBSFN subframes per 40ms | 1% PMCH BLER |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 21 | 1x1 | 1 | 13 |  |
| 2 | 10 | 21 | 1x1 | 2 | 13 |  |

PMCH mobility scenario

| Test number | Bandwidth (MHz) | MCS | Antenna configuration | Number of MBSFN subframes per 40ms | 1% PMCH BLER |
| --- | --- | --- | --- | --- | --- |
| 3 | 10 | 12 | 1x2 low | 39 |  |

(to be updated)

Proposals

In this contribution, we discuss and provide our updated simulation results on LTE-based 5G terrestrial broadcast demodulation requirements. Our observations and proposals are:

Proposal 1: Use MBMS UE category ≥ 2 for both MBSFN RS type 1 and 2 for PMCH 0.37kHz SCS.

Proposal 2: For PMCH cases with 0.37kHz SCS, change the Doppler to [5Hz] for propagation conditions.

Reference

R4-2005515, Simulation assumptions for LTE based 5G terrestrial broadcast performance requirements, RAN4#94bis-e, Huawei, HiSilicon

R1-2004912, LS on categories for terrestrial broadcast, RAN1#101-e, Qualcomm

R4-2008284, CR addition on FRC and propagation conditions definition for LTE-based 5G terrestrial broadcast, RAN4#95-e, Huawei, HiSilicon