3GPP TSG-RAN WG4 Meeting #96-e	R4-2010998

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion on CQI reporting requirements for NR DL 256QAM for FR2

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.3

Document for:	Discussion

Background

During RAN4#95-e meeting, way forward [1] for NR DL 256QAM for FR2 was approved. The open issues for CQI reporting requirements are shown below.

- In the next meeting, decide whether to define FR2 CQI reporting requirements for CQI table 2 - If it is agreed to define FR2 CQI reporting, consider the following test parameters: - Propagation condition - Option 1: Cover both AWGN and fading conditions - Other options are not precluded - SNR testing point - Option 1: Cover higher SNR testing point compared to that in Rel-15 FR2 CQI tests - Other options are not precluded - Other parameters - Option 1: Reuse the assumptions in Rel-15 FR2 CQI tests - Other options are not precluded

In this contribution, we share our views on CQI reporting requirements for NR DL 256QAM for FR2.

Discussion

Testable SNR for CQI reporting requirements

The mainly factor of testability is the testable SNR, which is in a certain range due to the limitation of the transmitted signal power of TE. The maximum SNR for single band UE is referenced from the latest TS 38.810 [2], as the Table 2.1-1 shows below.

Table 2.1-1: Maximum testable SNR for single band UE

| Test methodology | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| DNF | 100 MHz | [22.6 dB] |
|  | 200 MHz | [19.6 dB] |
| DFF | 100 MHz | [19.4 dB] |
|  | 200 MHz | [16.4 dB] |
| IFF | 100 MHz | [19.7 dB] |
|  | 200 MHz | [16.7 dB] |

UE can’t assume which test method is used, therefore only the maximum SNR 19.4dB can be ensured which is the maximum SNR using DFF method. For channel bandwidth 50MHz, we calculate that the maximum testable SNR using DFF method is 22.6dB derived from spreadsheet in TS 38.810.

Observation 1: For NR DL 256QAM for FR2, the maximum testable SNR is 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

To ensure the UE performance under 256QAM for FR2, RAN4 should define the performance requirements for NR DL 256QAM for FR2 with the cases that satisfying the demand that required SNR is less than maximum testable SNR.

Proposal 1: RAN4 should define the performance requirements for NR DL 256QAM for FR2 with the cases that satisfying the demand that required SNR is less than 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

Simulations

For AWGN channel

Table 2.2.1-1 CQI reporting for AWGN channel

| SNR [dB] | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| 19 | 11 | 100% | 0 | 0 | 0.12464 |
| 20 | 12 | 100% | 0 | 0 | 1 |
| 21 | 12 | 100% | 0 | 0 | 0.90763 |
| 22 | 13 | 100% | 0 | 0 | 1 |
| 23 | 13 | 100% | 0 | 5.4237e-5 | 1 |
| 24 | 14 | 100% | 0 | 0 | 1 |
| 25 | 14 | 100% | 0 | 1.3288e-3 | 1 |
| 26 | 15 | 100% | 0 | 0 | 1 |
| 27 | 15 | 100% | 0 | 8.1356e-5 | 1 |

Comparing ideal simulation results, the impairment results usually has about 3dB degradation. Considering the maximum testable SNR 22.6dB for 50MHz bandwidth, for AWGN case, Median CQI 11 is expected at SNR 22dB(add 3dB margin), of which the modulation order is 64QAM. Therefore, we propose to not define CQI reporting requirements under AWGN channel.

Proposal 2: Do not define CQI reporting requirements under AWGN channel.

For fading channel

(to be update)

Proposals

In this contribution, we discuss on CQI reporting requirements for NR DL 256QAM for FR2. Our observations and proposals are:

Observation 1: For NR DL 256QAM for FR2, the maximum testable SNR is 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

Proposal 1: RAN4 should define the performance requirements for NR DL 256QAM for FR2 with the cases that satisfying the demand that required SNR is less than 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

Proposal 2: Do not define CQI reporting requirements under AWGN channel.

Reference

R4-2008817, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4#95-e, China Telecom