3GPP TSG-RAN WG4 Meeting #96-e	R4-2011002

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Simulation results on NR UE HST performance requirements for multi-path fading channel

Source: 	Huawei, HiSilicon

Agenda item:	7.15.3.1.4

Document for:	Discussion

Background

During RAN4#95-e meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we provide our simulation results on NR Rel-16 UE HST demodulation requirements for multi-path fading channel for alignment. The simulation assumption is derived from WF [1] as Table 1-1 below.

Table 1-1 Simulation assumption for NR UE HST multi-path fading scenario

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

Simulations

The simulation results is shown in the table 2-1 and Figure 2-1 below.

Table 2-1: Ideal Simulation results for NR UE HST multi-path fading scenario

| Case Number | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | SNR@70% Max TP |
| --- | --- | --- | --- | --- |
| 1 | 2x2 | 10MHz/15kHz | 600 | 6.70 |
| 2 | 2x4 | 10MHz/15kHz | 600 | 2.94 |
| 3 | 2x2 | 40MHz/30kHz | 1200 | 7.60 |
| 4 | 2x4 | 40MHz/30kHz | 1200 | 3.54 |

Figure 2-1: Ideal Simulation results for NR UE HST multi-path fading scenario

Conclusion

In this paper we provide our simulation results on Rel-16 NR UE HST performance requirements for multi-path fading channel for alignment.

Reference

R4-2008820, WF on UE demodulation for NR HST, RAN4#95-e, CMCC