3GPP TSG-RAN WG4 Meeting #96-e	R4-2011397

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Simulation results on the NR HST PUSCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.15.3.2.1

Document for:	Information

Background

During RAN4#95-e meeting, way forward [1] for NR Rel-16 HST PUSCH demodulation was approved. In this contribution, we provide our updated simulation results on the PUSCH demodulation requirements for NR Rel-16 HST.

Simulations

For 350km/h

As per updated simulation assumption in WF [1], we get the following updated simulation results.

Table 2.1-1: Ideal Simulation results for 350km/h for NR PUSCH

| Case Number | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | Propagation condition | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1x1 | 10MHz/15kHz | 1340 | Tunnel | 2 | -3.51 |
| 2 | 1x1 | 10MHz/15kHz | 1340 | Tunnel | 16 | 8.49 |
| 3 | 1x2 | 10MHz/15kHz | 1340 | Tunnel | 2 | -6.60 |
| 4 | 1x2 | 10MHz/15kHz | 1340 | Tunnel | 16 | 5.41 |
| 5 | 1x2 | 10MHz/15kHz | 1340 | Open space | 2 | -6.60 |
| 6 | 1x2 | 10MHz/15kHz | 1340 | Open space | 16 | 5.40 |
| 7 | 1x8 | 10MHz/15kHz | 1340 | Open space | 2 | -12.57 |
| 8 | 1x8 | 10MHz/15kHz | 1340 | Open space | 16 | 0.40 |
| 9 | 1x1 | 40MHz/30kHz | 2334 | Tunnel | 2 | -3.51 |
| 10 | 1x1 | 40MHz/30kHz | 2334 | Tunnel | 16 | 8.50 |
| 11 | 1x2 | 40MHz/30kHz | 2334 | Tunnel | 2 | -6.59 |
| 12 | 1x2 | 40MHz/30kHz | 2334 | Tunnel | 16 | 5.41 |
| 13 | 1x2 | 40MHz/30kHz | 2334 | Open space | 2 | -6.67 |
| 14 | 1x2 | 40MHz/30kHz | 2334 | Open space | 16 | 5.40 |
| 15 | 1x8 | 40MHz/30kHz | 2334 | Open space | 2 | -12.60 |
| 16 | 1x8 | 40MHz/30kHz | 2334 | Open space | 16 | 0.40 |
| 17 | 1x1 | 5MHz/15kHz | 1340 | Tunnel | 2 | -3.51 |
| 18 | 1x1 | 5MHz/15kHz | 1340 | Tunnel | 16 | 8.49 |
| 19 | 1x2 | 5MHz/15kHz | 1340 | Tunnel | 2 | -6.51 |
| 20 | 1x2 | 5MHz/15kHz | 1340 | Tunnel | 16 | 5.49 |
| 21 | 1x2 | 5MHz/15kHz | 1340 | Open space | 2 | -6.51 |
| 22 | 1x2 | 5MHz/15kHz | 1340 | Open space | 16 | 5.49 |
| 23 | 1x8 | 5MHz/15kHz | 1340 | Open space | 2 | -12.44 |
| 24 | 1x8 | 5MHz/15kHz | 1340 | Open space | 16 | 0.49 |
| 25 | 1x1 | 10MHz/30kHz | 2334 | Tunnel | 2 | -3.51 |
| 26 | 1x1 | 10MHz/30kHz | 2334 | Tunnel | 16 | 8.50 |
| 27 | 1x2 | 10MHz/30kHz | 2334 | Tunnel | 2 | -6.51 |
| 28 | 1x2 | 10MHz/30kHz | 2334 | Tunnel | 16 | 5.50 |
| 29 | 1x2 | 10MHz/30kHz | 2334 | Open space | 2 | -6.54 |
| 30 | 1x2 | 10MHz/30kHz | 2334 | Open space | 16 | 5.49 |
| 31 | 1x8 | 10MHz/30kHz | 2334 | Open space | 2 | -12.48 |
| 32 | 1x8 | 10MHz/30kHz | 2334 | Open space | 16 | 0.49 |

For 500km/h

As per updated simulation assumption in WF [1], we get the following updated simulation results.

Table 2.2-1: Ideal Simulation results for 500km/h for NR PUSCH

| Case Number | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | Propagation condition | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1x1 | 10MHz/15kHz | 1740 | Tunnel | 2 | -3.51 |
| 2 | 1x1 | 10MHz/15kHz | 1740 | Tunnel | 16 | 8.50 |
| 3 | 1x2 | 10MHz/15kHz | 1740 | Tunnel | 2 | -6.60 |
| 4 | 1x2 | 10MHz/15kHz | 1740 | Tunnel | 16 | 5.41 |
| 5 | 1x2 | 10MHz/15kHz | 1740 | Open space | 2 | -6.60 |
| 6 | 1x2 | 10MHz/15kHz | 1740 | Open space | 16 | 5.40 |
| 7 | 1x8 | 10MHz/15kHz | 1740 | Open space | 2 | -12.57 |
| 8 | 1x8 | 10MHz/15kHz | 1740 | Open space | 16 | 0.40 |
| 9 | 1x1 | 40MHz/30kHz | 3334 | Tunnel | 2 | -3.50 |
| 10 | 1x1 | 40MHz/30kHz | 3334 | Tunnel | 16 | 8.50 |
| 11 | 1x2 | 40MHz/30kHz | 3334 | Tunnel | 2 | -6.59 |
| 12 | 1x2 | 40MHz/30kHz | 3334 | Tunnel | 16 | 5.41 |
| 13 | 1x2 | 40MHz/30kHz | 3334 | Open space | 2 | -6.67 |
| 14 | 1x2 | 40MHz/30kHz | 3334 | Open space | 16 | 5.40 |
| 15 | 1x8 | 40MHz/30kHz | 3334 | Open space | 2 | -12.60 |
| 16 | 1x8 | 40MHz/30kHz | 3334 | Open space | 16 | 0.40 |
| 17 | 1x1 | 5MHz/15kHz | 1740 | Tunnel | 2 | -3.51 |
| 18 | 1x1 | 5MHz/15kHz | 1740 | Tunnel | 16 | 8.49 |
| 19 | 1x2 | 5MHz/15kHz | 1740 | Tunnel | 2 | -6.51 |
| 20 | 1x2 | 5MHz/15kHz | 1740 | Tunnel | 16 | 5.50 |
| 21 | 1x2 | 5MHz/15kHz | 1740 | Open space | 2 | -6.51 |
| 22 | 1x2 | 5MHz/15kHz | 1740 | Open space | 16 | 5.49 |
| 23 | 1x8 | 5MHz/15kHz | 1740 | Open space | 2 | -12.44 |
| 24 | 1x8 | 5MHz/15kHz | 1740 | Open space | 16 | 0.49 |
| 25 | 1x1 | 10MHz/30kHz | 3334 | Tunnel | 2 | -3.51 |
| 26 | 1x1 | 10MHz/30kHz | 3334 | Tunnel | 16 | 8.50 |
| 27 | 1x2 | 10MHz/30kHz | 3334 | Tunnel | 2 | -6.52 |
| 28 | 1x2 | 10MHz/30kHz | 3334 | Tunnel | 16 | 5.50 |
| 29 | 1x2 | 10MHz/30kHz | 3334 | Open space | 2 | -6.54 |
| 30 | 1x2 | 10MHz/30kHz | 3334 | Open space | 16 | 5.49 |
| 31 | 1x8 | 10MHz/30kHz | 3334 | Open space | 2 | -12.42 |
| 32 | 1x8 | 10MHz/30kHz | 3334 | Open space | 16 | 0.49 |

Reference

R4-2008821, WF on Rel-16 NR HST BS demodulation requirements, RAN4#95-e, Nokia