3GPP TSG-RAN WG4 Meeting #96-e	R4-2011398

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Simulation results on the UL timing adjustment

Source: 	Huawei, HiSilicon

Agenda item:	7.15.3.2.3

Document for:	Information

Background

During RAN4#95-e meeting, way forward [1] for NR Rel-16 HST BS demodulation requirements was approved. In this contribution, we provide our updated simulation results on the UL timing adjustment demodulation requirements.

Simulations

As per updated simulation assumption in WF [1], we provide the following updated simulation results.

Table 3-1: Ideal Simulation results for UL TA

| Case Number | Scenario | CHBW/SCS | PUSCH mapping type | SNR@70% Max TP |
| --- | --- | --- | --- | --- |
| 1 | Y | 10MHz/15kHz | A | 5.90 |
| 2 | Y | 10MHz/15kHz | B | 6.10 |
| 3 | Y | 40MHz/30kHz | A | 6.03 |
| 4 | Y | 40MHz/30kHz | B | 6.25 |
| 5 | Y | 5MHz/15kHz | A | 5.57 |
| 6 | Y | 5MHz/15kHz | B | 5.87 |
| 7 | Y | 10MHz/30kHz | A | 5.67 |
| 8 | Y | 10MHz/30kHz | B | 5.97 |
| 9 | Z | 10MHz/15kHz | A | 6.01 |
| 10 | Z | 10MHz/15kHz | B | 6.22 |
| 11 | Z | 40MHz/30kHz | A | 6.04 |
| 12 | Z | 40MHz/30kHz | B | 6.26 |
| 13 | Z | 5MHz/15kHz | A | 5.57 |
| 14 | Z | 5MHz/15kHz | B | 5.87 |
| 15 | Z | 10MHz/30kHz | A | 5.67 |
| 16 | Z | 10MHz/30kHz | B | 5.97 |

Reference

R4-2008821, WF on Rel-16 NR HSTUL TA BS demodulation requirements, RAN4#95-e, ZTE