3GPP TSG-WG RAN4 Meeting #96-e	R4-2011408

# Electronic meeting, 17th – 28th August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0948 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.141: Correction to applicability of additional BC3 requirement (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | On top of generic Tx IM and blocking requirement, there is additional requirement for BC3 base stations which uses 1.28Mcps UTRA TDD signal. Since this signal is not used anymore in any deployment, it is not clear why such requirement would need to be applicable. This CR is proposing to remove this requirement for recently introduced CS16/17 base stations, at the same time discussion is needed if such requirement make sense for other capability sets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | N/A has been added for the following CS16/17 test requirements: Additional requirement (BC3) for Transmitter intermodulation, Additional BC3 blocking requirement for In-band selectivity and blocking |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary test requirement would exist which introduces additional test effort. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 5.1-1c: Test configurations for capability sets (CS16-19) for Multi-RAT capable BS

| Capability Set | NR + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 16) |  | NR + NB-IoT standalone + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 17) |  | GSM + NR + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 18) | UTRA + NR + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 19) |
| --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 and BC2 | BC3 | BC1 and BC2 | BC3 | BC2 | BC1 and BC2 |
| 6.2 Base Station output power | - | - | - | - |  |  |
| Base Station maximum output power | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG:TC22 | C: TC22 NI: TC22 NG:TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| E-UTRA for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| UTRA FDD secondary CPICH power | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| 6.3 Output power dynamics | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| GSM/EDGE | N/A | N/A | N/A | N/A | TC4b | N/A |
| 6.4 Transmit ON/OFF power | - | - | - | - | - | - |
| Transmitter OFF power | N/A | C: TC21 CNC: NTC21 | N/A | C: TC22 | N/A | N/A |
| Transmitter transient period | N/A | C: TC21 CNC: NTC21 | N/A | C: TC22 | N/A | N/A |
| 6.5 Transmitted signal quality | - | - | - | - | - | - |
| 6.5.1 Modulation quality | - | - | - | - | - | - |
| E-UTRA | C: TC21 NI, NG: (Note 4) CNC: TC21 NCNI, NCNG: (Note 4) C/NC: NTC21, TC21 C/NCNI, C/NCNG: (Note 4) | C: TC21 NI, NG: (Note 4) CNC: TC21 NCNI, NCNG: (Note 4) C/NC: NTC21, TC21 C/NCNI, C/NCNG: (Note 4) | C: TC22 NI, NG: (Note 4) | C: TC22 NI, NG: (Note 4) | C: TC21a NI, NG: (Note 4) CNC: TC21a NCNI, NCNG: (Note 4) C/NC: NTC21a, TC21a C/NCNI, C/NCNG: (Note 4) | C: TC21b NI, NG: (Note 4) CNC: TC21b NCNI, NCNG: (Note 4) C/NC: NTC21b, TC21b C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | Standalone: C: TC22 NI, NG: (Note 4) | Standalone C: TC22 NI, NG: (Note 4) | N/A (Note 4) | N/A (Note 4) |
| NR | C: TC21 CNC: TC21 C/NC: NTC21, TC21 | C: TC21 CNC: TC21 C/NC: NTC21, TC21 | C: TC22 | C: TC22 | C: TC21a CNC: TC21a C/NC: NTC21a, TC21a | C: TC21b CNC: TC21b C/NC: NTC21b, TC21b |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | C: TC21b CNC: TC21b C/NC: NTC21b, TC21b |
| GSM/EDGE | N/A | N/A | N/A | N/A | C: TC21a CNC: TC21a C/NC: NTC21a, TC21a | N/A |
| 6.5.2 Frequency error | - | - | - | - | - | - |
| E-UTRA | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | Same TC as 6.5.1 | Same TC as 6.5.1 | N/A (Note 4) | N/A (Note 4) |
| NR | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | Same TC as 6.5.1 |
| GSM/EDGE | N/A | N/A | N/A | N/A | Same TC as 6.5.1 | N/A |
| 6.5.3 Time alignment error | - | - |  |  | - | - |
| E-UTRA | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) | (TS 36.141) NI, NG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | Standalone: (TS 36.141) NI, NG: (Note 4) | Standalone: (TS 36.141) NI, NG: (Note 4) | N/A (Note 4) | N/A (Note 4) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| 6.6 Unwanted emissions | - | - | - | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - | - | - | - |
| (Category A) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| (Category B) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | N/A |
| Protection of the BS receiver of own or different BS | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional spurious emissions requirements | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Co-location with other Base Stations | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| 6.6.2 Operating band unwanted emissions | - | - | - | - | - | - |
| General requirement for Band Categories 1 and 3 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | C: TC22 NI: TC22 NG: TC22 SC: (Note 3) | C: TC22 NI: TC22 NG: TC22 SC: (Note 3) | N/A | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b SC: (Note 3) |
| General requirement for Band Category 2 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | N/A | C: TC22 NI: TC22 NG: TC22 SC: (Note 3) | N/A | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a SC: (Note 3) | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b SC: (Note 3) |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - | - | - | - |
| Minimum requirement | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 25.141) (TS 36.141) (TS 38.141-1) |
| 6.6.4 Adjacent Channel Leakage power Ratio (ACLR) | - | - | - | - | - | - |
| E- UTRA | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 | C: TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21b CNC: NTC21b C/NC: NTC21b, TC21b |
| NB-IoT | NI: TC21 NG: TC21 NCNI: NTC21 NCNG: NTC21 C/NCNI, C/NCNG: NTC21, TC21 | NI: TC21 NG: TC21 NCNI: NTC21 NCNG: NTC21 C/NCNI, C/NCNG: NTC21, TC21 | TC22 | TC22 | NI: TC21 NG: TC21 NCNI: NTC21 NCNG: NTC21 C/NCNI, C/NCNG: NTC21, TC21 | NI: TC21b NG: TC21b NCNI: NTC21b NCNG: NTC21b C/NCNI, C/NCNG: NTC21b, TC21b |
| NR | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 | C: TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21b CNC: NTC21b C/NC: NTC21b, TC21b |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | C: TC21b CNC: NTC21b C/NC: NTC21b, TC21b |
| Cumulative ACLR | CNC: NTC21 C/NC: NTC21 | CNC: NTC21 C/NC: NTC21 | N/A | N/A | CNC: NTC21 C/NC: NTC21 | CNC: NTC21b C/NC: NTC21b |
| 6.7 Transmitter intermodulation | - | - | - | - |  |  |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | CNC: NTC21 C/NC: NTC21 | N/A | Same TC as used in 6.6 |  | CNC: NTC21a C/NC: NTC21a | CNC: NTC21b C/NC: NTC21b |
| Additional requirement (BC3) |  | N/A |  | N/A |  |  |
| 7.2 Reference sensitivity level | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.3 Dynamic range |  |  |  |  |  |  |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.4 In- band selectivity and blocking | - | - | - | - |  |  |
| General blocking requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| General narrowband blocking requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional narrowband blocking requirement for GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| GSM/EDGE requirements for AM suppression | N/A | N/A | N/A | N/A | TC5b | N/A |
| Additional BC3 blocking requirement | N/A | N/A | N/A | N/A | N/A | N/A |
| 7.5 Out-of-band blocking | - | - | - | - |  |  |
| General requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Co-location requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| 7.6 Receiver spurious emissions | - | - | - | - | - | - |
| General requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional requirement for BC2 (Category B) | N/A | N/A | C: TC22 NI: TC22 NG: TC22 | N/A | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | N/A |
| 7.7 Receiver intermodulation | - | - | - | - |  |  |
| General intermodulation requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| General narrowband intermodulation requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional narrowband intermodulation requirement for GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.8 In-channel selectivity | - | - | - | - | - | - |
| E-UTRA requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| NOTE 1: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by “NI” shall not be used for testing. NOTE 2: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by “NG” shall not be used for testing. NOTE 3: For Operating band unwanted emissions, NR shall also be tested with SC with widest supported channel bandwidth and highest supported sub-carrier spacing. NOTE 4: There is no specific test with NB-IoT for those requirements, tests could be performed using E-UTRA signal only, without NB-IoT. |  |  |  |  |  |  |