# 3GPP TSG-RAN WG4 Meeting #96-e                     	R4-2011505

# Electronic Meeting, 17 – 28 August, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NPUSCH format 1 performance for Multi-TB scheduling

Agenda item:	6.2.3.2

Document for:	Discussion

Introduction

At RAN4 #95-e, UE and BS demodulation requirements for the WI on Additional enhancements for NB-IoT [1] were further discussed. In the WF [2], the following agreements on BS demodulation requirements for the Multi-TB scheduling enhancement are reflected.

NPUSCH format 1 performance requirements - Define NPUSCH format 1 performance requirements with multi-TB scheduled transmission by using the following simulation assumptions. Parameters; Value. Number of tones; 12. SCS; 15kHz. Antenna configuration; 1T2R. Channel model; ETU 1Hz. Frequency error; 0Hz. Timing error; 0μs. Performance target; SNR@ 70% of maximum throughput. NRU; 5. Repetition number; 64. HARQ process; 2. Transmission mode; Interleaved. TB size; 424. - Interested companies are welcome to provide simulation results in the next meeting

This contribution provides further detail on the simulation assumptions and simulation results for the above depicted simulation scenario. It is the revision of R4-2011504 with marked updates.

Discussion

In this section further simulation assumptions in addition to the table in section 1 are given and simulation results are reported.

Further simulation assumptions

Following additional simulation assumptions in Table 1 were used.

| Parameters | Value | Description |
| --- | --- | --- |
| Duplex scheme | FDD | Frame structure 1 is used. |
| Transmission gap duration | 0 ms | See note below. |
| DCI scheduling | Single DCI | For interleaved / non-interleaved mode: both transport blocks are scheduled with single DCI with Rv 0 for initial transmission and each of HARQ retransmissions. |
| RX impairments | no | Ideal simulation results for alignment. |
| Simulated time | 1400 s | Corresponds to 1076 simulated TBs. |

Table 1: Further simulation assumptions for NPUSCH format 1 for Multi-TB transmission.

Note: TS 36.211, subclause 10.1.3.6 specifies a transmission gap duration of 40 ms to be inserted after a transmission of maximum 256 ms for NPUSCH. In TS 36.104 however, no such gap is defined as part of the FRC channels defined in A.16.1. Thus, the transmission gap duration for the NPUSCH TP evaluation is set to 0 ms. Views from other companies on the transmission gap duration setting are invited.

Figure 1 below provides an illustration of the initial transmission for both non-interleaved and interleaved NPUSCH Multi-TB transmission.

Figure 1: Initial transmission for both non-interleaved and interleaved NPUSCH Multi-TB transmission.

With non-interleaved transmission, both TBs have a transmit duration of 320 ms, whilst with interleaved transmission, it is extended to 620 ms for both.

Simulation results

Simulation results without impairments are reported in Table 2 below.

| SNR@70% of maximum throughput (dB) |  | Gain (dB) |
| --- | --- | --- |
| Interleaved | Non-interleaved |  |
| -16.47 | -15.92 | 0.55 |

Table 2: Simulation results for NPUSCH format 1 for Multi-TB transmission.

The following observation is made:

For interleaved multi-TB transmission, the gain versus non-interleaved transmission is 0.55 dB.

Conclusion

This contribution has evaluated NPUSCH format 1 performance for the Multi-TB scheduling enhancement using simulation assumptions according to the agreed WF [2].

The following observation is made:

- For interleaved multi-TB transmission, the gain versus non-interleaved transmission is 0.55 dB.

References

- RP-200293, "WID revision: Additional enhancements for NB-IoT", source: Futurewei

- R4-2008759, "Way forward on LTE UE and BS performance requirements for additional enhancements for NB-IOT", source: Huawei, HiSilicon