3GPP TSG-RAN WG4 Meeting #96-e	R4-2011547

Electronic Meeting, 17 - 28 Aug, 2020

Agenda item:			7.11.1.5

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [96e][114] NR_RF_FR1_Part_2

Document for:	Information

Introduction

This email thread discusses the remaining issues for Tx switching between two uplink carriers in agenda 7.11.1.5.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to review the recommended WF in each sub-topic, and provide comments (if any) in section 1.3 and 2.3.

- 2nd round:

Topic #1: Applicability of DL interruption

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009578 | China Telecom | Proposal 1: For EN-DC and UL CA, DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations. Certain general principle to identify the difficult band combinations can be agreed in RAN4. Proposal 2: Regarding where to capture the mandatory support of no DL interruption, - If certain general principle is agreed, it can be captured in RRM spec - If it is agreed per BC basis, it can be captured in RF spec |
| R4-2010018 | Xiaomi | Proposal 1: the same DL interruption approach shall be used for both EN-DC case and UL CA case. Proposal 2: the option that introducing UE capability indication for each band combination is preferable. |
| R4-2010105 | CMCC | Proposal 1: DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementation. Encourage chipset and UE vendors to feedback if DL interruption can be avoided for the following band pairs. - Band (n)3 + Band n41 (CMCC) - Band (n)3 + Band n79 (CMCC, docomo) - Band (n)8 + Band n41 or n79 (CMCC) - Band (n)40+ Band n41 or n79 (CMCC) - Band (n)41+ Band n79 (CMCC) - Band (n)39+ Band n41 or n79 (CMCC) Proposal 2: It is proposed to specify in RRM spec for which band combinations DL interruptions are allowed. |
| R4-2010835 | Nokia, Nokia Shanghai Bell | Observation 1: Some of the band combinations listed in the WF of R4-2008476 already MSD on both DL or either of DL and/or operation restriction like SUO. Observation 2: Allowing DL interruption for EN-DC highly reduces the value of Tx switching feature to enable UL MIMO for NR UL and even if the feature was supported with such an explicit degradation, it may be difficult to use this feature due to severe legacy LTE performance implications. Observation 3: Specification allows UEs not to support Tx switching per band combination basis if it is difficult to support it for some band combinations. Proposal 1: DL interruptions due to UL Tx switching are not allowed for any EN-DC band combinations. Observation 4: Frequency gap between bands can be one of the criteria to select band combinations which requires DL interruptions. Observation 5: Reported switching period can be one of the criteria to select band combinations which requires DL interruptions. Proposal 2: - No DL interruptions allowed for any band combinations is the baseline for UL CA band combinations - Capture band combinations which need DL interruptions in the spec if specific criteria cannot be met - Discuss and identify potential criteria. Some candidates are provided as follows. - Criteria 1: Frequency gap between bands and TDD bands frequency range - Note that in principle if the gap is narrower, higher possibility to require DL interruption. - Criteria 2: Reported Tx switching period via uplinkTxSwitchingPeriod - Note that if the period is 210 us, higher possibility to require DL interruption - Criteria 3: combination of Criteria 1 and 2 - Other criteria are not precluded |

Open issues summary

Issue 1-1: Applicability of DL interruption for different band combinations

Agreement in RAN4 #95e (R4-2008476 in WF):

- For EN-DC: continue discussing the following options at RAN4 #96e

  - Option 1: DL interruptions due to UL Tx switching are not allowed for any EN-DC band combinations.

  - Option 2: DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations.

    - Encourage chipset and UE vendors to feedback if DL interruption can be aovided for the band pairs for in slide #7.

  - Option 3: DL interruptions on any band combinations with UE capability indication should be allowed

- For UL CA: continue discussing the following options at RAN4 #96e

  - Option 1: DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations.

    - Encourage chipset and UE vendors to feedback if DL interruption can be avoided for the band pairs for in slide #7.

  - Option 2: DL interruptions on any band combinations with UE capability indication should be allowed

- Interested band pairs from operators for FDD+TDD CA and FDD+TDD EN-DC (for information)

  - Band (n)1 + Band n78 (CTC, CU, KDDI, CHTTL)

  - Band (n)3 + Band n78 (CTC, CU, KDDI, CHTTL)

  - Band (n)1 + Band n77 (KDDI)

  - Band (n)3 + Band n77 (KDDI)

  - Band (n)18 + Band n77 or n78 (KDDI)

  - Band (n)28 + Band n77 or n78 (KDDI)

  - Band (n)3 + Band n41 (CMCC)

  - Band (n)3 + Band  n79 (CMCC, docomo)

  - Band (n)8 + Band n41 or n79 (CMCC)

  - Band (n)8 + Band n77 or n78 (SoftBank)

  - Band 11 + Band n77 or n78 (SoftBank)

  - Band (n)5 + Band n78 or n79 (docomo)

  - Band 19 + Band n78 or n79 (docomo)

  - Band 21 + Band n78 or n79 (docomo)

  - Band (n)1 + Band n79 (docomo)

Proposals:

- Proposals for EN-DC

  - Option 1: DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations. (China Telecom, CMCC)

  - Option 2: DL interruptions on any band combinations with UE capability indication should be allowed (Xiaomi)

  - Option 3: DL interruptions due to UL Tx switching are not allowed for any EN-DC band combinations. (Nokia)

- Proposals for UL CA

  - Option 1: DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations. (China Telecom, CMCC, Nokia)

    - Define general criteria to identify the difficult band combinations (China Telecom, Nokia)

      - Criteria A: Frequency gap between bands and TDD bands frequency range (Nokia)

        - Note that in principle if the gap is narrower, higher possibility to require DL interruption.

      - Criteria B: Reported Tx switching period via uplinkTxSwitchingPeriod (Nokia)

        - Note that if the period is 210 us, higher possibility to require DL interruption

      - Criteria C: combination of Criteria A and B (Nokia)

      - Other criteria

  - Option 2: DL interruptions on any band combinations with UE capability indication should be allowed (Xiaomi)

- Additional interested band pairs from operators

  - Band (n)40+ Band n41 or n79 (CMCC)

  - Band (n)41+ Band n79 (CMCC)

  - Band (n)39+ Band n41 or n79 (CMCC)

Moderator’s recommendation:

- Recommended WF

  - Can we agree with option 1 for both EN-DC and UL CA, i.e., DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations?

    - Encourage more feedback on the general criteria to identify the difficult band combinations, or feedback on whether DL interruption can be avoided for the band pairs from operators

Issue 1-2: Capture of the applicability on DL interruption

Agreement in RAN4 #95e (R4-2008476 in WF):

- Option 1: Specify in RRM spec for which band combinations DL interruptions are allowed.

- Option 2: Capture to TS38.101-1 and TS38.101-3

Proposals:

- Option 1: Specify in RRM spec (China Telecom, CMCC)

  - China Telecom: If certain general principle is agreed, it can be captured in RRM spec

  - CMCC: It is better to capture the applicability in the same spec together with DL interruption requirements in order to avoid the cross reference between different specs.

- Option 2: Specify in RF spec (China Telecom)

  - China Telecom: If it is agreed per BC basis, it can be captured in RF spec

Moderator’s recommendation:

- Recommended WF

  - TBA based on the feedback from more companies

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Issue 1-1: Applicability of DL interruption for different band combinations Agree with the WF recommended by Moderator. For the criteria on whether or not it is a difficult band combination, Criteria 3 seems more reasonable and generic. Issue 1-2: Capture of the applicability on DL interruption Option 2, since the DL interruption capability is defined on a per-BC basis. |
| Huawei | Issue 1-1: Option 1. We can agree on Option 1 for EN-DC and Option 1 for UL CA. But for UL CA, we prefer not to decide the criterion at the beginning. Rather, we prefer to look at the combination one by one. Based on the conclusions for those band combinations, we can try to summarize the criterion afterwards. Issue 1-2: Option 1. Since the corresponding interruption requirements will be specified in RRM specifications, we prefer to specify the list of band combinations for which the interruption is allowed or not allowed. And RAN4 should decide whether the list without interruption or the list with interruption should be specified. In our view, the baseline combination should be the combination with interruption and thus the list should be specified as the exception combination with no interruption allowed. |
| Xiaomi | Issue 1-1: Applicability of DL interruption for different band combinations In our paper, we support option 2 by considering it is very hard to give a common rule on how to justify which band combination should belong to the so called “difficult band combinations”, but if a criterion could be accept by companies, option 1 is also acceptable for us. For UL CA, we prefer to have the same approach as EN-DC since they have the same RF link for the same band combination. |
| CMCC | Issue 1-1: Option 1 Agree with the recommended WF. Regarding the criteria, we suggest getting feedback on the proposed band combinations, then trying to come out with a general critiera. Issue 1-2: Option 1 As we discussed in our paper. There is no interruption requirements description in RF spec, so we propose to specify in RRM spec together with the interruption requirements. |
| OPPO | Issue 1-1: Applicability of DL interruption for different band combinations Ok with Option 1. For the already identified criteria, it can be specified as one of the criteria in the spec. However, not sure whether other criterias can be agreed in this meeting, since the problems could be band combination specific, and time consuming to collecting all the issues then put them into formula. If not been solved in this meeting and the WI need to be closed then maybe it can left to UE implementation to decide. Issue 1-2: Capture of the applicability on DL interruption No strong concern on this issue. Slightly prefer Option 2, i.e. RF spec, to keep band combination related things together. |
| China Telecom | Issue 1-1: Applicability of DL interruption for different band combinations Support option 1 and the recommended WF. Encourage feedback from vendors on which band combinations would be the difficult ones. Otherwise, can we mandate no DL interruption for the band pairs provided by operators? Issue 1-2: Capture of the applicability on DL interruption As discussed in our paper, either option would be possible, depending on the outcome of issue 1-1. |
| CHTTL | Issue 1-1: we are fine with the moderator’s suggestion, option 1. Issue 1-2: In general we share the same view as China Telecom’s comments below the options of the proposals. |
| MediaTek | Issue 1-1: Applicability of DL interruption for different band combinations Agree with the WF recommended by Moderator. DL interruption shall be allowed for Band (n)40+ Band n41 if the two TDD bands are not synchronized for UL/DL configuration and frame timing. Issue 1-2: We think both options shall be adopted. There are only a few combinations may require DL interruption that it would be clear if they are listed in TS38.101-1 & -3. And RRM shall specify requirements for DL interruption anyway. |
| Apple | Issue 1-1: We support option 2. we think this is related to UE architecture and capability. We do not fully understand the rationale of the criterion specified under option 1. For example, in 38.133, interruption is assumed in general with all RF change, including power, carrier frequency etc. Why should we treat this case differently? Issue 1-2: option 1. It would be good to specify the DL interruption requirement with other interruption requirements. |
| CATT | Issue 1-1: Applicability of DL interruption for different band combinations Agree with the recommended WF by the Moderator. Regarding the criteria for difficult band combinations, feedbacks from UE vendors is expected at first then the group can decide some criteria. Issue 1-2: Capture of the applicability on DL interruption Generally RRM spec is Band/Band combination agnostic. Band specific issues for UE are usually captured in 38.101-1/2/3. From this perspective we prefer option 2. However we can also live with Option 1. |
| Nokia | Issue 1-1: Applicability of DL interruption for different band combinations We appreciate the recommended WF. We, however, believe that in order to move forward, WF should include more specific actions. Hence, allow us to make the WF more specific. Way forward - DL interruptions are allowed only the identified band combinations by explicit criterion (criteria) based on technical reasons. - The identified band combinations will be explicitly captured in the specification like SUO - Encourage people to at least check the validity of the following criteria mentioned in R4-2003986 by applying the criteria to band combinations listed in the approved WF (R4-2008476) in RAN4#95-e by the RAN4#97-e. Note: we are going to make the list of band combinations and its DL interruption applicability based on the following criteria. Criteria 1: FGAP > 200MHz separation when TDD band <=2GHz. Criteria 2: FGAP > 400MHz when 2GHz <=TDD band <=4GHz. Criteria 3: FGAP > 800MHz when TDD band >=4GHz. - Encourage people to provide views with technical reasons. Example one: Band (n)3 + Band n78 From Criteria 2, No DL interruption is needed for this case. Etc.. |
| Qualcomm | Issue 1-1: We would prefer option 2. Defining which combinations are “difficult” is always implementation specific and defining those by spefication does not allow for development of new solutions. It also will be difficult discussion to agree which are and will extend the work quite a bit. On a side note, how is this feature defined for TDD 2Tx + TDD 2Tx e.g. n41+n79? Is 2tx assumption still valid or do we assume three Tx and then switch between the 1tx+2Tx and 2Tx+1Tx? Issue 1-2: DL interruptions are always specified in RRM spec. |
| vivo | Issue 1-1: Prefer option2. Support Qualcomm’s view that identify “difficult” is not that easy. Issue 1-2: Prefer in RRM spec. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#1 | Issue 1-1: Applicability of DL interruption for different band combinations Candidate options: - For EN-DC and UL CA - Option 1: DL interruptions are only allowed for band combinations where it is difficult to avoid DL interruptions in practical UE implementations. (China Telecom, CMCC, ZTE, Huawei, Xiaomi, OPPO, CHTTL, MediaTek, CATT, Nokia) - Option 1a: Define general criteria to identify the difficult band combinations (China Telecom, Nokia, ZTE, MediaTek) - Criteria A: Frequency gap between bands and TDD bands frequency range (Nokia, MediaTek) - FGAP > 200MHz separation when TDD band <=2GHz. - FGAP > 400MHz when 2GHz <=TDD band <=4GHz. - FGAP > 800MHz when TDD band >=4GHz. - Option 1b: Check the band combinations one by one (CMCC, HW, China Telecom) - CMCC, HW: first get feedback on the proposed band combinations, then try to come out with a general criteria. - Option 2: DL interruptions on any band combinations with UE capability indication should be allowed (Xiaomi, Apple, Qualcomm, vivo) Recommendations for 2nd round: Continue discussion in the 2nd round. If it is difficult to agree on any generic criteria, can we check the following band combinations one by one? Given the limited time, each operator is also invited to provide information on which band combination(s) can be given with highest priority from the deployment point of view. - Band (n)1 + Band n78 (CTC, CU, KDDI, CHTTL) - Band (n)3 + Band n78 (CTC, CU, KDDI, CHTTL) - Band (n)1 + Band n77 (KDDI) - Band (n)3 + Band n77 (KDDI) - Band (n)18 + Band n77 or n78 (KDDI) - Band (n)28 + Band n77 or n78 (KDDI) - Band (n)3 + Band n41 (CMCC) - Band (n)3 + Band n79 (CMCC, docomo) - Band (n)8 + Band n41 or n79 (CMCC) - Band (n)8 + Band n77 or n78 (SoftBank) - Band 11 + Band n77 or n78 (SoftBank) - Band (n)5 + Band n78 or n79 (docomo) - Band 19 + Band n78 or n79 (docomo) - Band 21 + Band n78 or n79 (docomo) - Band (n)1 + Band n79 (docomo) - Band (n)40+ Band n41 or n79 (CMCC) - Band (n)41+ Band n79 (CMCC) - Band (n)39+ Band n41 or n79 (CMCC) Issue 1-2: Capture of the applicability on DL interruption Candidate options: - Option 1: Specify in RRM spec (CMCC, Huawei, MediaTek, Apple, CATT, Qualcomm, vivo) - Option 2: Specify in RF spec (OPPO, MediaTek, ZTE, CATT) - Option 3: Specify in RRM spec if certain general principle is agreed; specify in RF spec, if it is agreed per BC basis (CHTTL, China Telecom) Recommendations for 2nd round: Further discuss the above options and the following aspects: - Whether to list the band/band combination with or without DL interruption? - Can we define the band/band combination specific applicability in RRM spec? Issue 1-3: How is this feature defined for TDD 2Tx + TDD 2Tx (new issue raised in the 1st round discussion) QC: Is 2tx assumption still valid or do we assume three Tx and then switch between the 1tx+2Tx and 2Tx+1Tx? Recommendations for 2nd round: Feedback on the above question is welcome. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on remaining issues for Tx switching between two uplink carriers | China Telecom |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: CR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010348 | Ericsson | CR on Modification of Pcmax for UL CA with uplink Tx switching capability |

Open issues summary

Issue 2-1: Pcmax for UL CA with Tx switching

Agreement in RAN #88e (RP-201365, Final Proposal of Switched UL):

- …

- No other RAN4 spec impact due to change introduced related to this power boosting and UL Tx switching between case 1 and case 2

  - No other RAN4 spec impact due to the change introduced related to this power boosting and UL Tx switching between case 1 and case 2 does not preclude companies’ maintenance Cat F CR, which can be agreed based on consensus.

Proposals:

- Proposal 1 (Ericsson)

  - For an inter-band UL CA configuration with the capability uplinkTxSwitchingPeriod is present (switching between carrier 1 and carrier 2), the maximum power on carrier 2 is boosted by 3 dB if the capability uplinkTxSwitchingPowerBoosting is present and powerboostingTxSwitching is set to 1. This is currently specified in clause 6.3A.3.3 on the transmit ON/OFF time mask for inter-band CA, but should be specified in the clause on configured power (Pcmax) for CA. However, the Pcmax for UL CA does not allow 3 dB power boosting for the BC, the total power is capped by the default CA power class (PC3); a modification is needed.

Note: the revision of the CR has been uploaded in the following folder: https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_96_e/Inbox/Drafts/%5B114%5D%20NR_RF_FR1_Part_2/CR%20on%20Pcmax%20for%20UL%20CA

Moderator’s recommendation:

- Recommended WF

  - TBA based on the feedback from companies

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Issue 2-1: Pcmax for UL CA with Tx switching In order to perform power boosting correctly, Pcmax for UL CA with Tx switching should be modified accordingly. This is part of the power boosting scheme itself, and the corresponding Cat. F CR should be approved in this meeting. |
| Huawei | Issue 2-1: We oppose the proposal. The proposal is against the RAN plenary agreement as listed by moderator above, i.e., No other RAN4 spec impact due to change introduced related to this power boosting and UL Tx switching between case 1 and case 2. Using the other company’s wording, it is not a good precedence. In our view, companies tried to agree on something first by saying there is no big impact and then afterwards tried to insert the proposal with a big impact. Clearly the CR is not maintenance rather than a new scheme. In our view, such high power UE needs a complete solution, which also includes the complete SAR solution. We propose to solve this “last minute inserted” mechanism using an UE implementation solution in Rel-16, and further discuss it in UL CA&SUL HPUE WI to come up with a package of complete solution including SAR solution. In Rel-16 we cannot agree on it. |
| Ericsson | Issue 2-1: The proposed CR is a Rel-16 Cat F CR for making a correction to enable the agreed power-boosting feature already included in the Rel-16 specification. The intention in RAN was to enable a 3dB max power boost in order that UL CA would operate in the same manner as SUL, but RAN did not consider the Pcmax part of the specification. Hence if the CR is not agreed, the RAN plenary decision is not implemented correctly. The changes are only applicable to UEs with the switching capability. Hence maintenance and allowed as per the RAN agreement: “No other RAN4 spec impact due to the change introduced related to this power boosting and UL Tx switching between case 1 and case 2 does not preclude companies’ maintenance Cat F CR, which can be agreed based on consensus.” Huawei: we cannot agree on the argument in the response. According to RAN plenary agreement, the main change to implement the 3dB power boosting is in RAN2 rather than RAN4. The proposed change in the CR for Pcmax would be complicated and viewed as significant change, which fundamentally changes the CA based power configuration. Anyway in Rel-16 RAN4 was discussing inter-band UL CA based Tx switching, which falls in inter-band UL CA framework according to the agreement. According to our understanding in RAN plenary, 3dB power boosting is UE-implementation in Rel-16. The solution for facilitating SAR compliance is that same as that used for SUL in Rel-16: “up to UE implementation. The SAR issue for SUL with an average power exceeding 23 dBm is the same as that for UL CA. Solved by the same means for both SUL and UL CA. No difference. SUL + NUL has no cap at 23 dBm average power: 26 dBm allowed for the NUL even if the SUL is at full power and the duty cycle of the total signal is 100%. Huawei: We mean that in last RAN plenary we commented that there are a number of issues which need be addressed in RAN4 and maybe RAN1. But other companies argued that there is no big impact. As a compromise we agreed to introduce 3dB power boosting on carrier #2 in case 2. But in this meeting, the configured power is proposed and the change is big. So by mentioning SAR, we mean that we still think the introduction of 3dB power boosting has impact on RAN4 and maybe RAN1. So we prefer to leave it to UE implementation in Rel-16 and further discuss if and how to enhance it in future release as a package. |
| Apple | We are OK with the proposal to address the issue where UE support PC2 in single CC, which can be part of UL CA where only PC3 is specified. We also agree with clarifying this in the configured output power clause. |
| Qualcomm | Agree that the change proposed would clarify the spec. Referring to plenary agreement in letter may not make sense since we are just doing the changes needed and agreed in the plenary in to ran4 specification and making them better. Not doing this degrades the quality of the specification for this already complicated feature. Definitely support agreeing the CR. |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |
| Revised R4-2010348, Ericsson | ZTE: The proposed changes are necessary for UE to implement power boosting correctly. We support this CR. |
|  | Huawei: We oppose CR. The reason is given above. |
|  | Ericsson: this is a Rel-16 Cat F (maintenance) CR, should be agreed to enable the functionality approved by RAN. |
|  | Apple: we are OK with this CR |
|  | Nokia: Are there any reasons not have included ∆MPRc from the formula? |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Topic #2 | Issue 2-1: Pcmax for UL CA with Tx switching Candidate options: - Option 1: Update the configured transmitted power (Ericsson, ZTE, Apple, Qualcomm) - Option 2: Not update the configured transmitted power (Huawei) Recommendations for 2nd round: Continue discussion in the 2nd round, and also encourage offline among interested companies. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010348, Ericsson | To be revised. This CR has already been revised before the meeting, so a revision tdoc number is needed. |

Discussion on 2nd round

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |